3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316010

# Xiamen, 9th - 13th Oct, 2023

Title: 	Simulation results on UE multi-Rx demodulation requirements for HST FR2

Source: 	Huawei, HiSilicon

Agenda item:	5.12.4.3

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] for FR2 HST demodulation requirements was approved. In this contribution, we provide our initial simulation results about UE multi-Rx demodulation requirements for HST FR2.

Simulation

The simulation results are shown in the following Table 2-1 and Table 2-2 based on the simulation assumption specified in the latest WF in last meeting. In Table 2-1 the same power is transmitted for each TRxP with scaling factor 1/sqrt(2), while in Table 2 the relative power is modelled.

Table 2-1 Ideal simulation results for UE multi-Rx demodulation requirements for HST FR2

| Case number | MCS | SNR@70%maximum throughput |  | Diff |
| --- | --- | --- | --- | --- |
|  |  | 1FFT | 2FFT |  |
| 1 | 17, 13 | 11.3 | 10.1 | 1.2 |
| 2 | 17, 19 | 14.2 | 12.9 | 1.3 |
| 3 | 17 | 12.8 | 12.6 | 0.2 |
| Note: The same power is transmitted for each TRxP with scaling factor 1/sqrt(2). |  |  |  |  |

Table 2-2 Ideal simulation results for UE multi-Rx demodulation requirements for HST FR2

| Case number | SNR@70%maximum throughput |
| --- | --- |
| 1 | 15.4 |
| Note: The relative power is modelled. |  |

There is no large span between different assumption on FFT.

Conclusion

In this contribution, we provide our updated simulation results about UE multi-Rx demodulation requirements for HST FR2. Our observations are:

- There is no large span between different assumption on FFT.

Reference

R4-2313919, WF for FR2 HST demodulation, RAN4#108, Samsung