3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316012

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on NR UE ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.13.7.2

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about NR UE ATG demodulation requirements.

Discussion

PDCCH requirements

Test scope for PDCCH

- Option 1: To consider legacy PDCCH requirements for ATG PDCCH requirements. - 2T2R FDD: All test cases in 5.3.2.1.2 - 2T4R FDD: All test cases in 5.3.3.1.2 - 2T2R TDD: All test cases in 5.3.2.2.2 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 2: Select following legacy test cases for ATG UEs - 1T2R FDD: Test number 2, 3 and 4 in 5.3.2.1.1 - 2T2R FDD: All test cases in 5.3.2.1.2 - 1T2R TDD: Test number 1 and 2 in 5.3.2.2.1 - 2T2R TDD: All test cases in 5.3.2.2.2 - 1T4R FDD: Test number 2, 3 and 4 in 5.3.3.1.1 - 2T4R FDD: All test cases in 5.3.3.1.2 - 1T4R TDD: Test number 1 and 2 in 5.3.3.2.1 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 3: Select following legacy test cases for ATG UEs - 1T2R FDD: Test number 1, 3 and 5 in 5.3.2.1.1 - 2T2R FDD: Test number 3 in 5.3.2.1.2 - 1T2R TDD: All test cases in 5.3.2.2.1 - 2T2R TDD: All test cases in 5.3.2.2.2 - 1T4R FDD: Test number 1, 3 and 5 in 5.3.3.1.1 - 2T4R FDD: Test number 3 in 5.3.3.1.2 - 1T4R TDD: All test cases in 5.3.3.2.1 - 2T4R TDD: All test cases in 5.3.3.2.2 - Option 4: Do not consider in the ATG scope legacy PDCCH requirements

For the existing PDSCH test case, it is better to cover all aggregation level and select one case per aggregation level. We propose to select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements, i.e. Option 3. We are also OK with the Option 1. For the Option 2, there is no aggregation level 16 cases defined.

Table 2.2.1-1 FDD requirements in TS 38.101-4

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300-100 | 1x2 Low 1x4 Low | 1 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low 1x4 Medium A | 1 |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |
| 3 | 10 | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low 2x4 Low | 1 |

Table 2.2.1-2 TDD requirements in TS 38.101-4

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | TDLA30-10 | 1x2 Low 1x4 Low | 1 |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low 1x4 Low | 1 |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH. 2-2.1 TDD | TDLC300- 100 | 1x2 Low 1x4 Medium A | 1 |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH. 2-1.3 TDD | TDLC300-100 | 2x2 Low 2x4 Low | 1 |

Cover all aggregation level and select one case per aggregation level.

Select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements, i.e. Option 3.

CSI reporting requirements

Test scope for CSI reporting

Agreement: - For CQI requirements, reuse the following requirements: - 2T2R FDD: CQI requirements in 6.2.2.1.1.1 - 2T2R TDD: CQI requirements in 6.2.2.2.1.1 - 2T4R FDD: CQI requirements in 6.2.3.1.1.1 - 2T4R TDD: CQI requirements in 6.2.3.2.1.1 - Not introduce RI requirements for ATG UE - FFS for PMI requirements

As per WID, the maximum cell coverage is about 300km when the plane is above the sea, the maximum RTT delay is about 2ms. For more typical scenario, the ISD is 200km that is corresponding to 0.77ms RTT delay. We don’t think such propagation delay is  large comparing to the CQI/RI/PMI delay in the current CSI reporting cases (8ms or 9.5ms for AWGN case). In addition, there is no large Doppler in ATG scenario after UE compensation that means the channel is rather stable during tens of microsecond. So we think the CSI reporting is still useful for ATG scenario. We propose to reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

The CSI reporting is still useful for ATG scenario.

Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

Proposals

In this contribution, we discuss on NR UE ATG demodulation requirements. Our observations and proposals are:

- Cover all aggregation level and select one case per aggregation level.

Select the “yellow-marked” cases in the following Table 2.2.1-1 and Table 2.2.1-2 for ATG UE requirements, i.e. Option 3.

- The CSI reporting is still useful for ATG scenario.

Reuse the CSI reporting cases for ATG scenario from the existing legacy CSI reporting cases.

Reference

R4-2313875, WF for ATG demodulation requirements, RAN4#108, CMCC