3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316016

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	5.7.4.2

Document for:	Discussion

Background

During RAN4#108 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception.

Discussion

PTRS port for sDCI SDM scheme

Agreement: - Further evaluate both cases: one PTRS port across TRPs and one PTRS port for each TRP - Companies are encouraged to evaluate the performance impact and decide suitable parameters for each case. - Test applicable rules can be considered based on UE capability.

UE supporting one port per TRP for PTRS for single-DCI SDM scheme is up to UE capability as following Table 2.1-1. From our understanding, define requirements using the sub-level optional feature is not good for the test coverage since we are defining the minimum requirements.

Table 2.1-1 UE capability for supporting two PTRS ports for sDCI SDM scheme

| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Define requirements using the sub-level optional feature is not good for the test coverage since we are defining the minimum requirements.

In addition, the simulation results as following Table 2.1-2 also shows that there is negligible performance degradation for the phase noise impact for sDCI SDM scheme.

Table 2.1-2 Simulation results for the phase noise impact for sDCI SDM scheme

| Case number | MCS | Rank | ρ | UE processing | SNR@70%maximum throughput |  | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | w/ PN@39GHz | w/o PN |  |
| 1 | 13 | 2+2 | -15 | independent | 16.0 | 16.0 | 0.0 |
| 2 | 13 | 2+2 | -15 | joint | 15.3 | 15.1 | 0.2 |
| 3 | 13 | 2+2 | -12 | independent | 18.3 | 18.3 | 0.0 |
| 4 | 13 | 2+2 | -12 | joint | 15.5 | 15.3 | 0.2 |
| 5 | 17 | 1+1 | -9 | independent | 14.1 | 13.4 | 0.7 |
| 6 | 17 | 1+1 | -9 | joint | 12.8 | 12.4 | 0.4 |
| 7 | 17 | 1+1 | -6 | independent | 15.3 | 14.6 | 0.7 |
| 8 | 17 | 1+1 | -6 | joint | 13.1 | 12.6 | 0.5 |

There is negligible performance degradation for the phase noise impact for sDCI SDM scheme.

Above all, we propose to only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Define requirements using the sub-level optional feature is not good for the test coverage since we are defining the minimum requirements.

- There is negligible performance degradation for the phase noise impact for sDCI SDM scheme.

- Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Reference

R4-2313977, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#108, Qualcomm