3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316020

# Xiamen, 9th - 13th Oct, 2023

Title: 	Discussion on LS R5-235817

Source: 	Huawei, HiSilicon

Agenda item:	7.1.1

Document for:	Discussion

Background

During RAN5#100 meeting, LS [1] on Applicability of the DEMOD and CSI requirements for IoT NTN UE was sent to RAN4. In this contribution, we share our views about the issues raised in the LS.

Discussion

Question 1: What is RAN4’s plan to adapt LTE performance requirements for UEs supporting eMTC NTN only (not supporting TN)? Question 2: What is RAN4’s plan to define reporting of CSI requirements for UEs supporting eMTC NTN only (not supporting TN)?

For the Question 1 and the Question 2, based on UE capability specified in the TS 38.306, the UE supporting eMTC NTN is mandatory to support Category M1, so it is not possible for a UE supporting eMTC NTN only (not supporting TN). For the UE supporting eMTC NTN (therefore also supporting TN), the requirements in TS 36.101 Clause 8 and Clause 9 are also applies to UE according to the UE category and capability with the applicability rule specified in Clause 8.1 of the TS 36.101.

Answer 1/2: Based on TS 38.306, the UE supporting eMTC NTN is mandatory to support Category M1, so it is not possible for a UE supporting eMTC NTN only (not supporting TN). For the UE supporting eMTC NTN (therefore also supporting TN), the requirements in TS 36.101 Clause 8 and Clause 9 are also applies to UE according to the UE category and capability with the applicability rule specified in Clause 8.1 of the TS 36.101.

Question 3: What is RAN4 guidance on how to interpret Table B.2-1? Is it for Category M1 UEs only and there will be additional table to address Category NB1/NB2? Or is it for both Category M1 UEs and Category NB1/NB2 UEs and Table B.2-1 needs updates on Category NB1/NB2?

It seems that the connection setup table for Category NB1/NB2 UEs are missed. We think it is better to add a new table for Category NB1/NB2 UEs in the next RAN4 meeting.

Answer 3: The Table B.2-1 is for Category M1 UEs only. RAN4 will add a new table for Category NB1/NB2 UEs.

Proposals

In this contribution, we discuss on UE demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Answer 1/2: Based on TS 38.306, the UE supporting eMTC NTN is mandatory to support Category M1, so it is not possible for a UE supporting eMTC NTN only (not supporting TN). For the UE supporting eMTC NTN (therefore also supporting TN), the requirements in TS 36.101 Clause 8 and Clause 9 are also applies to UE according to the UE category and capability with the applicability rule specified in Clause 8.1 of the TS 36.101.

Answer 3: The Table B.2-1 is for Category M1 UEs only. RAN4 will add a new table for Category NB1/NB2 UEs.

Reference

R5-235817, LS on Applicability of the DEMOD and CSI requirements for IoT NTN UE, RAN5#100, CMCC