3GPP TSG-RAN WG4 Meeting #108bis	R4-2316031

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on RRM requirements for SL-U operations

Source: 	Huawei, HiSilicon

Agenda item:	5.30.3.2

Document for:	Discussion

Introduction

In RAN4#108 meeting, the impacted RRM requirements due to SL-U operation were discussed and the progress has been captured in WF [1], we will provide the discussion on the open issues for RRM requirements under SL-U operations in R18.

Discussion

Selection/Reselection of V2X synchronization reference source

For V2X synchronization reference source selection/reselection, RAN4 has agreed to extend the SyncRef UE detection time and the PSBCH-RSRP measurement period. In RAN4, the following agreements were achieved on SyncRef UE detection requirements in SL-U:

<Agreement> - For SyncRef UE synchronized to GNSS directly or indirectly when GNSS is of higher priority - The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as (A) at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of (B) for the purpose of selection/reselection to the SyncRef UE. - A: (1.6+1.6*x1) seconds - B: 30% of its SLSS transmissions during the Tdetect,SyncRef UE_V2X - FFS: The values x1 is the number 1.6s detection windows with at least 1 unavailable S-SSB period in the three selected S-SSB period for S-SSB search, and which of the three S-SSB periods in the 1.6s detection window are selected is up to UE implementation. x1 < x1_max, and FFS on x1_max. - For other cases - The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection/reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as (C) at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop a maximum of (D) for the purpose of selection/reselection to the SyncRef UE. - C: (8+8*x2) seconds - D: 6% of its V2X data and SLSS transmissions during the Tdetect,SyncRef UE_V2X - FFS: The value x2 is the number of 8s detection windows with at least 1 unavailable S-SSB period in the 480ms search windows in each 8s period, and the location of the 480ms search windows is up to UE implementation. x2 < x2_max, and FFS on x2_max.

The SyncRef UE detection time is extended based on the number of unavailable S-SSB in three selected searching windows for SyncRef UE detection. The number of unavailable S-SSB in three selected searching windows for SyncRef UE detection ide defined as x1 for sync case and defined as x2 for other cases. However, the max values of x1 and x2 are FFS. It can be observed that the SyncRef UE detection time is scaling by (1+x1/x2) times for SL-U operation. For example, when x1/x2 is equal to 1, then there are total 6 searching opportunities within the SyncRef UE detection time and the maximum number of unavailable searching opportunities is 3. When when x1/x2 is equal to 2, then there are total 9 searching opportunities within the SyncRef UE detection time and the maximum number of unavailable searching opportunities is 9. The values of x1_max and x2_max can be defined based on the assumed unavailable probability of S-SSB for each periodicity. If the unavailable probability of S-SSB for each periodicity is lower, then a larger x1_max/x2_max needs to be defined. If the unavailable probability of S-SSB for each periodicity is assumed as 50%, then the value of x1_max/x2_max

Proposal 1: For SyncRef UE detection under SL-U operation, the values of  x1_max and x2_max used for defining SyncRef UE detection time can be defined is 3.

For SL-U operation, RAN4 achieved the following agreement on how to extend PSBCH-RSRP measurement period.

<Agreement> For information what is the value ‘y’ and ‘y_max’, the agreement for requirements for Tmeasure,PSBCH-RSRP in the last RAN4 meeting is as follow: SL-DRX cycle [ms]; Tmeasure,PSBCH-RSRP [ms]. No SL-DRX; (2 + y)*160. SL-DRX cycle ≤ 160ms; (2 + y)*160. SL-DRX cycle > 160ms; (2 + y)*SL-DRX cycle. - y is the S-SSB periods in which the SLSS is not available due to LBT failures and FFS for detailed description and y_max which is capped.. - y_max = [2 or 4] - FFS if further updates are needed subject to RAN1 agreements on the number of additional SSB occasions

It is assumed that the PSBCH-RSRP measurement period includes two measurement samples. The value of y_max is defined as the maximum number of unavailable S-SSB occasion between two S-SSB measurement samples used for PSBCH-RSRP measurements. We suggest that the interval between two samples used for PSBCH-RSRP measurements is not defined to larger. So, the value of y_max is suggested to be defined as 2.

Proposal 2: For PSBCH-RSRP measurement under SL-U operation, the maximum number of unavailable S-SSB occasions within the measurement period can be defined as y_max=2.

Initiation/Cease of SLSS transmission

For initiation/cease of SLSS transmission with SyncRef UE as synchronization reference source, RAN4 agree to extend the evaluation period of PSBCH-RSRP measurements to (4+ x) S-SSB periods, where x is the number of S-SSB which is not available due to LBT failures during the evaluation period. The value of x is up to x_max. In last RAN4 meeting, the following agreements were achieved:

<Agreement> For information what is the value ‘x’ and ‘x_max’, the agreement for requirements for initiation/cease of SLSS transmission in the last RAN4 meeting is as follow: - Extending the measurement period to 4+x S-SSB periods, x is the S-SSB periods in which the SLSS is not available due to LBT failures, and is capped by x_max. FFS for detailed description and x_max.. - x_max = [4 or 6] - FFS if further updates are needed subject to RAN1 agreements on the number of additional SSB occasions

For legacy SL communication, the evaluation period for initiation/cease of SLSS transmission is equal to two PSBCH-RSRP measurement periods. So, the evaluation period shall at least include two PSBCH-RSRP measurement periods in SL-U operations. We suggest that the PSBCH-RSRP measurement period is extended as (2+ y) where y_max=2. Besides, the interval between two PSBCH-RSRP measurement periods is also suggested to be considered into the evaluation period. So, the value of y_max used for defining the evaluation period for initiation/cease of SLSS transmission is suggested to be defined as 6.

Proposal 3: For initiation/cease of SLSS transmission requirements with SyncRef UE as synchronization reference source under SL-U operation, the maximum number of S-SSB occasions not available within the evaluation period can be defined as x_max=6.

L1 SL-RSRP measurement

For NR SL resource (re)selection procedure, RAN4 needs to investigate whether to have L1 SL-RSRP requirement impact depending on RAN1’s conclusion for procedure design.  In RAN4, whether to reuse legacy requirements apply to SL-U in the next RAN4 meeting needs to be decided based on RAN1’s final conclusion.

In RAN1, the following agreements on SL-U mode 2 resource allocation have been achieved:

Agreement in RAN1#113 meeting Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Agreement in RAN1#114 meeting In Mode 2 resource allocation, - The higher layer can indicate a “number of consecutive slots for MCSt” (Nslot,MCSt) larger than 1 for L1 reporting multi-slots candidates to the higher layer. The candidate multi-slots resource definition is applied. - Otherwise, the candidate single-slot resource definition is applied (same as R16/17). - The higher layer selects resources from the reported SA according to one of the following based on UE implementation: - Random selection as per R16/17 - Higher layer is not restricted to select resources at random, and can select in consecutive slots - It is up to RAN2 to define detailed behaviour as needed - It is RAN1 intention that, once the higher layer selects a multi-slots candidate from the set SA, it will use all the single-slot resources of the selected multi-slots candidate for transmission. This RAN1 agreement has no intention on potential RAN2 discussion about how SL resource selection processes are defined in MCSt. - Note, the above is intended to support Approach 1 and 2 only.

For Mode 2 resource allocation in SL-U operation, a candidate resource can be either single-slot resource (same as R16/17) or multi-slots resource (a number of consecutive slots). The existing L1 resource allocation procedure in R16/R17 are reused for SL-U. UE still needs to perform single-shot L1 SL-RSRP measurement on each candidate resource. Hence, the existing L1 SL-RSRP measurement requirements need to be applied under SL-U operation.

Proposal 4: For NR SL mode 2 resource allocation procedure in SL-U operation, the legacy single-shot L1 SL-RSRP requirements in R16/R17 can be reused.

Congestion control measurement

For congestion control procedure, UE performs single-shot SL-RSSI measurement of each sub-channel included in the resource pool for estimating the channel busy ratio. For SL-RSSI measurements, three options were captured in WF [1].

<FFS> - Option 1: Existing requirements can be reused for SL-U - Option 2: The SL RSSI definition need to be revisited for SL-U considering the 2 candidate starting symbols for a PSCCH/PSSCH transmission - Option 3: The revision of SL RSSI definition is not in the scope of RAN4 and can be discussed in RAN1.

In RAN4, single-shot L1 SL-RSSI measurement requirements are defined for congestion control procedure. For resource (re-)selection procedure under SL-U operation, there have the enhancement on sub-channel design. However, UE still needs to perform single-shot L1 SL-RSSI measurements. SL-RSSI is the total received power of one sub-channel. The measurement performance of SL-RSSI usually will not be impacted due to the size of sub-channel.

Proposal 5: There is no impact on congestion control requirements due to SL-U operation, and the existing single-shot SL-RSSI measurement requirements can be applied.

Interruption

In RAN4, the following options has been captured in [1] for interruption requirements in SL-U operation.

Issue 7-1: Interruption requirements <FFS> - Option 1: Open to discuss whether and how to consider the interruption on WAN due to LBT operation - Option 2: Allow interruption to WAN due to LBT operation during SL-DRX off duration, and the requirements in clause 12.7.4 in TS38.133 could apply - Option 3: When SL-DRX is configured, no interruption is allowed during COT duration if the COT is shared. - Option 4: RAN4 to discuss the interruption requirement due to sensing during SL DRX off duration. - Option 5: No impact on interruption requirements due to sidelink operation on a unlicensed carrier

For SL-U operation, the some SLSS or data transmissions may be unavailable on some slot. However, the missing occasions of SLSS or data transmissions have no impacts on the interruption on WAN. For the interruptions to sidelink due to WAN actions, the interruption length will not be changed due to SL-U operation and the current interruption requirements can be reused.

Proposal 6: For SL-U operations, the existing interruption requirements for legacy sidelink communications are reused.

Conclusions

In this paper we provided our analysis on RRM impacts for R18 SL-U operations. The followings are provided.

Proposal 1: For SyncRef UE detection under SL-U operation, the values of  x1_max and x2_max used for defining SyncRef UE detection time can be defined is 3.

Proposal 2: For PSBCH-RSRP measurement under SL-U operation, the maximum number of unavailable S-SSB occasions within the measurement period can be defined as y_max=2.

Proposal 3: For initiation/cease of SLSS transmission requirements with SyncRef UE as synchronization reference source under SL-U operation, the maximum number of S-SSB occasions not available within the evaluation period can be defined as x_max=6.

Proposal 4: For NR SL mode 2 resource allocation procedure in SL-U operation, the legacy single-shot L1 SL-RSRP requirements in R16/R17 can be reused.

Proposal 5: There is no impact on congestion control requirements due to SL-U operation, and the existing single-shot SL-RSSI measurement requirements can be applied.

Proposal 6: For SL-U operations, the existing interruption requirements for legacy sidelink communications are reused.

Reference

- R4-2314349, “WF on R18 NR SL RRM requirements (part 1)”, LG Electronics