3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316059

Xiamen, China, 9 – 13 October, 2023

Title: 	Discussion on requirements for UE supporting multiple options

Source: 	Huawei, HiSilicon

Agenda item:	5.10.1

Document for:	Discussion

Introduction

RRM requirements for BWP without restriction are discussed in RAN4#108, and outcomes are captured in WF [1]. Based on [1] a generic issue on requirements for UE supporting multiple options need to be further discussed, including the following combinations.

- UE supporting both option B-1-1 and A

- UE supporting both option C and A

- UE supporting both option B-1-1 and B-1-2

In this paper we will provide our views on requirements for UE supporting multiple options.

Discussion

UE supporting both option B-1-1 and A

#### Issue 1-4: Requirements/UE behaviour for UE supporting both option B-1-1 and A - Option 1: - RAN4 doesn’t need to specifically discuss requirements for UE supporting both option B-1-1 and A since UE can always measure CD-SSB without gap or interruption. - Option 2: - There is no obvious gain if option A and option B-1-1 are supported simultaneously. - If it is supported that UE works with option A and option B-1-1 simultaneously, UE behaviour/conditions for RLM/BM/BFD measurements should be defined. - Option 3: - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour. - Option 4: - If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS)? - Option 5: - RAN4 not to define additional requirements or clarify UE behaviour for UE supporting multiple options. - Option 6: - Whether RAN4 should define requirements for a UE supporting multiple options could be discussed later.

We support option 3 and 5, i.e. no additional requirements are needed.

When UE supports both option B-1-1 and A, the relevant scenario is where UE is in an active BWP that does not contain CD- or NCD-SSB, but UE is configured to measure L1 both from SSB and CSI-RS. In this case, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP.

This is same to the R15 scenario where CD-SSB is within active BWP and UE is configured to measure L1 both from SSB and CSI-RS. We do not see additional UE behaviour or requirements are needed.

Proposal 1: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

UE supporting both option C and A

Issue 1-5: Requirements/UE behaviour for UE supporting both option C and A - Option 1: - For UE supporting both option C and A, when active BWP contains NCD-SSB, RAN4 shall discuss whether RLM/BM/BFD should be based on NCD-SSB or CSI-RS. At least we don’t expect UE needs to monitor both. - Option 2: - UE working with option A and option C simultaneously should be supported and UE behaviour/conditions for RLM/BM/BFD measurements should be defined. - Option 3: - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour. - Option 4: - RAN4 not to define additional requirements for UE supporting multiple options. - Option 5: - Whether RAN4 should define requirements for a UE supporting multiple options could be discussed later. - Option 6: - A UE supporting Option C must always additionally support at least on other options (A, B-1-1 or B-1-2).

We support option 3 and 4, i.e. no additional requirements are needed.

When UE supports both option C and A, the relevant scenario is where UE is in an active BWP that contains NCD-SSB, and UE is configured to measure L1 both from SSB and CSI-RS. In this case, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP.

This is same to the R15 scenario where CD-SSB is within active BWP and UE is configured to measure L1 both from SSB and CSI-RS. We do not see additional UE behaviour or requirements are needed.

Proposal 2: For UE supporting both option C and A, if configured by NW, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

UE supporting both option B-1-1 and B-1-2

Issue 1-6: Requirements/UE behaviour for UE supporting both option B-1-1 and B-1-2 UE shall not indicate support of both B-1-1 and B-1-2 for the same agreed granularity reporting. Whether and how to capture this in 38.133 is FFS depending on RAN1 conclusion.

In the latest RAN1 feature list [2], RAN1 agreed the reporting granularity for B-1-1 and B-1-2 is per FS. The restriction that UE shall not indicate support of both B-1-1 and B-1-2 for the same FS is not captured yet. As no technical agreement is needed in RAN4, we suggest RAN4 to further wait for RAN1 conclusion while working at the CRs for B-1-1 and B-1-2.

Conclusions

In this paper we provided our views on requirements for UE supporting multiple options.

Proposal 1: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

Proposal 2: For UE supporting both option C and A, if configured by NW, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

Reference

- R4-2314339, WF on support for bandwidth part operation without restriction, vivo

- R1-2308521, Updated RAN1 UE features list for Rel-18 NR after RAN1#114, Moderators (AT&T, NTT DOCOMO, INC.)