3GPP TSG-RAN4 Meeting #108bis	R4-2316076

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCRT | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: UE demodulation requirements for non-colocated NR-CA deployment scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Perf |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of PDSCH CA demodulation requirements for Type 2 UE with power imbalance and receive time difference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of PDSCH CA demodulation requirements for Type 2 UE with power imbalance and receive time difference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot veirify the Type 2 UE capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.7.5 (new), 5.2A.2.6 (new), A.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 5.1.1.7.5	Applicability and test rules for power imbalance and reception time difference parameters for intra-band non-contiguous CA

The performance requirements in Table 5.1.1.7.5-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.7.5-1: Applicability and test rules for intra-band non-contiguous CA UE demodulation tests

| UE feature/capability [14] | CA combinations where the tests apply | Test list | PCell CC configuration | Applicability notes |
| --- | --- | --- | --- | --- |
| Type 2 UE [intraBandNR-CA-non-collocated-r18] | CA_n77(2A) CA_n78(2A) | Clause 5.2A.2.6 | Any of CCs |  |

------------------------------------------------------------- End of change ------------------------------------------------------------

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### 5.2A.2.6	Minimum requirements for carrier aggregation with power imbalance and reception time difference

The performance requirements are specified in Table 5.2A.2.6-3, with the addition of test parameters in Table 5.2A.2.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.6-1.

Table 5.2A.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the ability of NR-CA Type 2 UE ([intraBandNR-CA-non-collocated-r18]) to demodulate the signal transmitted by the PCell and SCell in the presence of a stronger PCell signal on an adjacent frequency, and reception time difference between two cells. Throughput is measured on both the PCell and SCell simultaneously. | 1 |

Table 5.2A.2.6-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Length (L) |  | 12 for DL slot, 4 for special slot |
|  | PRB bundling size |  | Wideband |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmission |  |  | 1 |
| Redundancy version coding sequence |  |  | {0} |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table A.1.2-2 for FR1.30-1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 |
| Overhead for TBS determination |  |  | 0 |
| SSB transmission |  |  | Slot#0 with periodicity 20ms |
| Timing offset of the SCell from the PCell |  | µs | 33 |

Table 5.2A.2.6-3: Minimum performance for TDD CA with 30 kHz SCS

| Test number | Cell type | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation channel and antenna correlation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PCell | R.PDSCH.X.2 TDD | 40MHz/30kHz | [256QAM 0.82] | FR1.30-1 | 2x2 static condition (TS38.101-4 B.1.1) | 70% | TBD + 25dB |
|  | SCell | R.PDSCH.X.1 TDD | 40MHz/30kHz | [QPSK 0.3] | FR1.30-1 | 2x2 static condition (TS38.101-4 B.1.1) | 70% | TBD |

------------------------------------------------------------- End of change ------------------------------------------------------------

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

Table A.3.2.2.2-X: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (CA power imbalance)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-X.1 TDD | R.PDSCH.2-X.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 31 | 31 |  |  |  |
| MCS table |  | [256QAM] | [256QAM] |  |  |  |
| MCS index |  | [2] | [24] |  |  |  |
| Modulation |  | [QPSK] | [256QAM] |  |  |  |
| Target Coding Rate |  | [0.30] | [0.82] |  |  |  |
| Number of MIMO layers |  | [2] | [2] |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | [5376] | [58384] |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | [16896] | [184424] |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | [1] | [7] |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | [3] | [22] |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slots i = 20, 21 | Bits | [53424] | [213696] |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | [17808] | [71232] |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | [55968] | [223872] |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | [23.885] | [260.649] |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------