3GPP TSG-RAN WG4 Meeting #108bis	R4-2316077

Toulouse, France, 21 – 25 August 2023

Agenda Item:	5.12.4.2

Source:	Ericsson

Title:	Simulation results of PDSCH CA demodulation requirements for FR2 HST

Document for:	Discussion

# 1	Introduction

RAN4#108 agreed with the way forward on PDSCH demodulation requirements for Rel-18 FR2 HST [1] and updated the simulation assumption [2]. In this contribution, we discuss the detailed simulation assumption.

# 2	Simulation assumption

RAN4#108 has corrected the FRC table used for Rel-17 FR2 HST-DPS demodulation requirements and updated the FRC for Rel-18 CA demodulation requirements accordingly. This section shows the detailed description how to derive the FRC table.

## 2.1	HST-DPS Uni-directional scenario

In this scenario we need to consider 4 types of slots to design FRC table:

- Normal DL slots: 13 of 14 symbols are allocated for PDSCH (See Figure 1 (a)).

  - 18 symbols are used for DMRS (1+1+1) per PRB.

  - 10 symbols are used for PTRS per every other PRB.

  - 3 of 5 slots are normal DL slots (DDDSU).

- Special slots: 9 of 14 symbols are allocated for PDSCH (See Figure 1 (b)).

  - 18 symbols are used for DMRS (1+1+1) per PRB.

  - 6 symbols are used for PTRS per every other PRB.

  - 1 of 5 slots are special slots (DDDSU).

- TRS slots (See Figure 1 (c)):

  - 18 symbols are used for DMRS per PRB.

  - 24 symbols are used for TRS for 52 PRBs.

  - 10 symbols are used for PTRS per every other PRB.

  - In HST condition, TRS is transmitted in slots 5/6/85/86 every 160 slots.

- SSB slots: Configures 4 SSBs. In FR2, since 2 SSB can be transmitted in one slot, SSB slots are scheduled in 0 and 1 every 20ms (= 160 slots). No PDSCH is scheduled in the SSB slots.

| (a) Normal DL slots | (b) Special slots | (c) TRS slots |
| --- | --- | --- |

Figure 1	DMRS/PTRS/TRS allocation in FR2 HST-DPS Uni-directional scenario.

The total PDSCH symbols are derived as Table 1. Based on the total symbols the FRC tables are givens as in Appendix A.1.

Table 1	Available PDSCH symbols for FR2 HST-DPS Uni-directional scenario.

|  | 200MHz (132PRB) | 50MHz (32PRB) | 100MHz (66PRB) | 400MHz (264PRB) |
| --- | --- | --- | --- | --- |
| Normal slots | (12*13-18)*132 - 10*(132/2) = 17556 | (12*13-18)*32 - 10*(32/2) = 4256 | (12*13-18)*66 - 10*(66/2) = 8778 | (12*13-18)*264 - 10*(264/2) = 35112 |
| Special slots | (12*9-18)*132 - 6*(132/2) = 11484 | (12*9-18)*32 - 6*(32/2) = 2784 | (12*9-18)*66 - 6*(66/2) = 5742 | (12*9-18)*264 - 6*(264/2) = 22968 |
| TRS slots (*1) | (12*13-18)*132 - 10*(132/2) - 24*52 = 16308 | (12*13-18)*32 - 10*(32/2) - 24*32 = 3488 | (12*13-18)*66 - 10*(66/2) - 24*52 = 7530 | (12*13-18)*264 - 10*(264/2) - 24*52 = 33864 |
| Note 1: TRS is transmitted in Max(52, CBW) PRBs |  |  |  |  |

## 2.2	HST-DPS Bi-directional scenario

In this scenario we need to consider 4 types of slots to design FRC table:

- Normal DL slots: Same as Uni-directional scenario (See Figure 2 (a)).

- Special slots: Same as Uni-directional scenario (See Figure 2 (b)).

- TRS slots (See Figure 2 (c)):

  - 18 symbols are used for DMRS per PRB.

  - 48 symbols are used for TRS for 52 PRBs.

  - 6 symbols are used for PTRS per every other PRB in PRBs where TRS is transmitted.

  - 10 symbols are used for PTRS per every other PRB in PRBs where TRS is not transmitted.

  - In HST condition, TRS is transmitted in slots 5/6/85/86 every 160 slots.

- SSB slots: Configures 8 SSBs. In FR2, since 2 SSB can be transmitted in one slot, SSB slots are scheduled in 0/1/2/3 every 20ms (= 160 slots). No PDSCH is scheduled in the SSB slots.

| (a) Normal DL slots | (b) Special slots | (c) TRS slots |
| --- | --- | --- |

Figure 2	DMRS/PTRS/TRS allocation in FR2 HST-DPS bi-directional scenario.

The total PDSCH symbols are derived as Table 2. Based on the total symbols the FRC tables are givens as in Appendix A.2.

Table 2	Available PDSCH symbols for FR2 HST-DPS bi-directional scenario.

|  | 200MHz (132PRB) | 50MHz (32PRB) | 100MHz (66PRB) | 400MHz (264PRB) |
| --- | --- | --- | --- | --- |
| Normal slots | (12*13-18)*132 - 10*(132/2) = 17556 | (12*13-18)*32 - 10*(32/2) = 4256 | (12*13-18)*66 - 10*(66/2) = 8778 | (12*13-18)*264 - 10*(264/2) = 35112 |
| Special slots | (12*9-18)*132 - 6*(132/2) = 11484 | (12*9-18)*32 - 6*(32/2) = 2784 | (12*9-18)*66 - 6*(66/2) = 5742 | (12*9-18)*264 - 6*(264/2) = 22968 |
| TRS slots (*1) | (12*13-18)*132 - 6*(52/2) - 10*((132-52)/2) - 48*52 = 15164 | (12*13-18)*32 - 6*(32/2) - 48*32 = 2784 | (12*13-18)*66 - 6*(52/2) - 10*((66-52)/2) - 48*52 = 6386 | (12*13-18)*264 - 6*(52/2) - 10*((264-52)/2) - 48*52 = |
| Note 1: TRS is transmitted in Max(52, CBW) PRBs. 6 PTRS symbols in PRBs where TRS is transmitted. 10 PTRS symbols in PRBs where TRS is not transmitted. |  |  |  |  |

# 3	Simulation results

Table 3 shows our simulation results for FR2 HST-DPS according to the FRCs shown in Appendix.

Table 3	PDSCH CA simulation results for alignment.

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC BI-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-2 | FRC BI-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-3 | FRC BI-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 2-1 | FRC UNI-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-2 | FRC UNI-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-3 | FRC UNI-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |

# 4	Summary

Proposal: Confirm the FRC tables shown in Appendix for FR2 HST-DPS CA PDSCH demodulation requirements.

# References

- R4-2313919, “WF for FR2 HST demodulation”, Samsung.

- R4-2313920, “Simulation assumption for FR2 HST demodulation”, Ericsson.

- R4-2313933, “Correction CR on Rel-17 FR2 HST test setup and FRC”, Samsung, Ericsson, Anritsu, Cybercore.

# Appendix

The numbers highlighted in yellow are correction is the changes from [2].

## A.1	PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and uni-directional HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | UNI-1 TDD | UNI-2 TDD | UNI-3 TDD |  |
| Channel bandwidth | MHz | 50 | 100 | 400 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 32 | 66 | 264 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For slot i = 2 |  | 13 | 13 | 13 |  |
| For slot i = 3 |  | 9 | 9 | 9 |  |
| Allocated slots per 2 frames |  | 126 | 126 | 126 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 17 | 17 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 18 | 18 | 18 |  |
| For Slot i = 3 |  | 18 | 18 | 18 |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 13832 | 28680 | 114776 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 21504 | 45096 | 180376 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 21504 | 45096 | 180376 |  |
| For Slot i = 3 |  | 13832 | 28680 | 114776 |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 24 | 24 | 24 |  |
| For Slot i = 3 |  | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 2 | 4 | 14 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 3 | 6 | 22 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 3 | 6 | 22 |  |
| For Slot i = 3 |  | 2 | 4 | 14 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 41856 | 90360 | 406368 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 41856 | 90360 | 406368 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 84, 87, …, 159} | Bits | 33408 | 68904 | 275616 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 51072 | 105336 | 421344 |  |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 51072 | 105336 | 421344 |  |
| For Slot i = 3 |  | 33408 | 68904 | 275616 |  |
| Max. Throughput averaged over 2 frames | Mbps | 123.2 | 257.839 | 1031.409 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

## A.2	PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and bi-directional HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | BI-1 TDD | BI-2 TDD | BI-2 TDD |  |
| Channel bandwidth | MHz | 50 | 100 | 400 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 32 | 66 | 264 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 17 | 17 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 13832 | 28680 | 114776 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 21504 | 45096 | 180376 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 2 | 4 | 14 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 3 | 6 | 22 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 33408 | 76632 | 392640 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 33408 | 76632 | 392640 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 33408 | 68904 | 275616 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 51072 | 105336 | 421344 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 121.433 | 254.15 | 1016.651 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |