3GPP TSG-RAN WG4 Meeting #108bis	R4-2316079

Xiamen, China, 9 October – 13 October 2023

Agenda Item:	5.12.4.3

Source:	Ericsson

Title:	UE demodulation requirements for FR2 HST multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#108 agreed with the WF on the UE demodulation requirements for Rel-18 FR2 HST [1]. This contribution discusses the remaining open issues on the simultaneous multi-Rx reception scenario.

# 2	Discussion

## 2.1	Channel model

RAN4#108 agreed with the following channel model used for the simultaneous multi-Rx reception by reusing the Rel-17 bi-directional deployment model illustrated in Figure 1.

| Panel 1 | fst=fdcosθ(t) cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2, 0<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |
| --- | --- |
| Panel 2 | fst=fdcosθ(t) cosθt=-Ds-Ds_offset+vtDmin2+Ds-Ds_offset+vt2, 0<t≤2Ds_offsetv--Ds_offset+vtDmin2+-Ds_offset+vt2, 2Ds_offsetv<t≤Dsv cosθ(t)=cost modDsv, t>Dsv |

Figure 1	RRH coverage area for simultaneous multi-Rx reception scenario.

RAN4 also agreed to consider the deployment parameters shown in Table 1.

Table 1	Deployment parameters

| Parameters | Description | Scenario B-2 |
| --- | --- | --- |
| Ds | Inter-RRH distance | 700 m |
| Dmin | Distance between rail track and RRH | 150 m |
| v | Train velocity | 350 km/h |
| fd | Maximum Doppler frequency shift | 9722 Hz |
| fc | Carrier frequency | 30 GHz |
| Drx_panel | Distance between two UE Rx panels | 0 m |
| Ds_offset | Switching transmission point between adjacent RRHs | 350 m |

The remaining open issue is whether to model the relative received power between two Rx panel.

- Option 1 - Option 1a: Define power profiles based on the free-space pathloss - Option 1b: Define power profiles that considers UE and RRH beam gains in addition to pathloss - Option 2 - Model the relative power for HST FR2 multi-Rx channel modelling as following: - Power level Pk (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by: - Pk=-20lgy-xk-10lgi∈ii*Ds±Ds-Dsoffset≤a<i*Ds±Ds+Dsoffset1y-xi2for k*Ds±Ds-Dsoffset≤a<k*Ds±Ds+Dsoffset - Pk=-20lgy-xk-10lgi∈ii*Ds+Dsoffset≤a+0,1*Ds<i*Ds+Ds-Dsoffset1y-xi2for k*Ds+Dsoffset≤a+0,1*Ds<k*Ds+Ds-Dsoffset

We understand the introduction of power profile model is more realistic especially for FR2 HST scenario. However as discussed in RAN4#108, our concern is how to set the requirements. RAN4 usually fix the rank/MCS and SNR to measure the throughput for PDSCH demodulation requirements. We think this framework is beneficial not only to get better alignment of companies’ simulation results and to define the test procedure/verdict in RAN5 conformance test discussion. If we consider changing the receiver power (or SNR level) during the tests, RAN4 may need to consider several MCS indexes, and the detailed MCS index scheduling should be specified in FRC tables. We also concern the complexity of test setup and also the alignment of simulation results.

In our understanding, one of the motivations to consider the power profile in the channel model is to verify the independent Rx chains for PC6 UE multi-Rx simultaneous reception scenario. For this purpose, we propose not to define power profile model in FR2 HST simultaneous multi-Rx scenario, but instead consider received power difference between two Rx panels.

Proposal 1: Channel models for FR2 HST multi-Rx simultaneous reception should consider the fixed received power model. Set the different SNR level per Rx panel during the test assuming the different distance between Rx panel and serving TRP to verify the independent Rx chain performance.

## 2.2	Evaluation results

This section shows our simulation results for multi-Rx simultaneous reception scenario according to the parameters in Table 2 and simulation assumption [2].

Table 2	PDSCH simulation parameters

| Parameters | TRP#1 | TRP#2 |
| --- | --- | --- |
| Carrier frequency | 30 GHz |  |
| SCS/CBW | 120kHz / 200MHz |  |
| TDD Pattern | DDDSU, S=10D2G2U Schedule PDSCH in special slots |  |
| Antenna configuration | 2x2 | 2x2 |
| TRS periodicity | 10 ms (80 slots) | 10 ms (80 slots) |
| Number of active TCI States | 1 | 1 |
| Number of additional DMRS | 2 | 2 |
| rank | 2 | 2 |
| MCS index (MCS table 1) | 17 | Option 1: 13 Option 2: 19 |
| Note: Apply a constant scaling factor 1/2 to the transmitted PDSCH signal from each TRP. |  |  |

| (a) Relative throughput | (b) Ratio of distances between Rx panel and serving TRP |
| --- | --- |

Figure 4	Simultaneous multi-Rx panel reception simulation results with 2+2.

Figure 4 (a) shows our simulation results assuming simultaneous multi-Rx reception, where we assume the independent processing per Rx panel. Figure 4 (b) also shows the ratio of distance between Panel 1 and the serving TRP (dist(rx1)) and distance between Panel 2 and the serving TRP (dist(rx1)) to investigate the possible received power imbalance. According to Figure 4 (b), the largest distance ratio is about 2.2. This means the power difference between two Rx panels is 6.8dB, if we consider the simple free-space path-loss model, L=20log104πdλ. According to Figure 4 (a), the SNR to achieve 70% of the maximum throughput with Panel 1 MCS17 is about 12.4dB, and thus we think MCS13 is a good candidate since the SNR to achieve 70% of the maximum throughput is about 9.2dB.

Observation 1: Largest received power difference between two Rx panels are about 6.8dB for FR2 HST simultaneous multi-Rx scenario channel model.

Proposal 2: Set MCS17 for Rx panel 1 and Set MCS13 for Rx panel 2 for FR2 HST simultaneous multi-Rx scenario.

# 3	Summary

Proposal 1: Channel models for FR2 HST multi-Rx simultaneous reception should consider the fixed received power model. Set the different SNR level per Rx panel during the test assuming the different distance between Rx panel and serving TRP to verify the independent Rx chain performance.

Observation 1: Largest received power difference between two Rx panels are about 6.8dB for FR2 HST simultaneous multi-Rx scenario channel model.

Proposal 2: Set MCS17 for Rx panel 1 and Set MCS13 for Rx panel 2 for FR2 HST simultaneous multi-Rx scenario.

# References

- R4-2313919, “WF for FR2 HST demodulation”, Samsung.

- R4-2313920, “Simulation assumption for FR2 HST demodulation”, Ericsson.