3GPP TSG-RAN WG4 Meeting # 108bis						R4-2316120

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.30.2.1

Source: 	OPPO

Title: 	on the SL-U A-MRP simulation

Document for:	Approval

- Introduction

The sidelink unlicensed MPR-A-MPR simulation assumption have been discussed and captured in the TR. Based on the simulation assumption, and also some general consideration as specified in our companion paper, we have done the simulation for SL-U MPR and A-MPR. The simulation results are shown in this paper and with the results, we propose the MPR and A-MPR for the SL-U.

- Discussion

The SL-U A-MPR simulation has been discussed during the last meeting, There are agreements to finish 5 NS values in Rel-18 Sidelink WID and the agreement is captured as below:

Issue 3-2: A-MPR NS values

Agreement:

- Finalize the following NS values for each band in Rel-18:

    - n46 :  NS_31

    - n96 :  NS_53, NS_60, NS_61

    - n102 : NS_58

Below we propose the A-MPR simulation results for these 5 NS values.

Table 0 is to summarize the cases we have been used for simulation. For each case, the QPSK, 16QAM, 64QAM and 256QAM will be simulated.

Table 0 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Band n46 NS_31

The NS_31 is for band n46. The corresponding requirements captured in TS 38.101-1 Table 6.5F.3.3.5-1 and the PSD requirement is 10/7/4 dBm/MHz based on different channel bandwidth and center frequency as captured in table 6.2F.1-2.

Table 6.5F.3.3.4-1: Additional requirements for NR-U channels assigned within 5150-5250 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5150 | -27 | 1 MHz |
| f ≥ 5250 | -27 |  |

Table 6.5F.3.3.4-2: Additional requirements for NR-U channels assigned within 5250-5350 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5250 | -27 | 1 MHz |
| f ≥ 5350 | -27 |  |

Table 6.5F.3.3.4-3: Additional requirements for NR-U channels assigned within 5470-5725 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5470 | -27 | 1 MHz |
| f ≥ 5725 | -27 |  |

Table 6.5F.3.3.4-4: Additional requirements for NR-U channels assigned within 5725-5850 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| f ≤ 5725 | -27 | 1 MHz |
| f ≥ 5850 | -27 |  |

With different PSD requirement, the simulation result is shown as below.

For PSD = 10dBm/MHz：

Table 1 A-MPR for NS_31 single CC PSD=10 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 2 A-MPR for NS_31 Wideband operation PSD=10 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 7dBm/MHz：

Table 3 A-MPR for NS_31 single CC PSD=7 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 4 A-MPR for NS_31 Wideband operation PSD=7 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 3.0 | 2.7 | 2.9 | 2.6 | 2.9 | 2.6 | 2.9 | 2.7 | 2.7 | 2.7 | 2.9 | 2.7 | 2.8 | 3.0 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 3.0 | 2.8 | 3.0 | 3.0 | 3.0 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

For PSD = 4dBm/MHz：

Table 5 A-MPR for NS_31 single CC PSD=4 dBm/MHz

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 5.6 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 5.7 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.7 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 6.6 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 6 A-MPR for NS_31 Wideband operation PSD=4 dBm/MHz

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.5 | 3.0 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 2.8 | 2.8 | 3.5 | 2.8 | 2.8 | 3.6 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 5.9 | 5.9 | 3.1 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 | 5.9 | 2.9 | 2.7 | 2.7 | 5.9 | 3.0 | 2.8 | 6.0 |
|  | 16QAM | 6.0 | 6.0 | 3.1 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 5.9 | 3.0 | 3.0 | 2.8 | 6.0 | 3.0 | 3.0 | 6.0 |
|  | 64QAM | 6.0 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 | 5.9 | 4.2 | 4.1 | 4.0 | 6.0 | 4.2 | 4.1 | 5.9 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

From the table 1 to table 6 above, it can be found that even though the PSD requirement is different from 10 to 7 to 4 dBm/MHz, the A-MPR simulation result for both single CC and wide-band operation are the same. The reason here is that the PSD requirement is not the dominate factor and it is the SEM and EVM for some of the 256QAM. The figure 1 and figure 2 below show the A-MPR.

Figure 1 NS_31 Single CC A-MPR

Figure 2 NS_31 Wideband A-MPR

The A-MPR requirement is captured in sub-clause 6.2F.3.5 and the requirement is captured below:

Table 6.2F.3.5-1: A-MPR for NS_31 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | See Table 6.2F.2-1 | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

It is observed that for 64QAM especially for interlaced RB allocation, the A-MPR is about 1 to 1.5dB higher then corresponding full RB allocation values. For the 20MHz 15kHz SCS interlaced RB allocation with 4 dBm/MHz PSD is applied, the A-MPR for QPSK and 16QAM are also higher then other cases. We try to apply the similar table for A-MPR as shown above for NS_31 for SL-U. From our simulation results, the QPSK 16QAM and 64QAM limits are quite near to the simulation results and seems can be re-used. For the 256QAM, we have similar simulation results for MPR and it seems larger A-MPR is needed. Based on the simulation results, 9dB A-MPR for 256QAM is proposed.

Observation 1:

- For NS_31, 64QAM, interlaced RB allocation, the A-MPR is about 1 to 1.5dB higher then corresponding full RB allocation values.

- With limited PSD, the A-MPR for QPSK and 16QAM in interlaced RB allocation needs larger A-MPR.

- The legacy NR-U A-MPR requirement for NS_31 can cover QPSK, 16QAM, 64QAM for SL-U.

Proposal 1:

- To reuse the legacy A-MPR requirement for NS_31 for QPSK, 16QAM, 64QAM for SL-U.

- To use 9dB as the A-MPR for NS_31 for 256QAM for SL-U.

- Table 1-1 A-MPR for NS_31 for SL-U:

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | See Table 6.2F.2-1 | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 9.0 | ≤ 9.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Band n96 NS_53

The NS_53 is for band n96. The corresponding requirements captured in TS 38.101-1 Table 6.5F.3.3.5-1 and the PSD requirement is -1 dBm/MHz as captured in table 6.2F.1-2.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

Case 1,2,8,9 are for 20MHz; 3,4,10,11 are for 40MHz; 5,12 are for 60MHz; 6,13 are for 80MHz and 7,14 are for 100MHz.

Table 7 A-MPR for NS_53 single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 8.4 | 8.6 | 5.4 | 5.5 | 3.8 | 3.2 | 3.2 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 16QAM | 8.4 | 8.6 | 5.4 | 5.6 | 3.9 | 3.9 | 3.8 | 9.8 | 10.9 | 6.6 | 7.8 | 6.0 | 4.5 | 3.9 |
| 64QAM | 8.3 | 8.5 | 5.5 | 5.4 | 5.6 | 5.5 | 5.5 | 9.8 | 10.9 | 6.7 | 7.8 | 6.1 | 4.6 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 9.8 | 10.9 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 8 A-MPR for NS_53 Wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.5 | 5.5 | 5.5 | 5.8 | 4.0 | 4.0 | 4.0 | 2.8 |
|  | 16QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.6 | 5.8 | 5.6 | 5.6 | 5.6 | 5.7 | 4.0 | 4.0 | 4.0 | 3.6 |
|  | 64QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 5.8 | 5.5 | 5.5 | 5.5 | 5.7 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 8.5 | 8.5 | 8.6 | 8.5 | 8.6 | 8.6 | 8.5 | 8.6 | 7.7 | 7.3 | 7.3 | 7.3 | 8.5 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 10.9 | 10.9 | 11.0 | 10.9 | 10.9 | 10.9 | 10.9 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.2 | 6.4 | 6.4 | 5.1 |
|  | 16QAM | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 11.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.5 | 6.3 | 5.1 |
|  | 64QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 10.9 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.3 | 6.4 | 6.3 | 5.2 |
|  | 256QAM | 11.0 | 11.0 | 11.0 | 10.9 | 11.0 | 10.9 | 11.0 | 11.0 | 8.1 | 8.0 | 8.0 | 8.0 | 8.0 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 3 NS_53 Single CC A-MPR

Figure 4 NS_53 Wide-band A-MPR

The A-MPR requirement is captured in sub-clause 6.2F.3.6 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.2F.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [3.5] | ≤ [4.5] |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [4.0] | ≤ [4.5] |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ [5.5] | ≤ [5.5] |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

The reason why the structure is like the above captured table is that the PSD limit as -1dBm/MHz which dominates the A-MPR requirement and hence different channel bandwidth has different limit. Our simulation results also show similar trend as the legacy NR-U A-MPR requirement.

Based on the table 7 above, we can observe that for QPSK and 16QAM, the PSD is the dominate factor and hence the simulation result is quite aligned with the NR-U limit. However, for 256QAM and one case 5 for 64QAM with full RB allocation, the A-MPR is dominate by EVM and the number are quite similar with our simulation result for MPR and hence larger A-MPR values are needed.

For the wideband operation, similar observation can be found as single CC cases. For QPSK, 16QAM and 64QAM, the A-MPR is dominated by PSD and the legacy NR-U limit can cover the A-MPR. However, for 256QAM especially for the contiguous RB allocation cases, the EVM will dominate A-MPR and hence larger A-MPR is needed.

Observation 2

- For NS_53, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_53 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_53 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Proposal 2:

- To reuse the legacy A-MPR requirement for NS_53 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_53 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_53 for Inter-laced RB allocation.

- Table 2-1 A-MPR for NS_53 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [3.5] | ≤ [4.5] |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [4.0] | ≤ [4.5] |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ [5.5] | ≤ [5.5] |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 9.0 | ≤ 8.5 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 | ≤ [9.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Band n96 NS_60

The NS_60 is for band n96. The corresponding requirements captured in TS 38.101-1 Table 6.5F.3.3.5-1 and the PSD requirement is 2 dBm/MHz as captured in table 6.2F.1-2.

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 9 A-MPR for NS_60 single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.4 | 5.6 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 6.7 | 8.0 | 3.5 | 4.7 | 3.2 | 3.3 | 3.2 |
| 16QAM | 5.4 | 5.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 6.8 | 8.0 | 3.5 | 4.8 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.3 | 5.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 6.8 | 8.0 | 4.1 | 4.7 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 8.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 10 A-MPR for NS_60 wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 3.6 | 3.6 | 3.7 | 3.6 | 3.6 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 5.5 | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.5 | 5.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 3.1 | 3.3 | 3.4 | 2.7 |
|  | 16QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.0 | 5.0 | 5.0 | 5.1 | 3.2 | 3.5 | 3.3 | 2.8 |
|  | 64QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 5.1 | 5.1 | 5.1 | 5.1 | 5.1 | 4.1 | 4.1 | 4.1 | 4.0 |
|  | 256QAM | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 8.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 5 NS_60 Single CC A-MPR

Figure 6 NS_60 Wideband CC A-MPR

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.2F.3.10-1: A-MPR for NS_60 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: For larger channels than 80MHz the A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |  |  |  |  |  |  |

Similar to NS_53, the dominate factor is the PSD limit as 2 dBm/MHz and hence we can see the requirement are differentiated by different channel bandwidth. The simulation results also shows the same trend. When compared the simulation results and the legacy NR-U requirement, for interlaced RB allocation, the NR-U limit can cover the simulation results. For full RB allocation, for QPSK, 16QAM and 64QAM, the NR-U limit can cover the simulation results. For full RB allocation, 256QAM, higher A-MPR as 9dB is needed.

Observation 3

- For NS_60, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_60 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_60 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Proposal 3:

- To reuse the legacy A-MPR requirement for NS_60 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_60 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_60 for Inter-laced RB allocation.

- Table 2-2 A-MPR for NS_60 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 8.5 | ≤9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: For larger channels than 80MHz the A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |  |  |  |  |  |  |

Band n96 NS_61

The NS_61 is for band n96. The corresponding requirements captured in TS 38.101-1 Table 6.5F.3.3.7-1 and the PSD requirement is 1 dBm/MHz as captured in table 6.2F.1-2. Although the additional spectrum emission is as low as -34 which is very stringent, however, the ASE requirement starting frequency has 20MHz compared to the frequency band edge. In such case, still the PSD requirement dominate the A-MPR and hence the requirement is also based on different channel bandwidth.

Table 6.5F.3.3.7-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -34 | 1 MHz |
| f ≥ 6445 | -34 |  |

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 11 A-MPR for NS_61 single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6.4 | 6.6 | 3.4 | 3.5 | 3.5 | 3.5 | 3.5 | 7.8 | 8.9 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 16QAM | 6.4 | 6.7 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 7.8 | 9.0 | 4.5 | 5.8 | 4.0 | 3.6 | 3.5 |
| 64QAM | 6.3 | 6.6 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 7.8 | 9.0 | 4.6 | 5.8 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 9.0 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 12 A-MPR for NS_61 wideband

|  | Bitmap | 10 | 100 | 010 | 1000 | 0100 | 10000 | 01000 | 00100 | 110 | 1100 | 0110 | 11000 | 01100 | 1110 | 11100 | 01110 | 11110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 6.5 | 6.5 | 6.6 | 6.5 | 6.5 | 6.5 | 6.5 | 6.6 | 3.7 | 3.5 | 3.7 | 3.5 | 3.7 | 3.0 | 2.8 | 2.8 | 3.2 |
|  | 16QAM | 6.5 | 6.5 | 6.6 | 6.5 | 6.6 | 6.5 | 6.5 | 6.7 | 3.7 | 3.6 | 3.7 | 3.6 | 3.7 | 3.6 | 3.5 | 3.7 | 3.6 |
|  | 64QAM | 6.6 | 6.5 | 6.6 | 6.6 | 6.5 | 6.6 | 6.6 | 6.6 | 4.8 | 4.7 | 4.9 | 4.7 | 4.8 | 4.9 | 4.8 | 5.0 | 5.0 |
|  | 256QAM | 7.4 | 7.5 | 8.5 | 7.4 | 7.5 | 7.4 | 7.4 | 8.5 | 7.7 | 7.3 | 8.5 | 7.3 | 7.7 | 8.0 | 7.6 | 8.5 | 8.0 |
| Interlace | QPSK | 8.9 | 9.0 | 9.0 | 9.0 | 8.9 | 9.0 | 8.9 | 9.0 | 6.0 | 5.9 | 5.9 | 5.9 | 6.0 | 4.2 | 4.4 | 4.4 | 3.2 |
|  | 16QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 5.9 | 6.0 | 6.0 | 4.3 | 4.5 | 4.3 | 3.3 |
|  | 64QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.0 | 6.0 | 6.0 | 6.0 | 6.0 | 4.3 | 4.4 | 4.3 | 4.0 |
|  | 256QAM | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 9.0 | 6.8 | 6.8 | 6.8 | 6.8 | 6.8 | 6.7 | 6.7 | 6.7 | 6.6 |

Figure 7 NS_61 Single CC A-MPR

Figure 8 NS_61 Wideband A-MPR

The A-MPR requirement is captured in sub-clause 6.2F.3.11 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.2F.3.11-1: A-MPR for NS_61 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |

Similar observation and proposals can be found for NS_61 as NS_60 and the observations and proposals are listed as below:

Observation 4

- For NS_61, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_61 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_61 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Proposal 4:

- To reuse the legacy A-MPR requirement for NS_61 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_61 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_61 for Inter-laced RB allocation.

- Table 2-3 A-MPR for NS_60 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤9.0 | ≤ 10.5 | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |

Band n102 NS_58

The NS_58 is for band n102. The corresponding requirements captured in TS 38.101-1 Table 6.5F.3.3.7-1 and the PSD requirement is 1 dBm/MHz as captured in table 6.2F.1-2. Although the additional spectrum emission is as low as -34 which is very stringent, however, the ASE requirement starting frequency has 20MHz compared to the frequency band edge. In such case, still the PSD requirement dominate the A-MPR and hence the requirement is also based on different channel bandwidth.

Table 6.5F.3.3.7-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -34 | 1 MHz |
| f ≥ 6445 | -34 |  |

The A-MPR requirement is captured in sub-clause 6.2F.3.10 and the requirement is differentiated by different channel bandwidth as captured below:

Table 13  A-MPR for NS_58 single CC

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 3.2 | 3.1 | 3.3 | 3.2 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.4 | 4.3 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

The wide-band operation simulation result is further provided below:

Table 14 A-MPR for NS_58 wideband

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Figure 9 NS_61 Single CC A-MPR

Figure 10 NS_61 Wideband A-MPR

The A-MPR requirement is captured in sub-clause 6.2F.3.8 and the requirement is differentiated by different channel bandwidth as captured below:

Table 6.2F.3.8-1: A-MPR for NS_58 power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

For NS_58, the PSD requirement is 10dBm/MHz hence it will not dominate the A-MPR. For the out-of-band emissions there is 10MHz guard band and also the requirement is -22dBm/MHz which is also not the dominate factor. For QPSK, 16QAM and 64QAM, the legacy NR-U requirement can apply. For 256QAM where the EVM dominate, higher A-MPR as 9dB is needed.

Observation 5

- For NS_58, the PSD and out-of-band emission requirement do not dominate the A-MPR.

- The legacy NR-U A-MPR requirement for NS_58 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation and Interlaced RB allocation.

Proposal 5:

- To reuse the legacy A-MPR requirement for NS_58 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- To use 9dB as the A-MPR for NS_58 for 256QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- Table 3-1 A-MPR for NS_58 for SL-U:

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

How to capture A-MPR requirement

During the last meeting discussion, we have proposed a discussion paper discussing how to treat the situation that the A-MPR requirement for the NR-U already using NS values are different compared to SL-U. From our understanding, currently the A-MPR requirement for 256QAM for PSSCH and PSCCH are different between NR-U and SL-U. Even though for other modulation as QPSK, 16QAM and 64QAM they are the same. Furthermore, the exception table of bitmaps are also different.

Observation 6: The 256QAM A-MPR requirement for each NS value are different between NR-U and SL-U.

Observation 7: The exception table of bitmaps is different between NR-U and SL-U.

Furthermore, we also observed that for A-MPR and MPR requirement as well, there are further PSFCH and S-SSB requirement will be defined which is not the case in NR-U A-MPR and MPR requirement. In this case, reusing the NS values of NR-U in SL-U seems not a proper way.

Observation 8: Besides PSSCH/PSCCH A-MPR requirement, there are A-MPR requirement for PSFCH and S-SSB to be defined.

Based on the above observation 6 to 8, we believe the new NS values for SL-U is needed.

Proposal 6: To define new NS values for SL-U.

About how to define such NS values, we believe current method for NTN is a proper way. Below captures the mapping of network signaling label for NTN NS values.

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

The basic principle is for each band, if the A-MPR requirements are the same, then the same NS value can be reused as NS_24 and NS_100 for band n256. However, for band n255, new A-MPR requirement is introduced and NS_02N is used specifically.

With that, it is proposed below mapping of network signaling label for SL-U:

| NR SL unlicensed band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_02S |  |  |  |  |  |  |
| n96 | NS_01 | NS_03S | NS_04S | NS_05S |  |  |  |  |
| n102 | NS_01 | NS_06S |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

The corresponding requirements mapping are as below:

| SL-U NS values | NR-U NS values |
| --- | --- |
| NS_02S | NS_31 |
| NS_03S | NS_53 |
| NS_04S | NS_60 |
| NS_05S | NS_61 |
| NS_06S | NS_58 |

Proposal 7: To agree on new NS values for SL-U.

| NR SL unlicensed band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_02S |  |  |  |  |  |  |
| n96 | NS_01 | NS_03S | NS_04S | NS_05S |  |  |  |  |
| n102 | NS_01 | NS_06S |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

3	Conclusions

In this contribution, we give discussion on the A-MPR for 5 NS values and the observation and proposals are shown as below:

Observation 1:

- For NS_31, 64QAM, interlaced RB allocation, the A-MPR is about 1 to 1.5dB higher then corresponding full RB allocation values.

- With limited PSD, the A-MPR for QPSK and 16QAM in interlaced RB allocation needs larger A-MPR.

- The legacy NR-U A-MPR requirement for NS_31 can cover QPSK, 16QAM, 64QAM for SL-U.

Observation 2 :

- For NS_53, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_53 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_53 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Observation 3 :

- For NS_60, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_60 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_60 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Observation 4:

- For NS_61, because of the PSD requirement dominates the A-MPR in most cases, the requirement will be differentiated by different channel bandwidth as legacy for NR-U.

- The legacy NR-U A-MPR requirement for NS_61 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- The legacy NR-U A-MPR requirement for NS_61 can cover SL-U for Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation.

Observation 5 :

- For NS_58, the PSD and out-of-band emission requirement do not dominate the A-MPR.

- The legacy NR-U A-MPR requirement for NS_58 can cover QPSK, 16QAM, 64QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- For 256QAM where EVM dominate the A-MPR, larger number of A-MPR then NR-U is needed for Full RB allocation and Interlaced RB allocation.

Observation 6: The 256QAM A-MPR requirement for each NS value are different between NR-U and SL-U.

Observation 7: The exception table of bitmaps is different between NR-U and SL-U.

Observation 8: Besides PSSCH/PSCCH A-MPR requirement, there are A-MPR requirement for PSFCH and S-SSB to be defined.

Proposal 1:

- To reuse the legacy A-MPR requirement for NS_31 for QPSK, 16QAM, 64QAM for SL-U.

- To use 9dB as the A-MPR for NS_31 for 256QAM for SL-U.

- Table 1-1 A-MPR for NS_31 for SL-U:

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | See Table 6.2F.2-1 | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 9.0 | ≤ 9.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

Proposal 2:

- To reuse the legacy A-MPR requirement for NS_53 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_53 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_53 for Inter-laced RB allocation.

- Table 2-1 A-MPR for NS_53 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [3.5] | ≤ [4.5] |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ [4.0] | ≤ [4.5] |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ [5.5] | ≤ [5.5] |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 9.0 | ≤ 8.5 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 | ≤ [9.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3:

- To reuse the legacy A-MPR requirement for NS_60 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_60 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_60 for Inter-laced RB allocation.

- Table 2-2 A-MPR for NS_60 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 8.5 | ≤9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: For larger channels than 80MHz the A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |  |  |  |  |  |  |

Proposal 4:

- To reuse the legacy A-MPR requirement for NS_61 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation.

- To use 9dB as the A-MPR for NS_61 for 256QAM for SL-U for Full RB allocation.

- To reuse the legacy A-MPR requirement for NS_61 for Inter-laced RB allocation.

- Table 2-3 A-MPR for NS_60 for SL-U:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤9.0 | ≤ 10.5 | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 | ≤ 7.0 | ≤ 9.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |

Proposal 5:

- To reuse the legacy A-MPR requirement for NS_58 for QPSK, 16QAM, 64QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- To use 9dB as the A-MPR for NS_58 for 256QAM for SL-U for Full RB allocation and Interlaced RB allocation.

- Table 3-1 A-MPR for NS_58 for SL-U:

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 9.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

Proposal 6: To define new NS values for SL-U.

Proposal 7: To agree on new NS values for SL-U.

| NR SL unlicensed band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_02S |  |  |  |  |  |  |
| n96 | NS_01 | NS_03S | NS_04S | NS_05S |  |  |  |  |
| n102 | NS_01 | NS_06S |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

4 References

[1] TR 38.786