3GPP TSG-RAN WG4 Meeting # 108bis														R4-2316125

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-1 SL-U inter-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The draftCR is to capture the configred TX power for inter-band concurrent operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The configured TX power for inter-band conccurent operation is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirementsfor configured TX power for inter-band concurrent operation is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

6.2E.4	Configured transmitted power for V2X

6.2E.4.1	General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c,  PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + TIB,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X,  PRegulatory,c }

where

-	PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331

-	For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows:

PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + TIB,c , P-MPRc), PRegulatory,c}

PCMAX_H,f,c = MIN {PPowerClass, V2X,  PRegulatory,c}

-	For the total transmitted power PCMAX,PSFCH, PEMAX,c is the value given by IE sl-maxTransPower when single resource pool configured is transmitted at a given time and sum of the IEs sl-maxTransPower when multiple resource pools configured are transmitted at a given time, defined by TS 38.331.

-	PPowerClass,V2X is the maximum UE power specified in Table 6.2E.1.1-1 without taking into account the tolerance specified in the Table 6.2E.1.1-1;

-	MPRc and A-MPRc for serving cell c are specified in clause 6.2E.2 and clause 6.2E.3 for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	TIB,c,  and P-MPRc are specified in clause 6.2.4

-	PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [12] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied.

For NR V2X UE supporting SL MIMO or Tx Diversity, the transmitted power is configured per each UE.

For NR V2X UE with two transmit antenna connectors at the same time, the tolerance is specified in Table 6.2E.4.1-1. The requirements shall be met with SL MIMO configurations specified in Table 6.2D.1-2.

Table 6.2E.4.1-1: PCMAX,c tolerance schemes for MIMO

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

6.2E.4.2	Configured transmitted power for inter-band V2X con-current operation

When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,NR and PCMAX,c,V2X for the configured NR uplink carrier and the configured NR V2X carrier, respectively, and its total configured maximum output power PCMAX,c.

The configured maximum output power PCMAX c,NR(p) in slot p for the configured NR uplink carrier shall be set within the bounds:

PCMAX_L,c,NR (p) ≤  PCMAX,c,NR (p) ≤  PCMAX_H,c,NR (p)

where PCMAX_L,c,NR and PCMAX_H,c,NR are the limit as specified in clause 6.2E.4.1.

The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR V2X carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in clause 6.2E.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,NR (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,NR are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,NR (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the NR uplink and NR V2X carriers is

PUMAX = 10 log10 [pUMAX,c,NR + pUMAX,c,V2X],

where pUMAX,c,NR  denotes the measured output power of serving cell c for the configured NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X carrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2E.4.1-1.. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

6.2E.4.3	Configured transmitted power for intra-band V2X con-current operation

For intra-band con-current operation, if transmission of Uu and SL does not overlap in time, the configured output power PCMAX,c  specified in clause 6.2E.4.1 and 6.2.4 apply for SL and Uu transmission respectively; otherwise, if transmission of Uu and SL overlap in time, the configured maximum output power PCMAX,c  on serving cell c for SL and Uu shall be set as specified in clause 6.2E.4.1 and in clause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2E.2.3 for both PC3 and PC2 and subclause 6.2E.3.4, respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For intra-band concurrent operation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PPowerClass,con-current – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PPowerClass,con-current}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for Uu serving cell c or by IE sl-MaxTransPower for SL defined in [7];

-	PPowerClass,con-current is the maximum UE power specified in Table 6.2E.1.2-2 without taking into account the tolerance;

-	MPR and A-MPR are specified in clause 6.2E.2 and 6.2E.3, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in clause 6.2E.4.3

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c;

For intra-band concurrent operation, when at least one different numerology/slot pattern is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of slot numerology type i, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in slot p of serving cell c(i) on slot numerology type i shall be set within the following bounds:

PCMAX_L,f,c(i),i (p) ≤  PCMAX,f,c(i), i (p) ≤  PCMAX_H,f,c(i),i (p)

where PCMAX_L,f,c (i),i (p) and PCMAX_H,f,c(i),i (p) are the limits for a serving cell c(i) of slot numerology type i as specified in clause 6.2.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of slot numerology or symbol pattern i,  and a slot q of slot numerology or symbol pattern j that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

When slots p and q have different transmissions lengths and belong to different cells on same band for intra-band operation:

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,f,c(i),Uu,i (p) + pCMAX_L,f,c(i),V2X,j (q)], PPowerClass,con-current}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_ H,f,c(i), Uu,,i (p) + pCMAX_ H,f,c(i),V2X,j (q)], PPowerClass,con-current}

where pCMAX_L,f,c (i),Uu,i  and pCMAX_ H,f,c(i),Uu,i  are the respective limits PCMAX_L,f,c (i),Uu,i and PCMAX_H,f,c(i),Uu,i expressed in linear scale.

TREF and Teval are specified in Table 6.2E.4.3-1 when same and different slot patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. PPowerClass,Concurrent shall not be exceeded by the UE during any period of time.

Table 6.2E.4.3-1: PCMAX evaluation window for different slot and channel durations

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| TREF of largest slot duration over both UL and SL CCs | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured maximum output power PUMAX over all serving cells with same slot pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2E.4.3-2. The tolerance TL is the absolute value of the lower tolerance for applicable NRV2X concurrent operation configuration as specified in Table 6.2 E.1.2-2 for intra-band NR V2X concurrent operation.

The measured maximum output power PUMAX over all serving cells, when at least one slot has a different transmission numerology or slot pattern, shall be within the following range:

P'CMAX_L–  MAX{TL, TLOW (P'CMAX_L)} ≤  P'UMAX  ≤  P'CMAX_H + THIGH (P'CMAX_H)

P'UMAX = 10 log10 ∑ p'UMAX,c

where p'UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P'CMAX) and THIGH(P'CMAX) for applicable values of P'CMAX are specified in Table 6.2E.4.3-2. The tolerance TL is the absolute value of the lower tolerance for applicable NR V2X concurrent operation configuration as specified in Table 6.2E.1.2-2 for intra-band NR V2X concurrent operation.

where:

P'CMAX_L  = MIN{ MIN {10log10∑( pCMAX_L,f,c(i),i), PPowerClass,concurrent} over all overlapping slots in TREF}

P'CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass,concurrent} over all overlapping slots in TREF}

Table 6.2E.4.3-2: PCMAX tolerance for SL intra-band con-current operation

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 26 ≤ PCMAX < 23 | 3 | 2 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

A UE supporting sidelink operation can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

For sidelink resource allocation in either mode 1 or mode 2, if UE is in RRC_CONNECTED state, and the preparation procedure time for transmission of sidelink physical channel is available before of PUSCH preparation procedure time, for transmission of Uu and SL not overlap in time, the configured output power PCMAX,c  specified in clause 6.2E.4.1 and in clause 6.2.4 apply for SL and Uu transmission respectively, otherwise, the configured maximum output power PCMAX specified in this clause shall apply.

For sidelink resource allocation mode 2, if UE is in RRC_IDLE state, sidelink transmission is based on pre-configured sidelink resource pool, the UE configured output power is determined by sidelink only, where the configured output power specified in clause 6.2E.4.1 apply.

For sidelink resource allocation mode 2, if UE is in RRC_INACTIVE state, and Uu does not support SDT, the configured output power specified in clause 6.2E.4.1 apply, otherwise, the configured maximum output power PCMAX in this clause shall apply.

6.2E.4F	Configured transmitted power for Sidelink Unlicensed

6.2E.4F.2	Configured transmitted power for inter-band con-current operation

- When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,NR and PCMAX,c,V2X for the configured NR uplink carrier and the configured NR V2X carrier, respectively, and its total configured maximum output power PCMAX,c.

- The configured maximum output power PCMAX c,NR(p) in slot p for the configured NR uplink carrier shall be set within the bounds:

- PCMAX_L,c,NR (p) ≤  PCMAX,c,NR (p) ≤  PCMAX_H,c,NR (p)

- where PCMAX_L,c,NR and PCMAX_H,c,NR are the limit as specified in TS 38.101-1 clause 6.2E.4.1.

- The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR V2X carrier shall be set within the bounds:

- PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

- where PCMAX_H,c,V2X is the limit as specified in TS 38.101-1 clause 6.2E.4.

- The total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

- PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

- with

- PCMAX_L (p,q) =  min{10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)] - ΔTIB,c, Ppowerclass,concurrent}

- PCMAX_H (p,q) = min{10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)], Ppowerclass,concurrent}

- where pCMAX_H,c,V2X and pCMAX_H,c,NR are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,NR (p) expressed in linear scale.

- Ppowerclass,concurrent is the power class for inter-band concurrent operation defined in subclause 6.2.1.

- The measured total maximum output power PUMAX over both the NR uplink and NR V2X carriers is

- PUMAX = 10 log10 [pUMAX,c,NR + pUMAX,c,V2X],

- where pUMAX,c,NR  denotes the measured output power of serving cell c for the configured NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X carrier expressed in linear scale.

- When a UE is configured for synchronous V2X sidelink and uplink transmissions,

- PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

- where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2E.4.1-1.. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

<<end of change>>