3GPP TSG-RAN WG4 Meeting # 108-bis                               R4-2316140

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.9.2.2

Source:	China Telecom

Title:	Discussion on Case 1 requirements for Pre-MG and concurrent MG

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed further and the WF [1] was approved.

In this paper, we present our views about Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG issues captured in the WF [1].

- Discussion

Regarding Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG, there are issues about collision handling being discussed in this paper.

Collision handling

Status in the WF [1]:

- Issue 3-2-2: [Case 1] - [Scenario 1] Further clarification on the agreement from scenario 1? < Background > - The collision scenario in this issue is depicted in the figure below: < Agreement >: - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG and UE should continue the measurement within the MG#2 - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used - FFS the exact wording to be captured in the specification in CR draft directly. Issue 3-2-3: [Case 1] - [Scenario 2] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) - Background: - The collision scenario in this issue is depicted in the figure below: < Way forward >: - FFS the options: - Option 1: - Same agreement as in (scenario 1), which is Agreement of Issue 3-3-2 from WF [R4-2310175]. - Option 2: - When a pre-MG and a Type-2 MG collide and the pre-MG has higher priority, UE should drop the colliding Type-2 MG occasion, if - the deactivation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the Type-2 MG occasion and ends at 4ms after the Type-2 MG occasion, and - the deactivation procedure of pre-MG ends earlier than the start of pre-MG occasion. - Option 2a: Additionally, - the Pre-MG will be deactivated immediately after the Pre-MG deactivation procedure. - data scheduling is expected within the MG occasion colliding with the Pre-MG deactivation procedure and the Pre-MG occasion after Pre-MG deactivation procedure. Issue 3-2-4: [Case 1] - [Scenario 3] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG - Background: - The collision scenario in this issue is depicted in the figure below: < Way forward>: - Option 1: Same agreement as issue 3-3-2 (scenario 1). - Option 2: The UE continues the measurement within the overlapped concurrent gap occasion (MG#2), i.e. existing priority rule applies without any change.

The collision handing rule was agreed in last meeting for Scenario 1 that when the Pre-MG activation procedure is overlapped with one of concurrent gap occasion and Pre-MG has higher priority than the MG. In our understanding, since the Pre-MG status shall be applied 5ms after the overlapping MG, the Pre-MG activation/deactivation delay requirement is expected to be extended.

Proposal 1: For Scenario 1, since the Pre-MG status shall be applied 5ms after the overlapping MG, the Pre-MG activation/deactivation delay requirement is expected to be extended.

For Scenario 2 that when the Pre-MG deactivation procedure is overlapped with one of concurrent gap occasion and Pre-MG has higher priority than the MG, in our views, the collision handing rule follows the same logic with the rule of Pre-MG activation procedure.

Proposal 2: For Scenario 2, when the Pre-MG deactivation procedure is overlapped with one of concurrent gap occasion and Pre-MG has higher priority than the MG, the collision handing rule follows the same logic with the rule of Pre-MG activation procedure.

For Scenario 3 that when the Pre-MG activation procedure is overlapped with one of concurrent gap occasion and the MG has higher priority than the Pre-MG, the existing gap collision dropping rule can be applied directly, that is, UE performs measurements in the occasion of MG with higher priority and the occasion of Pre-MG with lower priority is dropped.

Proposal 3: For Scenario 3, when the Pre-MG activation procedure is overlapped with one of concurrent gap occasion and the MG has higher priority than the Pre-MG, the existing gap collision dropping rule cWan be applied directly, that is, UE performs measurements in the occasion of MG with higher priority and the occasion of Pre-MG with lower priority is dropped.

- Conclusion

The contribution presents our views about Case 1 RRM requirements for the combination of Pre-configured MG and concurrent MG, with the following proposals:

Proposal 1: For Scenario 1, since the Pre-MG status shall be applied 5ms after the overlapping MG, the Pre-MG activation/deactivation delay requirement is expected to be extended.

Proposal 2: For Scenario 2, when the Pre-MG deactivation procedure is overlapped with one of concurrent gap occasion and Pre-MG has higher priority than the MG, the collision handing rule follows the same logic with the rule of Pre-MG activation procedure.

Proposal 3: For Scenario 3, when the Pre-MG activation procedure is overlapped with one of concurrent gap occasion and the MG has higher priority than the Pre-MG, the existing gap collision dropping rule can be applied directly, that is, UE performs measurements in the occasion of MG with higher priority and the occasion of Pre-MG with lower priority is dropped.

- References

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc., RAN WG4 Meeting #108, August, 2023.