3GPP TSG-RAN WG4 Meeting #108bis              	R4- 2316146

Xiamen, China, 9th Oct-13th Oct, 2023

Agenda item:	5.12.4.3

Source:	Samsung

Title:	Discussion and simulation results for PDSCH requirements with multi-Rx reception

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [1].  Regarding the deployment scenario and test scope of PDSCH requirement for multi-Rx reception are made as

- Requirements need to be introduced for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirement - Define single requirement for scenario A and scenario B in Bi-directional deployment scenario for PDSCH demodulation requirements, considering Scenario B-1 for requirement Parameters; Description; Scenario B-1. Ds; Inter-RRH distance; 700m. Dmin; Distance between rail track and RRH; 150m. v; Train velocity; 350km/h. fd; Maximum Doppler frequency shift; 9722Hz. fc; Carrier frequency; 30GHz. Drx_panel; Distance between two UE Rx panels; 0m. Ds_offset; Switching transmission point between adjacent RRHs; 100m. - General for PDSCH requirement with multi-Rx reception - UE processing assumption for the FFT window - Encourage companies to evaluate the performance difference with assumption on FFT (single FFT across Rx chains, and separate FFT per RF chain) - Test setup for PDSCH requirement with multi-Rx reception - Transmission schemes - Do not consider single-DCI based multi-TRP scheduling for PDSCH requirement - PDSCH resource scheduling for requirements - Only cover full overlapping - Layer combination for full overlapping - 2+2 - PDSCH rate matching in mTRP transmission - PT-RS allocation does not overlap with PDSCH allocation per TRP - PDSCH scheduling and Number of DMRS in TDD DL special slot - PDSCH scheduling in the special DL slot is assumed with 3 DMRS configuration - Number of SSB and TCI state configuration for each cell - Maximum 8 SSB and TCI states configuration for each cell is configured - Number of active TCI tracking - RAN4 to assume the baseline behaviour for FR2 HST UE under test is to track 2 Active TCI states, one per panel - TRS periodicity - RAN4 to choose 10ms TRS periodicity to align with existing FR2 HST requirements - PDSCH allocation timeline in the UE demod test - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. - The detail of scheduling pattern can be further discussed in the next meeting - Option 1: RAN4 to follow the same approach used in FR1 HST DPS requirements and FR2 HST DPS requirements (Bidirectional) and define a PDCCH/PDSCH allocation timeline independent per each Panel, including a number of slots in which PDCCH and PDSCH are DTX and throughput statistics are not considered, starting from the TCI state switch and until the UE has received one TRS from the new RRH (TfirstTRS) and has had time to process it (TTRSproc), including THARQ and TMAC proc; - Other options are not precluded - Applicability rule for PDSCH requirement with simultaneous multi-Rx reception - Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least: - UE power class 6 (PC6) - [simultaneousReceptionDiffTypeD-r16] - maxNumberActiveTCI-PerBWP > 1 - Additional UE necessary capabilities should be considered as - Multi-DCI based multi-TRP transmission - Overlapping PDSCHs in time and fully overlapping in frequency and time - Other UE capability could be considered if introduced in the RRM session

In this contribution, the view on remaining issue for PDSCH requirement with multi-Rx reception are provided. Meanwhile, the initial simulation results are provided for alignment purpose

# 2	Test setup for PDSCH requirement with multi-Rx reception

PDSCH allocation timeline

Regarding the PDSCH allocation timeline, it is related with the number of active TCI states.  As agreed in the last meeting, the baseline behavior for FR2 HST UE under test is to track 2 active TCI states, one per panel, which means that

- RAN4 to assume the baseline behavior for FR2 HST UE under test is to track 2 Active TCI states, one per panel

since only 1 TCI is activated per panel, similar as DPS scheme 1a in Bi-directional B of Rel-17 FR2 HST, UE can not do pre-tracking for new TCI state, then, additional delay for SSB processing required. In general, the overview period after receiving MAC CE activate TCI switching from the through statistic can be reused as

- THARQ+TMAC Proc+TfirstSSB + TSSB proc +TfirstTRSafterSSB+ TTRS pro

As agreed, the UE start position (t=0) is the coverage area of the first RRH as following (option1), where the -Ds_offset for the right panel of UE, and +Ds_offset  is switching point of left panel of UE,

Figure 1:  PDSCH allocation timeline

Assuming 350km/h UE speed and Ds_offset =100m, the exacting slots for scheduling TCI switching command can be slot#57600n for the right panel and slot#57600n+12698 for the left panel, respectively.

For left panel, since the SSB transmission period configuration is 80 slots in FR2 HST, then, the closest SSB transmission slots to scheduling TCI switching command using MCS4 should be #57600n+12800.

For the value of TSSB proc  and TTRS pro,  they are 16 slots.

As for TfirstSSB , it is calculated with min(SSB@slot#160n-THARQ-TMAC Proc) as 132, and TTRS pro , it is calculated with min(TRS@slot#(80n+5)- TSSB proc) as 69

Proposal 1: The overview period after receiving MAC CE activate TCI switching for each panel from the through statistic can be reused

- THARQ+TMAC Proc+TfirstSSB + TSSB proc +TfirstTRSafterSSB+ TTRS pro

- Scheduling TCI switching command can be slot#57600n for the right panel and slot#57600n+12800for the left panel,

- THARQ    = 4 (slots),  TMAC Proc = 24 (slots),  TSSB proc=16slots, and TTRS pro =16 (slots)

- TfirstSSB =132 (slots),

- TfirstTRSafterSSB =69 (slots)

Test applicability for PDSCH requirement with simultaneous multi-Rx reception

In the last meeting, the test applicability rule for simultaneous multi-Rx reception requirements was under discussion. Based on agreement made in RRM session, new UE capability [simultaneousReceptionFR2HST-r18] to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE as following

Issue 1-2-1: Whether need to define a new or reuse the existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception - Agreement: - Define a new UE capability [simultaneousReceptionFR2HST-r18] to indicate support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE - Details can be discussed in Rel-18 feature list discussion. - The conclusion from Rel-18 Multi-RX WI could be considered.

Therefore, the related test applicability rule can be updated as

- Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

  - UE power class 6 (PC6)

  - [simultaneousReceptionFR2HST -r18]

  - maxNumberActiveTCI-PerBWP > 1

- Additional UE necessary capabilities should be considered as

  - Multi-DCI based multi-TRP transmission

  - Overlapping PDSCHs in time and fully overlapping in frequency and time

Proposal 2: The related test applicability rule for PDSCH requirement with multi-Rx reception  can be updated as

- Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

  - UE power class 6 (PC6)

  - [simultaneousReceptionFR2HST -r18]

  - maxNumberActiveTCI-PerBWP > 1

- Additional UE necessary capabilities should be considered as

  - Multi-DCI based multi-TRP transmission

  - Overlapping PDSCHs in time and fully overlapping in frequency and time

# 3	Simulation results

In this section, the initial simulation results are provided based on initial simulation assumption in the last meeting.

Initial ideal results for multi-Rx reception with different scenarios

| Scenario | Antenna configuration per each TRP and panel | Doppler Frequency | BW | Layer | Rx Panel | (MCS) | SNR (70%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario B-1 | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Left | MCS 17 | 13.65 |
|  | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Right | MCS 17 | 13.7 |
|  | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Left | MCS 17 |  |
|  | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Right | MCS 13 |  |
|  | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Left | MCS 17 |  |
|  | 2x2 TDD | 9722Hz | 200MHz | 2+2 | Right | MCS 19 |  |

# 4	Conclusion

In this contribution, the view on remaining issue of test setup for FR2 HST PDSCH requirement with multi-Rx simultaneous reception in Bi-directional deployment for open space scenario was provided

Proposal 1: The overview period after receiving MAC CE activate TCI switching for each panel from the through statistic can be reused

- THARQ+TMAC Proc+TfirstSSB + TSSB proc +TfirstTRSafterSSB+ TTRS pro

- Scheduling TCI switching command can be slot#57600n for the right panel and slot#57600n+12800for the left panel,

- THARQ    = 4 (slots),  TMAC Proc = 24 (slots),  TSSB proc=16slots, and TTRS pro =16 (slots)

- TfirstSSB =132 (slots),

- TfirstTRSafterSSB =69 (slots)

Proposal 2: The related test applicability rule for PDSCH requirement with multi-Rx reception  can be updated as

- Simultaneous multi-Rx reception requirements are appliable for UE with the following capabilities at least:

  - UE power class 6 (PC6)

  - [simultaneousReceptionFR2HST -r18]

  - maxNumberActiveTCI-PerBWP > 1

- Additional UE necessary capabilities should be considered as

  - Multi-DCI based multi-TRP transmission

Overlapping PDSCHs in time and fully overlapping in frequency and time

# References

R4-2313919, WF for FR2 HST demodulation, RAN4#108, Samsung