# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316155

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.7.2.1

Source: 	OPPO

Title: 	On general aspects for FR2_multiRX_DL

Document for:	Approval

1	Introduction

A WF on Rel-18 FR2_multiRX_DL was approved in RAN4#108 meeting [1].

In this contribution, we further discuss the remaining issues related to scope and scenarios for FR2-1 multi-Rx reception.

2	Discussion

Applicability and conditions

#### Issue 1-3-3: Detectable condition of RS signals - The issue is covered by issue 2-1-4 for TCI state switch. #### Issue 1-3-5: UE behaviour when a condition becomes violated during a measurement - FFS if it is to define particular behavior when side conditions for multi-Rx operation are violated

The set of conditions for the L1 measurement requirements have been discussed for several meetings. We don’t see it is necessary to have general agreements on the set of conditions to be considered in the L1 measurement requirements. Different applicable conditions may need to be specified for different requirements. They can be discussed in the corresponding requirements and drafting CR. When side conditions for multi-Rx operation are violated, UE is not expected to meet the requirements of multi-Rx operation.

Proposal 1: When side conditions for multi-Rx operation are violated, UE is not expected to meet the requirements of multi-Rx operation.

UE capabilities

#### Issue 1-4-4: UE capability for simultaneous reception with different QCL typeD for L1 measurements - Option 1: - A new UE capability of supporting simultaneous reception of RS and data from different directions with different QCL type D RSs for enhanced L1 measurements for multi-Rx is introduced. - A new UE capability of supporting simultaneous reception of RS and RS from different directions with different QCL type D RSs for enhanced L1 measurements for multi-Rx is introduced, if measurement restriction is enhanced for multi-Rx. - Option 2: - specify UE capability to indicate whether the UE can support simultaneous reception of data and L1 measurement - Option 3: - RAN4 to define new additional UE capability to indicate support of Rel-18 multi-Rx DL simultaneous reception. #### Issue 1-4-5: UE capability for indication of whether additional delay is needed in dual TCI state switching for multi-Rx - FFS: - To indicate whether additional delay is needed in dual TCI state switching. By default, a UE should support dual TCI state switching without the additional delay. #### Issue 1-4-6: UE feature list - UE capabilities related to multi-Rx operations shall be discussed in UE feature list.

Since Rel-16 UE capability simultaneousReceptionDiffTypeD-r16 is applicable only for PDSCH reception, we slightly prefer to introduce a new capability in R18 to make the UE capability clearer. Most companies are fine with the UE capability of simultaneous reception of L1 RS and data. We also support to specify UE capability to indicate whether the UE can support simultaneous reception of data and L1 measurement, but whether it is an independent UE capability or it is included in the feature of Multi-Rx operation can be discussed together with R18 UE feature list. We slightly prefer not to define additional UE capability based on the capability of Rel-18 multi-Rx DL simultaneous reception.

Proposal 2: Specify UE capability to indicate whether the UE can support simultaneous reception of data and L1 measurement. Prefer not to define additional UE capability based on the capability of Rel-18 multi-Rx DL simultaneous reception.

3	Conclusion

Proposal 1: When side conditions for multi-Rx operation are violated, UE is not expected to meet the requirements of multi-Rx operation.

Proposal 2: Specify UE capability to indicate whether the UE can support simultaneous reception of data and L1 measurement. Prefer not to define additional UE capability based on the capability of Rel-18 multi-Rx DL simultaneous reception.

4	Reference

[1] R4-2314477, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo