3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316172

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.22.2.3

Source: 	OPPO

Title: 	Discussion on LPHAP use case

Document for:	Approval

1	Introduction

As per the new WID on Expanded and Improved NR Positioning, LPHAP use case 6 will be considered in Rel-18. And RAN4 reached some agreements on PRS measurements in RRC INACTIVE and IDLE states [1].

2	Discussion

2.1	eDRX in RRC INACTIVE

Issue 1-1-2: TDRX in PRS requirements with RAN eDRX <= 10.24s - Wayforward - Option 1: - When CN eDRX <= 10.24 s, TDRX is defined as T in Rel-17 TS 38.304. - When CN eDRX > 10.24 s, TDRX is set to the maximum of the T inside and outside of the CN PTW, where T inside and outside of the CN PTW are defined in Rel-17 TS 38.304. - Option 2: - Based on T calculation in clause 7.1 of TS 38.304

For PRS measurements in RRC INACTIVE state, how to derive TDRX based on eDRX configuration is discussed and the above two options are proposed during the last meeting. Compared with option 2, option 1 additional considers the case where T inside and outside of the CN PTW could be different when CN eDRX cycle > 10.24s, and the maximum of T inside and outside of the CN PTW is used as in this case. This method aligns with the current procedure in TS 38.304 and should be supported.

Proposal 1: In RRC INACTIVE state, support option 1 to derive the TDRX in PRS requirements:

- When CN eDRX <= 10.24 s, TDRX is defined as T in Rel-17 TS 38.304.

- When CN eDRX > 10.24 s, TDRX is set to the maximum of the T inside and outside of the CN PTW, where T inside and outside of the CN PTW are defined in Rel-17 TS 38.304.

Issue 1-1-3: Tavailabe in PRS requirements with RAN eDRX > 10.24s - Wayforward - Option 1: - Tavailable is defined as PRS periodicity TPRS. - Option 2: - Tavailable is LCM (TPRS,i, TDRX), and TDRX is the maximum of the DRX cycles within the CN PTW and the RAN PTW, according to RAN2 definitions of UE DRX cycle in Rel-18 eRedCap. - Option 3: - FFS

- Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as: Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Where: - Ti corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], - Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX - TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

The next issue is how to derive Tavailable for PRS measurements in RRC inactive state. As shown above, the existing Tavailable in Rel-17 is defined as LCM(TPRS,i, TDRX) where TDRX is the configured DRX cycle length. In Rel-18, the only difference is that DRX is further extended to eDRX. With the clear solution to derive TDRX based on eDRX configuration in issue 1-1-2, the equation Tavailable = LCM (TPRS,i, TDRX) can be reused.

Proposal 2: In RRC INACTIVE state, Tavailable = LCM (TPRS,i, TDRX), and TDRX is derived according to proposal 1.

2.2	SRS positioning validity area

Issue 1-2-1: Determination of UL timing to transmit SRS (related to RAN1 LS R1-2306248) - Question 1: whether it is feasible for UE to autonomously adjust the TA when cell-reselection happens - Agreement (from AH session) - It is feasible for UE to autonomously adjust the TA when cell-reselection happens within the validity area. - Question 2: how UE should autonomously adjust the TA when cell-reselection happens, if confirmed to be feasible - Wayforward (from AH session) - FFS: How UE should autonomously adjust the TA when cell-reselection happens, if confirmed to be feasible: - Option 1: UE autonomously adjusts the TA based on twice of the DL timing difference, and use the requirements for one shot large UL timing adjustment in clause 7.1.2.3 as baseline. - Option 2: autonomously adjusts the TA based on the DL timing difference, to maintain the same UL timing immediately before and after each cell reselection. - Question 3: thresholds for the one-shot autonomous TA adjustment, if agreed to be applied - Wayforward (from AH session) - Threshold for the one-shot autonomous TA adjustment is FFS. - Question 4: UL timing requirements for the subsequent SRS transmission after the one-shot autonomous TA adjustment, if agreed to be applied - Wayforward (from AH session) - UL timing requirements for the subsequent SRS transmission after the one-shot autonomous TA adjustment, if agreed to be applied is FFS

#### 7.1.2.3 One shot large UL timing adjustment for FR2 Power Class 6 UE When highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and [largeOneStepUL-timingFR2-r17] capability, the following requirements apply to the UE: - If the absolute value Told-Tnew≤CP/4, the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch. - Otherwise, the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply. - The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew). Above, - Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state. - Told (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

Regarding the UL timing to transmit SRS captured in RAN1 LS R1-2306248, RAN4 reached a conclusion that UE autonomous TA adjustment when cell-reselection happens within the validity area is feasible. The details on how to adjust TA and UL timing requirements need further discussion. For question 2 on how does UE autonomously adjust the TA, the methodology for FR2 HST can be used as baseline, i.e. to adjust TA based on twice of the DL timing difference as captured in clause 7.1.2.3.

Proposal 3: UE autonomously adjusts the TA based on twice of the DL timing difference.

For FR2 HST, threshold = CP/4 is used to determine whether to apply one-shot large UL timing adjustment. Similarly, such the condition can be used for SRS transmission in our view.

Proposal 4: Define threshold=CP/4 to apply autonomous TA adjustment.

Two different UL timing requirements are considered for FR2 HST, Te when the new target TCI is within active TCI list and ±7*64*Tc otherwise. And ±7*64*Tc is a little bit smaller than 2/CP=9*64*Tc for 120kHz numerology to guarantee the demodulation performance of UL transmission. Therefore, we suggest CP/2- NMargin*Tc as the UL timing requirements for SRS transmission when cell reselection happens.

Proposal 5: UL timing requirements could be CP/2 - NMargin*Tc.

Issue 1-2-2: Impact of SRS positioning validity area - Wayforward - Option 1: - UE restarts Rx-Tx time difference measurement after autonomous adjustment of UL timing. - Option 2: - RAN4 shall study the RRM impact including the applicability of UE Rx-Tx measurement requirements once the design in RAN1 and RAN2 is stable.

Generally, autonomous TA adjustment will lead to large step UL timing change. For example, if threshold=CP/4 in proposal 4 is agreed, then the UL timing adjustment would be larger than CP/2 (2*threshold) and is comparable to timing adjustment due to TA command. It is not reasonable to average Rx-Tx time difference measurements before and after autonomous TA adjustment and UE should restart measurements.

Proposal 6: UE should restart Rx-Tx time difference measurement after autonomous adjustment of UL timing.

2.3	PRS measurement in RRC IDLE

Issue 1-3-1: Tavailable in PRS requirements - Wayforward - Option 1: - for both cases with eDRX <= 10.24s and eDRX > 10.24s, TDRX is modified to follow the RAN2 definition of T in Rel-17 TS 38.304.

Compared with RRC inactive state, T definition in RRC IDLE state is much simple and T is only defined within PTW when CN eDRX is larger than 10.24s as shown above. Option 1 can be supported.

Proposal 7: In RRC IDLE state, TDRX is modified to follow the RAN2 definition of T in Rel-17 TS 38.304.

3	Conclusion

This contribution gave our considerations on defining RRM requirements for LPHAP use case 6 and the following proposals:

Proposal 1: In RRC INACTIVE state, support option 1 to derive the TDRX in PRS requirements:

- When CN eDRX <= 10.24 s, TDRX is defined as T in Rel-17 TS 38.304.

- When CN eDRX > 10.24 s, TDRX is set to the maximum of the T inside and outside of the CN PTW, where T inside and outside of the CN PTW are defined in Rel-17 TS 38.304.

Proposal 2: In RRC INACTIVE state, Tavailable = LCM (TPRS,i, TDRX), and TDRX is derived according to proposal 1.

Proposal 3: UE autonomously adjusts the TA based on twice of the DL timing difference.

Proposal 4: Define threshold=CP/4 to apply autonomous TA adjustment.

Proposal 5: UL timing requirements could be CP/2 - NMargin*Tc.

Proposal 6: UE should restart Rx-Tx time difference measurement after autonomous adjustment of UL timing.

Proposal 7: In RRC IDLE state, TDRX is modified to follow the RAN2 definition of T in Rel-17 TS 38.304.

4	Reference

- R4-2314462, WF on R18 NR Positioning – LPHAP, Huawei, HiSilicon