# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316185

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.30.3.1

Source:	OPPO

Title:	On remaining RRM issues for SL CA

Document for:	Discussions

1	Introduction

In RAN4#108 meeting, a WF was approved for SL-CA RRM and some issues were identified for further study in [1].

Issue 1-1: Interruption location of the interruption to WAN at SL carrier addition/release - Agreements - For Mode 2 SL CA operation do not specify the exact interruptions location - FFS if constraints on interruptions during WAN paging or SIB reception shall be introduced Issue 1-2: Interruption length of the interruption to WAN at SL carrier addition/release - Agreements - Interruption length is NR Slot length (ms); Interruption length (slots). 0; 1; 2. 1; 0.5; 3. 2; 0.25; 5. Issue 1-3: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation - Agreements - Reuse LTE TS 36.133 clause 13.10 RRM requirements of V2X (re)selection of Synchronization reference source for NR SL CA operation. - Note: the agreement can be revisited in case of additional RAN1/2 agreements on NR SL CA operation

In this contribution, we provide our further views on remaining issues of SL-CA RRM requirements.

2	Discussions

In LTE spec TS 36.133, RAN4 did not consider the constraints on any interruption requirements during WAN paging or SIB reception, including interruptions to WAN due to Sidelink Carrier Aggregation.

In NR, RAN4 agreed on interruption requirements to WAN carriers when NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, and not to specify the exact interruptions location. However, whether constraints on interruptions during WAN paging or SIB reception shall be introduced still needs to be clarified.

As for Mode 2 UE, UE could be in idle/inactive or connected mode, and perhaps receive paging or SIB information.

As SL-DRX is newly introduce for NR and not defined for LTE, constraints on interruptions during receiving paging or SIB were specified when RAN4 defined interruptions to WAN at transitions between active and non-active in section 12.7.4 in TS 38.133. And paging or SIB reception are periodized. The logic can also apply for SL carrier addition or release.

In our view, it is valid that SL CA addition or release are colliding with receiving paging or SIB for UE. For this case, the similar constraints for interruption due to SL CA addition/release can also apply. Thus, we propose to introduce the applicability rule for interruptions to WAN due to SL CA.

Proposal 1: Introduce the applicability for interruptions to WAN due to Sidelink Carrier Aggregation during WAN paging or SIB reception.

Proposal 2: The proposed text can be considered for CR of interruption requirements:

When NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

3	Summary

In this contribution we have discussed the remaining issues for supporting sidelink CA. Based on the discussion, we have made following proposals:

Proposal 1: Introduce the applicability for interruptions to WAN due to Sidelink Carrier Aggregation during WAN paging or SIB reception.

Proposal 2: The proposed text can be considered for CR of interruption requirements:

When NR SL UE performs SL carrier addition/release using the RRCConnectionReconfiguration message, the interruptions in this clause shall not apply when one of the following conditions is met:

-	While receiving paging,

-	While receiving system information.

References

R4-2314350, “WF on SL evolution RRM requirements – SL CA and co-channel co-existence”, OPPO