3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2316200

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Simulation result for ATG non-synchronized scenarios

Agenda Item:	5.13.1.4

Document for:	Approval

Introduction

In the RAN4#108 meeting, the way forward [1] on simulation for ATG non-synchronized scenarios was approved. In this paper, we’d like to share our simulation results for ATG non-synchronized scenarios assuming FSPL model between ATG BS and TN BS.

Discussion

Referring to TR 38.876 [2], we have the following non-synchronized scenarios.

Table 1 non-synchronized scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 4 GHz |  | Phase 2 |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 4 GHz |  | Phase 2 |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | Phase 2 |
| NOTE 1: ACLR/ACS requirement for ATG BS and ATG UE are derived based on the synchronized scenario in Phase 1. NOTE 2: the non-synchronized coexistence scenarios listed in phase 2 are mainly used to identify the isolation distance between ATG BS and the the legacy TN BSs. And it’s agreed to reuse 45dB ACLR and 46dB ACS for both TN and ATG BS for non-synchronized scenarios to derive isolated distance as starting point. NOTE 3: For scenario 11, if simulator doesn’t support additional ring of TN network, i.e. additional 18 site and 54 cells, it’s allowed to double the aggregate interference power from the 57 TN cells to simplify the platform modification, i.e. interference +3dB. NOTE 4: For scenario 6, 8 and 13 which is CLI between ATG UE and TN UE, it’s not expected to have further coexistence simulation evaluation in Phase 2 due to low interference level. |  |  |  |  |  |  |  |  |

Simulation results assuming FSPL model between BSs

Scenario 5: ATG DL -> TN UL (4GHz)

The simulation results for scenario 5 were provided below assuming 5km isolation distance. The throughput loss is 2.6% which is less than 5%. Thus, there is no serious throughput loss for scenario 5.

(a)                                             (b)

Figure 2 (a) CDF of UL SINR for TN BS from ATG BS, (b) CDF of UL throughput for TN BS from ATG BS

Scenario 7: TN rural DL -> ATG UL (4GHz)

The initial simulation results for scenario 7 are provided below. It’s assumed that Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero. And 5km, 100km and 1000km are used to derive the initial simulation results.

(a)                                             (b)

Figure 1 (a) CDF of UL couple loss, (b) CDF of ATG UE UL transmitter power

(a)                                             (b)

(c)

Figure 2 CDF of UL SINR, (a) 5km isolation distance, (b) 100km isolation distance, (c) 1000km isolation distance

(a)                                             (b)

(c)

Figure 3 CDF of UL Throughput, (a) 5km isolation distance, (b) 100km isolation distance, (c) 1000km isolation distance

Table 2 the performance loss for different isolation distance

| Isolation distance | 5km | 100km | 1000km |
| --- | --- | --- | --- |
| SINR at 50% CDF point | -8dB | 2 | 7dB |
| SINR at 5% CDF point | -30dB | -10 | 1dB |
| Throughput loss at 5% CDF point | 100% (51.589Mbps loss) | 85.8% (43.698Mbps loss) | 6.7% (3.48Mbps loss) |

Scenario 14: TN rural DL -> ATG UL (2GHz)

The initial simulation results for scenario 14 were provided in the contribution [3] below. This is a resubmission. It’s assumed that Angle between ATG BS boresight and nearest TN BS boresight in azimuth is zero. And 10km, 100km and 1000km are used to derive the initial simulation results.

Generally, the CDFs of UL couple loss and ATG UE UL transmitter power are shown below.

(a)                                             (b)

Figure 1 (a) CDF of UL couple loss, (b) CDF of ATG UE UL transmitter power

(a)                                             (b)

(c)

Figure 2 CDF of UL SINR, (a) 10km isolation distance, (b) 100km isolation distance, (c) 1000km isolation distance

(a)                                             (b)

(c)

Figure 3 CDF of UL Throughput, (a) 10km isolation distance, (b) 100km isolation distance, (c) 1000km isolation distance

Table 2 the performance loss for different isolation distance

| Isolation distance | 10km | 100km | 1000km |
| --- | --- | --- | --- |
| SINR at 50% CDF point | -8dB | 7dB | 14.5dB |
| SINR at 5% CDF point | -25dB | -2.5dB | 11dB |
| Throughput loss at 5% CDF point | 100% (40.22Mbps loss) | 87.7% (32.295Mbps loss) | 24.8% (9.99Mbps loss) |

Observations

Observation 1: There is no serious throughput loss for scenario 5.

Observation 2: For scenario 7 and 14, throughput loss larger than 5% can be observed even assuming FSPL model between ATG BS and TN BS, even if 100km isolation distance is set.

Observation 3: For scenario 14, the reason why such large throughput loss can be observed is that ATG single carrier scenario without adjacent channel interference has a good throughput performance. As the UL Tx power for ATG UE is restricted, ATG UE can transmit higher power to ease the interference from TN network.

Summary

Observation 1: There is no serious throughput loss for scenario 5.

Observation 2: For scenario 7 and 14, throughput loss larger than 5% can be observed assuming FSPL model between ATG BS and TN BS, even if 100km isolation distance is set.

Observation 3: For scenario 14, the reason why such large throughput loss can be observed is that ATG single carrier scenario without adjacent channel interference has a good throughput performance. As the UL Tx power for ATG UE is restricted, ATG UE can transmit higher power to ease the interference from TN network.

- References

[1] R4-2314755, WF on ATG co-existence evaluation, CMCC

[2] TR 38.876, Study on Air-to-ground network for NR, v0.5.0

[3] R4-2312960, Discussion on simulation assumption to derive the isolation distance for ATG scenario, Huawei, HiSilicon