3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2316203

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on ATG UE Rx requirements

Agenda Item:	5.13.2.3

Document for:	Approval

Introduction

In the RAN4#108 meetings, one way forward [1] were approved with the following assumptions to further discuss the maximum input level requirements. In addition, out-of-band blocking requirements are still FFS. In this paper, we’d like to share our views on these issues.

|  | Omni-direction antenna | Antenna array |
| --- | --- | --- |
| Carrier frequency (GHz) | 2 | 4 |
| Distance (km) | TBD | TBD |
| Path Loss (dB) | TBD | TBD |
| BS output power (dBm) | 46 | 53 |
| BS antenna gain (dBi) | TBD | TBD |
| ATG UE antenna gain (dBi) | 0 | TBD |
| Maximum input level (dBm) | TBD | TBD |

Discussion on maximum output power requirements

Referring to the contribution [2], the following assumptions were proposed to further discuss the maximum input level requirements.

- ATG BS antenna gain: Referring to Table 6.2.3.1-1 of TR 38.876, we have the following assumption for ATG BS antenna model. ATG. Base Station Antenna Characteristics. Antenna pattern; TR 38.921. Element gain (dBi) (Note 2); 7.1. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 54º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements. Number of supported polarizations, P; 2. Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.9 of wavelength for V. Array Ohmic loss (dB) (Note 2); 2. Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 25. Base station maximum coverage angle in the horizontal plane (degrees); 120. Base station vertical coverage range (degrees) (Note 1); 25. Mechanical uptilt (degrees); 14. Based on that, we can assume BS antenna gain = 10*log10(8*8) + 7.1 = 25.2dB - ATG BS maximum output power: (For ATG UE maximum input level evaluation) 53dBm sum of two polarizations for 4GHz 46dBm sum of two polarizations for 2GHz - ATG UE antenna gain Referring to Table 6.2.3.2-1 of TR 38.876, we have the following assumption for ATG UE antenna model for 4GHz Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 90º for V. Element gain (dBi); 5 dBi. Horizontal/vertical front-to-back ratio (dB); 30dBc. Antenna polarization; Linear ±90º. Antenna array configuration (Row × Column x Polarization) (Note 4); (8x2x2) or (16x1x2). Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.5 of wavelength for V. UE antenna orientation; Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route. The flight route is pointed at the BS.. 4GHz ATG UE phased array antenna gain = 10*log10(16) + 5 = 17dB 2GHz ATG UE Omni-direction antenna gain = 0dB - Pathloss model: Free space Pathloss ( fc_GHz, MinimumDistance_m ) = 32.5 +20log10(fc_GHz)+20log10(MinimumDistance_m) - The assumption of minimum distance between ATG BS and ATG UE is 3km ~ 20km. - The maximum input level = ATG BS output power + ATG BS antenna gain – FSPL(Distance, Fc) + ATG UE antenna gain.

Referring to the template in the WF [1], we can get the following calculations and results.

Table 1 the results for maximum input level requirements

| Parameters | case 1 | case 2 | case 3 | case 4 | case 5 | case 6 |
| --- | --- | --- | --- | --- | --- | --- |
| center frequency GHz | 2 | 2 | 2 | 4 | 4 | 4 |
| Distance meter | 3000 | 3000 | 20000 | 3000 | 3000 | 20000 |
| Pathloss dB | 108.063 | 108.063 | 124.5412 | 114.0836 | 114.0836 | 130.5618 |
| ATG BS output power dBm | 46 | 46 | 46 | 53 | 53 | 53 |
| ATG BS antenna gain | 25 | 7 | 25 | 25 | 7 | 25 |
| ATG UE antenna gain | 0 | 0 | 0 | 17 | 5 | 17 |
| ATG UE body loss | 1 | 1 | 1 | 1 | 1 | 1 |
| maximum input level dBm | -38.063 | -56.063 | -54.5412 | -20.0836 | -50.0836 | -36.5618 |

Case 1 and case 4 can be considered as the worst cases. The minimum distance between ATG BS and ATG UE is 3000m and the maximum antenna gains for BS and UE are assumed. During the discussion in last meeting, some companies think that it’s impossible to assume the maximum antenna gain when the aircraft is over the ATG BS. Thus, the element gains for ATG BS and UE are assumed for case 2 and case 5.

20000 minimum distance is assumed for case 3 and case 6, so the maximum input level is -36.6 dBm for 4GHz and -54.5 for 2GHz.

Proposal 1: to use -35dBm (case 6) to derive the maximum input level for 4GHz and use -50dBm (case 3) as baseline to derive the maximum input level for 2GHz.

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Receiver characteristics specified at the antenna connector(s) | Receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -50 | -35 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -52 | -37 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

Discussion on out-of-band blocking requirements

Referring to the ITU-R M.2059-0 [3], Tables 1 and 2 of ITU-R M.2059-0 provide technical characteristics for representative analogue and digital FMCW radio altimeters. They are summarized below.

|  | Radio altimeter A1 | Radio altimeter A2 | Radio altimeter A3 | Radio altimeter A4 | Radio altimeter A5 | Radio altimeter A6 | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |
| Nominal centre frequency | 4 300 | 4 300 | 4 300 | 4 300 | 4 300 | 4 300 | MHz |
| Transmitted power | 0.600 | 1 | 0.1 to 0.25 | 100 | 5 | 40 | W (peak) |
| Modulation (FMCW or Pulsed) | FMCW | FMCW | FMCW | Pulsed | Pulsed | Pulsed |  |
| Pulse width | Not applicable | Not applicable | Not applicable | 130 | 200 | 75 | ns |
| 3 dB emission bandwidth | 110 | 162.8 | 171 | 8 | 7 | 15 | MHz |
| Transmitted PSD (dBm/MHz) | 7.4 | 7.9 | 1.65 | 41 | 28.5 | 34.3 | (dBm/MHz) |

|  | Radio altimeter D1 | Radio altimeter D2 | Radio altimeter D3 | Radio altimeter D4 | Units |
| --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |
| Nominal centre frequency | 4 300 | 4 300 | 4 300 | 4 300 | MHz |
| Transmitted power (peak) | 0.400 | 0.100 | 0.1 to 1 | 5 | W (peak) |
| Modulation | FMCW | FMCW | FMCW | Pulsed |  |
| Pulse width | Not applicable | Not applicable | Not applicable | 30 or 225 | ns |
| 3 dB emission bandwidth | 150 | 177 | 175 | 5 or 31 | MHz |
| Transmitted PSD (dBm/MHz) | 4.3 | -2.5 | 7.6 | 30 | (dBm/MHz) |

It can be observed that the Transmitted PSD for FMCW radio altimeters is about -2.5dBm/MHz ~ 7.9dBm/MHz. And the Transmitted PSD for Pulsed radio altimeters is about 28.5dBm/MHz ~ 41dBm/MHz. As the Radio altimeters and ATG UE could be installed together on the bottom of aircraft cabin, the OOB blocking should be considered as co-collocation scenario.

Generally, It’s assumed 30~40dB minimum coupling loss. Thus, we can get the OOB blocking requirement is 41dBm/MHz – 40 (minimum coupling loss) – 4dB (body loss) = -3dBm/MHz.

If we take 29 dBm/MHz as the transmitted power, then we can get the OOB blocking requirement is 29 dBm/MHz - 40 (minimum coupling loss) – 4dB (body loss) = -15dBm/MHz

Proposal 2: RAN4 discuss whether the band filter for ATG UE is even much better than the general filter for smartphone without size restriction. If so, that means the OOB blocking requirements for ATG UE can be tighten than smart phone from implementation perspective.

Proposal 3: RAN4 can further trade off the following options for OOB blocking requirements.

Option 1: if 41dBm/MHz is assumed for Radio altimeters, the OOB blocking requirement is -3dBm/MHz (41dBm/MHz – 40 minimum coupling loss – 4dB body loss)

Option 2: if 29dBm/MHz is assumed for Radio altimeters, the OOB blocking requirement is -15dBm/MHz (29dBm/MHz – 40 minimum coupling loss – 4dB body loss)

Proposal 4: Spurious response requirements can be scaled based on the OOB blocking requirements.

e.g. general UE: -15dBm OOB blocking requirements =>-44dBm Spurious response requirements

e.g. ATG UE: -3dBm OOB blocking requirements =>-32dBm Spurious response requirements

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: to use -35dBm (case 6) to derive the maximum input level for 4GHz and use -50dBm (case 3) as baseline to derive the maximum input level for 2GHz.

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Receiver characteristics specified at the antenna connector(s) | Receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -50 | -35 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -52 | -37 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

Proposal 2: RAN4 discuss whether the band filter for ATG UE is even much better than the general filter for smartphone without size restriction. If so, that means the OOB blocking requirements for ATG UE can be tighten than smart phone from implementation perspective.

Proposal 3: RAN4 can further trade off the following options for OOB blocking requirements.

Option 1: if 41dBm/MHz is assumed for Radio altimeters, the OOB blocking requirement is -3dBm/MHz (41dBm/MHz – 40 minimum coupling loss – 4dB body loss)

Option 2: if 29dBm/MHz is assumed for Radio altimeters, the OOB blocking requirement is -15dBm/MHz (29dBm/MHz – 40 minimum coupling loss – 4dB body loss)

Proposal 4: Spurious response requirements can be scaled based on the OOB blocking requirements.

e.g. general UE: -15dBm OOB blocking requirements =>-44dBm Spurious response requirements

e.g. ATG UE: -3dBm OOB blocking requirements =>-32dBm Spurious response requirements

- References

[1] R4-2314761, WF on NR_ATG_UERF_part2, Huawei, HiSilicon

[2] R4-2312965, Discussion on ATG UE Rx requirements, Huawei, HiSilicon

[3] ITU-R M.2059-0, Operational and technical characteristics and protection criteria of radio altimeters utilizing the band 4 200-4 400 MHz