3GPP TSG-RAN WG4 Meeting # 108bis                             R4-2316204

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce some consideration for OOBB requirements

Agenda Item:	5.13.2.3

Document for:	Approval

Introduction

It’s proposed to capture the following technical analysis for OOB blocking requirements.

- References

[1]

[2]

- Text proposal

- <<Start of Change for TR 38.876>>

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-230279, Air-to-ground network for NR, CMCC

[3]		ERC Recommendation 74-01: "Unwanted emissions in the spurious domain".

[4]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[5]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[6]	R4-2308745, Discussion on ATG UE RF Tx requirements, Ericsson

[7]	3GPP TS 38.101-5:  “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”.

[8]	ITU-R M.2059-0:  “Operational and technical characteristics and protection criteria of radio altimeters utilizing the band 4 200-4 400 MHz”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

- <<Next of Change>>

- 7.1.3.7 Out-of-Band blocking requirements/ Spurious response

Referring to the ITU-R M.2059-0 [8], Tables 1 and 2 of ITU-R M.2059-0 provide technical characteristics for representative analogue and digital FMCW radio altimeters. They are summarized below.

Table 7.1.3.7-1: Analogue radio altimeters

|  | Radio altimeter A1 | Radio altimeter A2 | Radio altimeter A3 | Radio altimeter A4 | Radio altimeter A5 | Radio altimeter A6 | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |
| Nominal centre frequency | 4 300 | 4 300 | 4 300 | 4 300 | 4 300 | 4 300 | MHz |
| Transmitted power | 0.600 | 1 | 0.1 to 0.25 | 100 | 5 | 40 | W (peak) |
| Modulation (FMCW or Pulsed) | FMCW | FMCW | FMCW | Pulsed | Pulsed | Pulsed |  |
| Pulse width | Not applicable | Not applicable | Not applicable | 130 | 200 | 75 | ns |
| 3 dB emission bandwidth | 110 | 162.8 | 171 | 8 | 7 | 15 | MHz |
| Transmitted PSD (dBm/MHz) | 7.4 | 7.9 | 1.65 | 41 | 28.5 | 34.3 | (dBm/MHz) |

Table 7.1.3.7-2: Digital radio altimeters

|  | Radio altimeter D1 | Radio altimeter D2 | Radio altimeter D3 | Radio altimeter D4 | Units |
| --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |
| Nominal centre frequency | 4 300 | 4 300 | 4 300 | 4 300 | MHz |
| Transmitted power (peak) | 0.400 | 0.100 | 0.1 to 1 | 5 | W (peak) |
| Modulation | FMCW | FMCW | FMCW | Pulsed |  |
| Pulse width | Not applicable | Not applicable | Not applicable | 30 or 225 | ns |
| 3 dB emission bandwidth | 150 | 177 | 175 | 5 or 31 | MHz |
| Transmitted PSD (dBm/MHz) | 4.3 | -2.5 | 7.6 | 30 | (dBm/MHz) |

It can be observed that the Transmitted PSD for FMCW radio altimeters is about -2.5dBm/MHz ~ 7.9dBm/MHz. And the Transmitted PSD for Pulsed radio altimeters is about 28.5dBm/MHz ~ 41dBm/MHz. As the Radio altimeters and ATG UE could be installed together on the bottom of aircraft cabin, the OOB blocking should be considered as co-collocation scenario.

- <<End of Change>>