# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316206

# Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF requirements for CA_n26(2A)

Agenda Item:	4.30.1.5

Document for:	Approval

Introduction

In the RAN#101 meeting, a revised WID [1] was approved to include more sub-1GHz band combinations, i.e. CA_n26(2A) and CA_n5A-n28A-n105A. As it is the first UL intra-band non-contiguous CA in FDD band that will be discussed in RAN4 for UL CA_n26(2A). The NR CA configurations are copied as below.

NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n5A-n8A; -; n5; 5, 10; 0. n8; 5, 10. CA_n5A-n28A; CA_n5A-n28A; n5; 5, 10, 15, 20; 0. n28; 5, 10, 15, 20, 30. CA_n8A-n20A-n28A; CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A; n8; 5, 10, 15, 20; 0. n20; 5, 10, 15, 20. n28; 5, 10, 15, 20. CA_n5A-n105A; CA_n5A-n105A; n5; 5, 10, 15, 20; 0. n105; 5, 10, 15, 20, 25, 30, 35. CA_n28A-n105A; -; n28; 5, 10, 15, 20, 25, 30; 0. n105; 5, 10, 15, 20, 25, 30, 35. CA_n26A-n28A; CA_n26A-n28A; n26; 5, 10, 15, 20, 25, 30; 0. n28; 5, 10, 15, 20, 25, 30. CA_n5A-n28A-n105A; CA_n5A-n105A CA_n5A-n28A; n5; 5, 10, 15, 20; 0. n28; 5, 10, 15, 20, 25, 30. n105; 5, 10, 15, 20, 25, 30, 35. CA_n26(2A); CA_n26(2A); n26; 5, 10, 15; 0.

The demands are from Telstra and the spectrum holding in band n5 and additional n26 is shown below.

figure 1 spectrum holding in band n5 and additional n26 from operator

In this paper, we’d like to analysis the impacts on RF requirements and potential RF architecture.

Discussion on MPR and AMPR requirements for UL CA_n26(2A)

In current clause 6.2A.2.2 of TS 38.101-1, the MPR requirements for intra-band non-contiguous CA were specified below considering both dual PA architecture and single PA architecture.

6.2A.2.2 UE maximum output power reduction for Intra-band non-contiguous CA 6.2A.2.2.0 General 6.2A.2.2.1 MPR to meet -30dBm/MHz 6.2A.2.2.1.1 PC3 with indicating dualPA-Architecture supported 6.2A.2.2.1.2 PC2 with indicating dualPA-Architecture supported 6.2A.2.2.1.3 PC3 without indicating dualPA-Architecture supported 6.2A.2.2.1.4 PC2 without indicating dualPA-Architecture supported 6.2A.2.2.2 MPR to meet -13dBm/MHz 6.2A.2.2.2.1 PC3 with indicating dualPA-Architecture supported 6.2A.2.2.2.2 PC2 with indicating dualPA-Architecture supported 6.2A.2.2.2.3 PC3 without indicating dualPA-Architecture supported 6.2A.2.2.2.4 PC2 without indicating dualPA-Architecture supported

Based on the spectrum holding of operator, single PA should support 30MHz bandwidth and the relative bandwidth is about 3.6% (30MHz / 832MHz). Comparing to intra-band non-contiguous CA for TDD n41 with 190 bandwidth, the relative bandwidth is about 7.3% (190MHz / 2590MHz). Thus, both dual PA architecture and single PA architecture can support UL CA_n26(2A).

Observation 1: both dual PA architecture and single PA architecture can support UL CA_n26(2A).

As the MPR requirements for intra-band non-contiguous CA have been specified, there is no need to develop new MPR requirements for UL CA_n26(2A).

Proposal 1: Current MPR requirements are applicable to UL CA_n26(2A).

In addition, in current spec, the ASE requirements for band n26 and n5 are listed below.

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_12 | 6.5.3.3.17 | n26 | 5,10 | Table 6.2.3.21-1 | Table 6.2.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.22-1 | Table 6.2.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10,15,20 | Table 6.2.3.23-1 | Table 6.2.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5,10,15,20 | Table 6.2.3.24-1 | Table 6.2.3.24-2 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |

Referring to the approved contribution [2], the following primary scenarios for additional spurious emission requirements from operators in US for band n26 are listed be

Range 1 (806 ≤ f ≤ 813.5) additional emissions requirements: - 5 MHz carrier in 814-819 MHz (SouthernLINC) - 10 MHz carrier in 814-824 MHz (SouthernLINC) There would be a slight change in the table from 36.101 for NS_12 Table 6.6.3.3.5-1: Additional requirements Frequency band (MHz); Channel bandwidth / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz, 10 MHz, 15 MHz. 806 ≤ f ≤ 813.5; -42; 6.25 kHz. NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB.. Range 2 (806 ≤ f ≤ 816) additional emissions requirements: - 5 MHz carrier in 817-822 MHz (Sprint) - 5 MHz carrier in 819-824 MHz (Sprint) This would align with NS_13 in 36.101 with the following change: Table 6.6.3.3.6-1: Additional requirements Frequency band (MHz); Channel bandwidth / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz. 806 ≤ f ≤ 816; -42; 6.25 kHz. NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 819 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB.. - 5 MHz carrier between 824 and 849 MHz - 10 MHz carrier between 824 and 849 MHz - 15 MHz carrier between 824 and 849 MHz - 20 MHz cattier between 824 and 849 MHz This would align with NS_14: Table 6.6.3.3.7-1: Additional requirements Frequency band (MHz); Channel bandwidth / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz, 10 MHz, 15 MHz. 806 ≤ f ≤ 816; -42; 6.25 kHz. NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 824 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB.. Range 3 (851 ≤ f ≤ 859) additional emissions requirements: - 5 MHz carrier between 814 and 824 MHz - 5 MHz carrier between 824 and 849 MHz - 10 MHz carrier in 814-824 MHz - 10 MHz carrier in 825-835 MHz (Region 2 A block) - 10 MHz carrier in 835-845 MHz (Region 2 B block) - 10 MHz carrier in 839-849 MHz - 15 MHz carrier in 824-839 MHz - 15 MHz carrier in 834-849 MHz - 20 MHz carrier in 824-844 MHz - 20 MHz carrier in 829-849 MHz These scenarios align with NS_15 in 36.101 with the following change: Table 6.6.3.3.8-1: Additional requirements Frequency band (MHz); Channel bandwidth / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz, 10 MHz, 15 MHz, 20 MHz. 851 ≤ f ≤ 859; -53; 6.25 kHz. NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB..

As it’s clearly clarified that CA_n26(2A) is not intended for use in the USA in the revised WID [1], there is no need to consider NS_12/ NS_13/ NS_14/ NS_15 for UL CA_n26(2A).

Note 2: CA_n26(2A) is not intended for use in the USA.

Proposal 2: NS_12/ NS_13/ NS_14/ NS_15 are not applicable to UL CA_n26(2A) as interest scenarios are different from original n26 single carrier scenarios in NA.

As far as I know, there is no need to consider the coexistence scenario between NR and UTRA in band n26 in Australia. Thus, NS_100 are not applicable to UL CA_n26(2A).

Proposal 3: NS_100 is not applicable to UL CA_n26(2A) in Rel-18.

Discussion on MSD requirements for UL CA_n26(2A)

Based on the current specification TS 38.101-1, the REFSENS degradation for intra-band non-contiguous CA_n26(2A) with one UL configuration is shown below based on the technical inputs [3][4][5].

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

The potential factors which may have impacts on the DL REFSENS for CA_n26(2A) are listed below, e.g. ACLR2, IMD3/IMD5 from UL carrier 1 and UL carrier 2.

figure 2 scenario 1 based on operator’s spectrum holding

figure 3 scenario 2 based on operator’s spectrum holding

Thus, it’s proposed to consider the following cases for MSD evolution.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 10MHz + 10MHz | Wgap = 10.0 | TBD | TBD | TBD | FDD |
| CA_n26(2A) | 15/15 | 10MHz + 15MHz | Wgap = 5.0 | TBD | TBD | TBD | FDD |

Proposal 4: the following test configurations can be used for UL CA_n26(2A) MSD evolution.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 10MHz + 10MHz | Wgap = 10.0 | TBD | TBD | TBD | FDD |
| CA_n26(2A) | 15/15 | 10MHz + 15MHz | Wgap = 5.0 | TBD | TBD | TBD | FDD |

Summary

Observation 1: both dual PA architecture and single PA architecture can support UL CA_n26(2A).

Proposal 1: Current MPR requirements are applicable to UL CA_n26(2A).

Proposal 2: NS_12/ NS_13/ NS_14/ NS_15 are not applicable to UL CA_n26(2A) as interest scenarios are different from original n26 single carrier scenarios in NA.

Proposal 3: NS_100 is not applicable to UL CA_n26(2A) in Rel-18.

Proposal 4: the following test configurations can be used for UL CA_n26(2A) MSD evolution.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 10MHz + 10MHz | Wgap = 10.0 | TBD | TBD | TBD | FDD |
| CA_n26(2A) | 15/15 | 10MHz + 15MHz | Wgap = 5.0 | TBD | TBD | TBD | FDD |

- References

[1] RP-232680, Revised WID on enhancement for 700/800/900MHz band combinations, CATT

[2] R4-1913078, n26 Emissions requirements, Sprint, SouthernLINC, Dish Network

[3] R4-2303574, TP for 38.718-01-01 to include CA_n26(2A), Ericsson, Telstra

[4] R4-2300938, CA_n26(2A) MSD analysis for TR 38.718-01-01, Skyworks Solutions Inc.

[5] R4-2302616, CA_n26(2A) ?RIBNC, Murata Manufacturing Co Ltd.