3GPP TSG-RAN WG4 Meeting # 108-bis 	R4-2316240

Xiamen, China, October 09th – 13th, 2023

Agenda Item: 5.9.5

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for measurements without gaps

Document for: Discussion

# Introduction

The work item description (WID) [1] indicates that the performance part should start from RAN4 meeting 108-bis in October 2023. Therefore, in this paper we discuss the principles to define the test cases.

# Discussion

From Rel-17 MG enhancements test cases discussion [2][3][4] and defined test cases, the following principles are down-selected:

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- Only define test case under non-DRX

- Define test case without SBI reporting.

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

Therefore, RAN4 can take the above principles from Rel-17 at starting point and further study what functionalities to be tested for NeedForGap and inter-RAT.

Additionally, RAN4 can consider

- No measurement gaps are configured in all test cases.

- Merge no-gap-with-interruption and no-gap-no-interruption into the same test cases. Check whether caused interruptions align with UE’s capability reporting

- RAN4 shall take the following Rel-17 principles from MGE as starting point to define test cases for Rel-18 NeedForGap and inter-RAT gapless measurements and FFS what functionalities for Rel-18 MGE-2 part 2 need to be tested.

- Do not introduce the test for L1 impact - Do not introduce test cases for intra-freq measurement without gap - Define a minimum set of test cases for SSB-based measurement - Define test case without SBI reporting. - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations - Do not define test cases with simultaneously FR1 and FR2 gaps configured. - Test cases are limited to single serving carrier - No measurement gaps are configured in all test cases - Merge no-gap-with-interruption and no-gap-no-interruption into the same test cases. Check whether caused interruptions align with UE’s capability reporting

Regarding the list of test cases for NeedforGap, we think the list agreed in [4] for NCSG can be used as a baseline with the test cases on SCC removed. We provide a tentative list as below

| No. | Test case | Test setup and scenario | Purpose of test |
| --- | --- | --- | --- |
| 2-1-1 | Event triggered reporting test on intra-frequency in FR1 | - Pcell (Cell1) on F1 and unknown neighbor cell (Cell2) on F1. The SSBs of Cell1 and Cell2 are not within UE’s active BWP - F1 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test | - Intra-frequency cell search delay is met for Cell2, and - UE receives data in Cell1 with interruption aligned with UE’s capability reporting |
| 2-1-2 | Event triggered reporting test on intra-frequency in FR2 | - Pcell (Cell1) on F1 and unknown neighbor cell (Cell2) on F1. The SSBs of Cell1 and Cell2 are not within UE’s active BWP - F1 in FR2 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test | - Same as in test case 2-1-1 |
| 2-1-3 | Event triggered reporting test on inter-frequency in FR1 | - Pcell (Cell1) on F1 and unknown inter-frequency neighbor cell (Cell2) on F1 - Both F1 and F2 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test | - Inter-frequency cell search delay is met for Cell2, and - UE receives data in Cell1 with interruption aligned with UE’s capability reporting |
| 2-1-4 | Event triggered reporting test on inter-frequency in FR2 | - Same as in test case 1 except that F1 and F2 are in FR2 | - Same as in test case 2-1-3 |
| 2-1-5 | Event triggered reporting test on inter-RAT LTE in FR1 | - Pcell (Cell1) on F1 and unknown LTE neighbor cell (Cell2) on F2 - Both F1 and F2 in FR1 - non-DRX, AWGN - RS to measure: CRS - Data scheduled on Cell1 during the whole test | - Inter-RAT LTE cell search delay is met for Cell2, and - UE receives data in Cell1 with interruption aligned with UE’s capability reporting |

Regarding the test case for inter-RAT measurements. We provide a tentative list as below

| No. | Test case | Test setup and scenario | Purpose of test |
| --- | --- | --- | --- |
| 2-2-1 | Event triggered reporting test on inter-RAT NR in FR1 | - LTE Pcell (Cell1) on F1 and unknown NR neighbor cell (Cell2) on F2. - Both F1 and F2 in FR1 - non-DRX, AWGN - RS to measure: SSB w/o SBI reporting - Data scheduled on Cell1 during the whole test | - Intra-RAT cell search delay is met for Cell2, and - UE receives data in Cell1 with interruption aligned with UE’s capability reporting |
| 2-2-2 | Event triggered reporting test on inter-RAT LTE in FR1 | - NR Pcell (Cell1) on F1 and unknown LTE neighbor cell (Cell2) on F2. - Both F1 and F2 in FR1 - non-DRX, AWGN - RS to measure: CRS - Data scheduled on Cell1 during the whole test | - Same as in test case 2-2-1 |
| 2-2-3 | Event triggered reporting test on coexisting inter-RAT LTE in FR1 | - NR Pcell (Cell1) on F1 and unknown LTE neighbor cell (Cell2) on F1. The channel BW of Cell2 is within UE’s active BWP - F1 in FR1 - non-DRX, AWGN - RS to measure: CRS - Data scheduled on Cell1 during the whole test | - Inter-frequency cell search delay is met for Cell2, and - UE receives data in Cell1 meeting scheduling restriction requirements |

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall take the following Rel-17 principles from MGE as starting point to define test cases for Rel-18 NeedForGap and inter-RAT gapless measurements and FFS what functionalities for Rel-18 MGE-2 part 2 need to be tested.

- Only define test case in NR SA in both FR1 and FR2

- Do not introduce the test for L1 impact

- Do not introduce test cases for intra-freq measurement without gap

- Define a minimum set of test cases for SSB-based measurement

- Only define test case under non-DRX

- Define test case without SBI reporting.

- On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations

- Do not define test cases with simultaneously FR1 and FR2 gaps configured.

- Test cases are limited to single serving carrier

- Only use mandatory gap patterns to define test cases

- Focus on only fully non-overlp and partially partial overlap in the test case design

- Verify gap dropping behaviour without introducing additional test cases

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2210586, “WF on R17 NR MG enhancements – Pre-configured MG”, Intel, e-meeting, May 2022.

- R4-2210585, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MTK, e-meeting, May 2022.

- R4-2210588, “WF on NCSG”, Apple, e-meeting, May 2022.