3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316251

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On the applicability of NS signalling for aerial UEs

Agenda item:	5.35.1

Document for:	Discussion

Introduction

The UAV WI, as presented in [1], includes objectives to address the ECC decision on additional emission requirements for aerial UEs. These objectives are highlighted below:

5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information.

This objective is split into two parts: identify the subset of UEs to which the ECC requirements apply (i.e. aerial UEs) in the network and convey the information to these users that they must respect the additional requirements when operating in an area covered by the ECC rule.

RAN4 has previously reached agreements about what characterizes an aerial UE, on how a feature-specific NS signalling is preferred [2]. RAN4 also instructed RAN2 [3] to provide associated signalling, although the preferred framework (“multiNS” or “new dedicated IE”) is left for RAN2 discussion, as it is on realm of RAN2 attributions.

The proposals presented in this technical contribution aims at provide further clarification on RAN4 definitions and to advance the RAN4 specification work in parallel with the signaling design being elaborated by RAN2.

Discussion

On the characterization of an aerial UE.

Characterization of an aerial UE

In RAN4 #107, an agreement was reached such that an aerial UE is defined from RAN4 point of view according to the box below[4]:

Agreement at RAN4#107 It was discussed what definition to add for an aerial UE/UAV device in RAN4 specification Agreement: - Uncrewed Arial UE: [A UE supporting UAS (Uncrewed Aircraft Systems) capability (Replace with reference to RAN2 specification where the capability will be defined), and have an aerial subscription as described in TS 25.401]

However, from our point of view, one further clarification is needed on this definition. The requirement for the applicability of the additional requirements to observe the subscription type is paramount in our point of view. The reason is that if this extra applicability rule is not added, all UEs that merely possess the capability for aerial features will be restrained by the additional transmission requirements, even if their end user never deploys it in a UAV.

It is our understanding that  the UE can straightforward verify the first applicability rule, as the UE is aware of its capabilities at all times. For the second rule, though, there might be a problem as the UE might be unaware of its subscription type. It is though, mandatory that a UE that supports aerial features shall be capable of perform authentication and authorization of the aerial subscription [5]. After the UE has received a message indicating the successful authentication of its aerial subscription, the UE becomes aware of its aerial status. This procedure seems in-line with the ECC decision [6]:

“According to this ECC Decision, an aerial UE refers to a UE supporting UAS features and services and requiring an aerial subscription. An aerial UE is installed either on-board an Unmanned Aircraft (e.g. drones) or on-board manned aircraft (e.g. helicopter). It identifies itself to the mobile network as being in this class.”

Therefore, we propose:

Add a clarification on the applicability of the additional requirements for aerial UEs that “the UE is considered to have an aerial subscription after the UE has performed a successful authentication and authorization of the aerial subscription”.

On multi-carrier configurations

Aerial UEs are not a different type of UEs, except that they are capable of using the aerial features. They are not precluded to implement any other features available for the regular UEs. This means that an aerial UE might be configured with Carrier Aggregation (CA) or with NR-DC operation.

We think that to avoid alternative interpretations and future misunderstandings our doubts about precedence of requirements regarding the different suffixes in TS 38.101-1 that the specification provide clarification on the applicability of the requirements when one of the bands (n3/3, n27/7) is configured with other bands. Therefore, we propose:

Clarify that when the aerial UE is configured with CA, with at least one of the CC including n3(3) and/or n7/7 or n38/38 the UE shall comply with the additional aerial requirements to this (these) CCs, and legacy requirements apply for the carriers in the remainder bands.

Clarify that when the aerial UE is configured with NR-DC, with at least one of the carriers includ n3(3) and/or n27/7 the UE will apply the aerial requirements to the respective band, and legacy requirements apply for the configured carriers in the remainder bands.

Conclusion

In this paper we discussed the applicability of NS signalling for aerial UEs to indicate the need for further spectrum emission requirements. Upon our discussion and with further review of RAN2 specifications, the following proposals were made:

Proposal 1: Add a clarification on the applicability of the additional requirements for aerial UEs that “the UE is considered to have an aerial subscription after the UE has performed a successful authentication and authorization of the aerial subscription”.

Proposal 2: Clarify that when the aerial UE is configured with CA, with at least one of the CC including n3(3) and/or n7/7 or n38/38 the UE shall comply with the additional aerial requirements to this (these) CCs, and legacy requirements apply for the carriers in the remainder bands.

Proposal 3: Clarify that when the aerial UE is configured with NR-DC, with at least one of the carriers includ n3(3) and/or n27/7 the UE will apply the aerial requirements to the respective band, and legacy requirements apply for the configured carriers in the remainder bands.

- References

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)

- R4-2314699, “WF on [108][152] NR_LTE_UAV”, Nokia, Nokia Shanghai Bell, RAN 4 #108, Toulouse, France, August 21 – August 25, 2023

- R4-2314700, “LS on the handling of additional regulatory requirements for UAV UEs”, Ericsson, RAN 4 #108, Toulouse, France, August 21 – August 25, 2023

- R4-2310487, “WF on NR_LTE_UAV”, Nokia, RAN4 #107, Incheon, Korea, May 22-26, 2023

- TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”, Release 18.3.0, 3GPP, July 2023.

- RP-230025/R4-2304033, Reply to LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, ETSI TC MSG/TFES