3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316253

Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on NS signalling for UAVs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV |  |  |  |  |  | Date: |  |  | 2023- |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implementing new signaling for further RF requirements for UAVs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the NS signalling in tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Aerial UEs cannot be informed they are within the authority region of ECC where further RF requirements are to be applied for out of band emissions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Change #1>

## 4.3	Specification suffix information

Unless stated otherwise, the  suffixes shown in Table 4.3-1 are used for indicating at 2nd level clause. For shared spectrum channel access, suffices A, B, and D are used for indicating at 3rd level clause.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation(CA) with UL MIMO |
| I | RedCap |
| J | Aerial UEs |

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to J) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to J) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.

A terminal which supports advanced V2X services, public safety services and other commercial use cases related to NR sidelink operation shall meet all of the separate corresponding requirements in suffix E.

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For a terminal that supports public safety service using sidelink, the minimum requirements are applicable when

-	The UE is associated with a serving cell on PS carrier, or

-	The UE is not associated with a serving cell on the PS carrier and is provisioned with the preconfigured radio parameters for PS that are associated with known Geographical Area, or

-	The UE is associated with a serving cell on a carrier different than the PS carrier, and the radio parameters for PS that are provided by the serving cell, or

-	The UE is associated with a serving cell on a carrier different than the PS carrier, and has a non-serving cell selected on the PS carrier with the preconfigured radio parameters.

When the advanced-V2X or PS UE is not associated with a serving cell on the V2X or PS carrier, and the UE does not have knowledge of its geographical area, or is provisioned with preconfigured radio parameters that are not associated with any Geographical Area, V2X or PS UE’ transmissions are not allowed, and the requirements in Section 6.3E.2 apply.

For a terminal that supports operation in shared spectrum, the current version of this specification assumes in the uplink sub-bands within a wideband channel shall be contiguously allocated to the UE.  The uplink requirements for one or more non-transmitted sub-bands between two transmitted sub-bands does not form a part of the current version of this specification.

Terminal that supports inter-band NR-DC configuration shall meet the minimum requirements for corresponding CA configuration (suffix A), unless otherwise specified.

A UE that is capable of supporting the aerial features as indicated by the [aerial capability] in [15] and that has an aerial subscription is considered an Aerial UE for the purposes of this specification and shall meet the minimum requirements defined for this feature (suffix J) if applicable for any of the UL configured bands.

[Editor’s Note: Clarify whether it is necessary for the UE to perform authentication and authorization of the UE subscription to meet the condition of  “having an aerial subscription”]

<Change #2>

## 6.2J	Transmitter power for Aerial UEs

### 6.2J.1	Reserved

## 6.2J.2	Reserved

### 6.2J.3	UE additional maximum output power reduction for NR-DC

#### 6.2J.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) as described in clause 6.2.3.1.

Table 6.2J3.1-1 specify the list of additional requirements applicable for aerial UEs (see clause 4.3), with their respective network signalling value and applicable bands. If  an aerial UE is configured in UL with one of the bands indicated in Table 6.2J.3.1-1, the UE shall fulfil the requirements indicaed in this table. In order to meet the additional requirements, additional power reduction (A-MPR) is allowed for the maximum transmit power as specified in Table 6.2J.3.1-1.

If an aerial UE is configured with multiple UL bands (for example, UEs configured with CA or NR-DC), and at least one of the configured UL bands is indicated in 6.2J.3.1-1, then the UE shall apply the requirements in Table 6.2J.3.1-1 for these bands and apply the requirements in clause 6.2.3 for configured bands not indicated in Table 6.2J.3.1-1.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_63 | [6.5J.2.3.1, 6.5J.3.3.1] | n3 |  |  | Clause 6.2J.3.2 |
| NS_64 | [6.5J.2.3.2, 6.5J.3.3.2] | n38 |  |  | Clause 6.2J.3.3 |

#### 6.2J.3.2	A-MPR for NS_63

#### [Editor’s Note: TBD]

#### 6.2J.3.3	A-MPR for NS_64

[Editor’s Note: TBD]

<Change #3>

## 6.5J	Output RF spectrum emissions for Aerial UEs

### 6.5J.1	Reserved

### 6.5J.2	Out of band emission

[Editor’s Note: TBD on th eadditional requirements]

### 6.5J.3	Spurious emissions

[Editor’s Note: TBD]

<End Of Changes>