3GPP TSG-RAN WG4 Meeting # 108-bis		R4-2316265

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.26.2

Source:	Samsung

Title:	Discussion on simulation assumptions and results of NTN co-existence study in above 10GHz bands

Document for:	Discussion

- Introduction

According to the Rel-18 NR NTN enhancement WID, RAN4 has been mandated to study and identify NTN example band in above 10 GHz.

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminals and scenarios exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

For co-existence studies, following considerations should be taken into account:

- Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow.

- For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands.

From the RAN4 #104-bis-e to 108 meeting, agreements on most assumptions have been achieved. And the remaining issues have been discussed in this paper.

- Discussions

Simulation parameters for co-existence study

Equivalent satellite antenna aperture

The highlighted parameter, Equivalent satellite antenna aperture, in Table 2.3.1-2 of [1], is still pending. These values are for 20GHz DL NTN satellite. However, as the previous agreement NTN DL frequency is 17GHz. Therefore this antenna aperture value would change as the DL frequency is changed, which will lead to different antenna pattern in the simulation and then change the interference between NTN SAN and TN network.

Proposal 1: Discuss and determine Equivalent satellite antenna aperture values for different satellite types.

Table 2.3.1-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  | 1200 km |  | 600 km |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz |  | 10 dBW/MHz |  | 4 dBW/MHz |  |
| Satellite max TX power in dBm | BW (MHz) |  | 200 | 400 | 200 | 400 | 200 | 400 |
|  | SCS kHz |  | 120 | 120 | 120 | 120 | 120 | 120 |
|  | NRB |  | 132 | 264 | 132 | 264 | 132 | 264 |
| Satellite Tx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 dBi |  |
| Equivalent satellite antenna aperture |  |  | [5 m] |  | [0.5 m] |  | [0.5 m] |  |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.1765 deg |  | 1.7647 deg |  | 1.7647 deg |  |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | Note 1 |  | Note 1 |  | Note 1 |  |
| Satellite beam diameter |  |  | 110 km |  | 40 km |  | 20 km |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |
| Satellite Rx max Gain |  |  | 58.5 dBi |  | 38.5 dBi |  | 38.5 dBi |  |
| Note 1: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) - with ABS [degree]=180/pi x ABS[rad] and - with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |  |  |  |  |  |  |

Nosie Figure for NTN SAN and UE

A final agreement on SAN Noise Figure was made after the approval of [1] by RAN4 #108 meeting, in which it is agreed that the Noise Figure of LEO is 3.5dB. Therefore, the corresponding value in [1] should updated accordingly

|  | SAN NF | UE NF |
| --- | --- | --- |
| For GEO | 3.5 dB | 2.5 dB |
| For LEO (option 1) | 3.5 dB | 2.5 dB |
| For LEO (option 2) | 5.9 3.5 dB | 6 dB |

Proposal 2: Update [2] with NTN SAN noise figure as 3.5dB. And the updated co-existence simulation assumptions are captured in Attachment 1.

Co-existend study results

The tentative results in [2] which has captured the latest outcome from companies indicate the calibration is relatively stable. Therefore, it is proposed to proceed the co-existence simulation of NTN in above 10GHz bands ASAP. And a template table to collect results of such simulation is proposed in Attachment 2. Latest results of coexistence study for NTN interfering TN cases from Samsung are captured as well.

Proposal 3: To use the table in Attachment 2 to collect results of the co-existence study of NTN in above 10GHz bands.

- Conclusion

Proposal 1: Discuss and determine Equivalent satellite antenna aperture values for different satellite types.

Proposal 2: Update [2] with NTN SAN noise figure as 3.5dB. And the updated co-existence simulation assumptions are captured in Attachment 1.

Proposal 3: To use the table in Attachment 2 to collect results of the co-existence study of NTN in above 10GHz bands.

- Reference

[1] R4-2313890 Simulation assumptions for NTN co-existence study in bands above 10GHz

[2] R4-2316250 Assumption and results of calibration for NTN co-existence in above 10GHz bands

Attachment 1 Simulation assumptions for NTN co-existence study in bands above 10GHz

Attachment 2 Co-existence data collection table