3GPP TSG-RAN WG4#108bis	R4-2316269

Xiamen, China, October 9th – 13th, 2023

Agenda item:	4.30.1.5

Source: 	Qualcomm France

Title: 	Considerations for UL CA_n26(2A)

Document for:	Approval

# Introduction

Analysis and proposals on UL CA_n26(2A) are provided in this paper.

# Considerations

Intra-band non-contiguous UL CA was approved as part of 700800900 WID [1]:

Table 1: Band combinations studied in this WI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n26(2A) | CA_n26(2A) | Telstra |
| Note 2: CA_n26(2A) is not intended for use in the USA. |  |  |

The channel bandwidths per operating band for each band combination are listed in the following table.

Table 2: Channel bandwidths per operating band for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26(2A) | CA_n26(2A) | n26 | 5, 10, 15 | 0 |

In RAN4#108bis the group should agree on the MSD test point. It is obvious that in FDD with narrow RX-TX separation the IMD products land on top of DL’s causing significant desense. We propose to use worst case test point, which is narrowest CC BW’s and frequencies where IMD3 and IMD5 hit on top of DL CC’s

We propose as follows. With these test points, the IMD3 peak is just at the middle of PCC, and IMD5 peak is just at the middle of SCC. Here we have assumed normal UL RB allocations for each UL CC.

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 25 (RBSTART = 0) | 25 (RBSTART = 0) | 861.5 | 884 | TBD | TBD | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |  |  |  |

We propose to specify UL CA_n26(2A) only using single PA as that would be the reality in implementations.

# Conclusion

Initial considerations on test point and PA configuration was provided for UL CA_n26(2A)

Proposal 1: Use the following test points:

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 25 (RBSTART = 0) | 25 (RBSTART = 0) | 861.5 | 884 | TBD | TBD | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Specify UL CA_n26(2) using one PA

# Reference

[1] RP-232680, Revised WID on enhancement for 700/800/900MHz band combinations