3GPP TSG-RAN WG4 Meeting # 108bis					R4-2316280

Xiamen, China, October 09 – October 13, 2023

Source:		Ericsson

Title:			TP on WUS guard RB

Agenda item:		5.20.2

Document for:		Approval

# Introduction

In this contribution, we present our TP on the WUS guard RB simulation results with observation and proposals.

# Discussion

In previous agreed TP for WUS guard band, the below text is agreed:

RAN4 suggests overall bandwidth of the wake-up signal in the NR channel i.e., desired signal along with all the required guard RBs shall fit in the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 of TS 38.101-1.

Figure 7.1.2.1-1: Definition of the guardband of NR channel and guard RB for LP-WUS

Figure 7.1.2.2-2: WUS at the edge of NR channel

In current TR 38.869 and RAN1 agreement below, there is no agreement on the guard RB placement yet, e.g (within or outside of BW X).

#### 7.2.1.2 Bandwidth and location

For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further

- whether BW of LP-WUS is configurable (implicitly or explicitly)

- size of guard band [FFS: within or outside of BW X], if any

- whether there is different X for Idle, Connected, Inactive modes

So above understanding in RAN4 is that there are guard RBs which can be outside the 5MHz WUS signal allocation while in RAN1 current agreement, whether the guard RB can be within or outside the BW of WUS signal is not decided. Because RAN4 does not finalize the simulation activities and it is reasonable to keep such discussion open in RAN4.

RAN4 made the assumption the guard RB can be outside the WUS BW but RAN1 has no agreement on this yet

Whether RAN4 define the guard RB outside the WUS BW needs more discussion.

Because of this, in the following discussion and also the simulation, we use the number of shifted RB instead of the guard RB so that the eMBB signal can be still placed outside the LP-WUS BW. It is illustrated in Figure 1. This is to  differentiate that case where eMBB signal must be blanked for the guard RBs when placed outside the LP-WUS BW. From the simulation, we observe that the ACS case may need more shifted RB to increase the distance between LP-WUS signal and adjacent carrier and therefore more attenuation can be achieved for a certain filter design.

Figure 1:RB offset definition for ACS case

## Simulation results

To study the impact on the SNR performance of different filter order and the number of the shifted RB, the LLS simulation is performed. The simulation parameters are listed in Table 1 below.

Table 1: LLS parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Channel | AWGN |
| SCS | 30 kHz |
| UE Rx antennas | 1 for WUR |
| WUS bandwidth | For OOK WUS: - 5 MHz (for 30 kHz SCS: 12 PRBs+ ~ 1 PRB guard band each side of WUS) |
| Receiver filter | Chebyshev filter of order 4, 6,8,10 |
| Non-WUS transmissions | ACS and ASCS case |
| Rx approach | Payload based: based on Manchester decoding. |
| RF impairments | - Phase noise profile in [2] Phase noise (dBc/Hz). Offset; NF 9 dB; NF 12 dB; NF 15 dB. @5MHz; -99.4; -99.4; -99.4. @10MHz; -108.1; -105; -102.1. @15MHz; -120; -117; -114. |

### Simulation chain

Figure 2: Simulation signal processing chain

Figure 1 is illustrated the signal processing chain including the transmitter, channel and receiver. The IF BPF is used after the mixer and envelop detector then down-convert the IF signal to baseband. The phase noise is added to signal before the BPF to study the phase noise impact. The adjacent carrier is added together with AWGN noise before input to the receiver.

### ASCS case

Figure 3: ASCS simulation configuration with guard RB within WUS signal

In [1], our simulation shows there is no need to have more than 1 RB for the guard RB within the WUS signal BW. We simulate another case for ASCS which simulation configuration is illustrated in Figure 3. In this case, the WUS is placed at the channel edge near adjacent carrier and the power spectrum density of adjacent carrier is set to the same with NR cell carrier. The BLER performance is illustrated in Figure 4 for OOK1 and guard RB = 0.5 and guard RB=1. It can be observed that there is slightly SNR degradation (0.1 dB) for different filter order when phase noise is enabled, so the phase noise impact for ASCS is not obvious. It can be observed in Figure 4 that the filters with higher order than 4 have similar performance and 0.2 dB SNR does not motive the have better filter than filter order 4. In Figure 5, the BLER performance is compared for guard RB = 0.5 and guard RB= 1 case for filter order = 4. When increasing the nGuardRB from ½ to 1 at each side of the WUS, the SNR is degraded by 0.3 dB for the 4th order BPF when phase noise is OFF and by 0.1 dB when phase noise is ON. To confirm the trend, additional case is added for guard RB number = 1.5, this is illustrated in Figure 6, the same trend is observed for guard number =1.5 which has  and this means increasing the number of guard RB within the WUS signal may have penalty on the SNR.

The phase noise has no obvious impact on the ASCS performance for filter order 4 to 10.

The guard RB within the WUS signal BW should be less than 1 RB at each side of the WUS signal. Increasing the number of guard RB within the WUS BW has penalty on the SNR.

Figure 4: BLER performance for ASCS for OOK1 and Guard RB = 0.5 and 1

Figure 5: BLER for OOK1 with phase noise ON/OFF for ASCS case and nGuardRB =1/2 and nGuard RB =1 (zoom in figure is placed at the right side of the general figure)

Figure 6:BLER for OOK1 with phase noise ON/OFF for ASCS case and Guard RB number =1/2, 1 and 1.5 for filter order 4

### ACS case

Figure 7: ACS simulation configuration with guard RB and shift RB

As discussed above, in our opinion, RAN4 still needs to decide whether the WUS guard RB should be placed outside the WUS BW if to follow the RAN1 discussion. Therefore, the simulation configuration is illustrated in Figure 4 using the shifted RB instead of guard RB. The # of shifted RB is set to different size for a certain filter order design.

The BLER performance for the ACS case is illustrated from Figure 8 to Figure 10. It can be observed that the phase noise has strong impact on the SNR performance, for example, in Figure 8, for filter order of 8, SNR is degraded by 3 dB for OOK 4bit and 5 dB for OOK 2 bit and 3 dB for OOK 1 bit. It can also be observed in Figure 9 that filter order of 4 may only work if it were placed at middle of the 20MHz carrier. For OOK 1bit the WUS BW needs to be shifted at least by 18 RB, however, the filter order of 4 may not work for OOK 2bit, because even shifting the WUS guards by maximum of 19 RB doesn’t help to reach the 1% BLER illustrated in Figure 9.  In Figure 10, the needed shifting number of RB is around 6 to 10 for OOK 1bit to keep the SNR comparable to the ASCS case for filter order of 6. The needed shifted number of RB is around 20 for OOK 2 bit in Figure 10 which means it will be best to place WUS in the middle of the 20MHz channel.

The phase noise has strong impact on the ACS case.  At least 3 dB SNR degradation is observed for filter order of 8 and 10.

The BLER WUS performance has dependency on the modulation order of the OOK symbols (1bit, 2bit or 4 bit), filter order and shifted number of RB to the adjacent carrier.

The needed shifted RB number is in the range of 6 to 10 for OOK 1 bit with filter order of 6, for OOK 2 bit of the same filter with order 6, it is preferred to place the WUS in the middle of the 20MHz channel.

Figure 8: BLER performance for different filter order w/wo phase noise at channel edge (nShiftRB=0)

Figure 9: BLER performance for BFP order =4 and different # of shift RB with ACI = 33 dB

Figure 10:BLER performance for BFP order =6 and different # of shift RB with ACI = 33 dB

### Summary

For ASCS case, phase noise has little impact on the needed guard RB.  For ACS case, the phase noise may degrade the SNR at least 3 dB for filter order of 8 and 10.

The number of guard RB within the WUS signal for ASCS case does not need to be greater than 1, increasing the number of the guard RB within the WUS signal may have penalty on the SNR.

For ACS case, there is a need to shift the WUS away from the adjacent carrier to get more filter attenuation for the adjacent carrier, the number of the shifted RB depends on the WUR filter order and the number of bits per OFDM symbol. For example, it is preferred placing the WUS signal in the middle of 20MHz channel for OOK-2 for WUR with a filter of order 6. For OOK1 and for WUR with a filter of order 6, the WUS signal should be placed with 6 to 10 RB away from the Adjacent carrier channel edge excluding the NR channel guard band.

The observations from ACS simulation impacts the ACS design. The ACS is to test the UE receiver selectivity and the requirement is specified with a special test condition, e.g ACI level, the UL and DL RB placement. From the simulation results, it can be observed that it is necessary to introduce additional offset from the edge of WUS signal to ACI for certain filter design.  As it is not clear whether to test OOK1, OOK2 or OOK4 and which filter order WUR should be implemented, the detailed design of ACS test case could be discussed in work item phase.

It is necessary to introduce additional RB offset between WUS signal to the ACI in the ACS test.

Observed from the simulation, we do not think it is necessary to introduce additional guard RB beyond the WUS BW. The reason is that for ASCS, more than 1 RB is not necessary. For ACS, blanking more RB adjacent to the WUS BW does not help to the leakage power from ACI obviously, shifting the WUS signal away from the ACI helps to attenuate ACI as leakage power from ACI much higher than leakage power from adjacent subcarrier.

Guard RB should be placed only within the WUS BW.

## TP proposal

#### 7.1.2.2	ACS evaluation

Company 4 (R4-2313476 and R4-2316280)

The simulation parameters are listed in Table 7.1.2.2-7 below.

Table 7.1.2.2-7: LLS parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Channel | AWGN |
| SCS | 30 kHz |
| UE Rx antennas | 1 for WUR |
| WUR sampling rate | 10 Msps |
| WUS bandwidth | For OOK WUS: 5 MHz (for 30 kHz SCS: 12 PRBs+ ~ 1 PRB guard band each side of WUS) |
| Number of ADC bits | 8 |
| Receiver filter | 4, 6 , 8 and 10th order of chebyshev filter, filter BW is the same as WUS BW filter? |
| Non-WUS transmissions | Adjacent channel interference from other NR transmissions is considered. |
| Max. false alarm due to noise or random symbols | 0.1% |
| Frequency offset (ppm) | For OOK WUS: 50 ppm maximum frequency error |
| Rx approach | Payload based: based on Manchester decoding. |
| RF impairments | Phase noise profile Phase noise (dBc/Hz). Offset; NF 9 dB; NF 12 dB; NF 15 dB. @5MHz; -99.4; -99.4; -99.4. @10MHz; -108.1; -105; -102.1. @15MHz; -120; -117; -114. |

For ASCS case, phase noise has little impact on the needed guard RB.  For ACS case, the phase noise may degrade the SNR at least 3 dB for filter order of 8 and 10.

The number of guard RB within the WUS signal for ASCS case does not need to be greater than 1 as illustrated in Figure 7.1.2.2.-12, increasing the number of the guard RB within the WUS signal may have penalty on the SNR in Figure 7.1.2.2.-13.

For ACS case, there is a need to shift the WUS away from the adjacent carrier to get more filter attenuation for the adjacent carrier, the number of the shifted RB depends on the WUR filter order and the number of bits per OFDM symbol. For example, it is preferred placing the WUS signal in the middle of 20MHz channel for OOK-2 for WUR with a filter of order 6. For OOK1 and for WUR with a filter of order 6, the WUS signal should be placed with 6 to 10 RB away from the Adjacent carrier channel edge excluding the NR channel guard band. This is illustrated in Figure 7.1.1.2.2-14.

The observations from ACS simulation impacts the ACS test case design. The ACS is to test the UE receiver selectivity and the requirement is specified with a special test condition, e.g ACI level, the UL and DL RB placement. From the simulation results, it can be observed that it is necessary to introduce additional offset from the edge of WUS signal to ACI for certain filter design.  As it is not clear whether to test OOK1, OOK2 or OOK4 and which filter order WUR should be implemented, the detailed design of ACS test case could be discussed in work item phase.

Observed from the simulation, there is no need to introduce additional guard RB beyond the WUS BW. For ACS, blanking more RB adjacent to the WUS BW does not help to attenuate the leakage power from ACI, shifting the WUS signal away from the ACI helps more to attenuate ACI because the leakage power from ACI much higher than leakage power from adjacent subcarrier.

Figure 7.1.2.2-12: BLER for OOK1 w/wo phase noise for ASCS case

Figure 7.1.2.2-13:BLER for OOK1 with phase noise ON/OFF for ASCS case and Guard RB number =1/2, 1 and 1.5 for filter order 4

Figure 7.1.2.2-14:BLER performance for BFP order =6 and different # of shift RB with ACI = 33 dB

#### 7.1.2.4	Phase noise

Editor’s note: reserved

At WUR receiver, reciprocal mixing occurs when a high-power level adjacent interferer signal that presents at antenna mixes with the oscillator noise sideband (phase noise).  The reciprocal mixing generates cochannel interference to the WUR receiver passband as illustrated in Figure 7.1.2.4-1.  One example for reciprocal mixing in ACS case is illustrated in Figure 7.1.2.4-2 based on one company simulation.

￼

Figure 7.1.2.4-1 Reciprocal mixing for ACI case

Figure 7.1.2.4-2 Reciprocal mixing for ACI case before the BPF

# Conclusions

In this contribution, we present our view on guard RB simulation for WUS signal with below observations and proposals:

Observation 1 RAN4 made the assumption the guard RB can be outside the WUS BW but RAN1 has no agreement on this yet

Proposal-1:Whether RAN4 define the guard RB outside the WUS BW needs more discussion.

Observation 2 The phase noise has no obvious impact on the ASCS performance for filter order 4 to 10.

Observation 3 The guard RB within the WUS signal BW should be less than 1 RB at each side of the WUS signal. Increasing the number of guard RB within the WUS BW has penalty on the SNR.

Observation 4 The phase noise has strong impact on the ACS case.  At least 3 dB SNR degradation is observed for filter order of 8 and 10.

Observation 5 The BLER WUS performance has dependency on the modulation order of the OOK symbols (1bit, 2bit or 4 bit), filter order and shifted number of RB to the adjacent carrier.

Observation 6 The needed shifted RB number is in the range of 6 to 10 for OOK 1 bit with filter order of 6, for OOK 2 bit of the same filter with order 6, it is preferred to place the WUS in the middle of the 20MHz channel.

Proposal-2: It is necessary to introduce additional RB offset between WUS signal to the ACI in the ACS test.

Proposal-3: Guard RB should be placed only within the WUS BW.

# References

R4-2313476  LS reply on low-power wake-up receiver architectures, Ericsson

R4-2309204 , Evaluation of Low power wake-up receiver architectures , Ericsson