3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316296

# Xiamen, China, October 09 – October 13, 2023

Source: 	vivo

Title: 	Discussion on SSB-less SCell operation for network energy saving

Agenda Item:	5.24.3.2

Document for:	Discussion

Introduction

In RAN4 #108, the discussion on the SSB-less is extensive, and the WF is agreed. [1]

On the other hand, the issue on scenario 2a is discussed in RAN #101. The conclusion is as follows.

RP-232175	Discussion on Rel-18 network energy saving	ZTE, Sanechips

Replaces

Proposal 1: RAN tasks RAN4/RAN2 to support Scenario 2a and specify the corresponding requirements.

RAN4 chair (Huawei): was discussed in RAN4 for a few meeting, CR was not agreed so no consensus

RAN2 chair (Mediatek): RAN2 received one LS from RAN4 but not concluded yet, so work is still open

Ericsson: agrees with rapporteur that scenario 2a is not in the scope of the WI so it should not be in REL-18

ZTE: scenario 2a is in the WI

Intel: impacts on RAN1/RAN2 were feared for scenario 2a; we have no RAN1 objective in WID

Nokia: scenario 2a is not fitting for energy saving feature

T-Mobile: we could live with all the scenarios as we see no benefit in this

RAN2 chair (Mediatek): received LS, discussed it, is currently not supported in the spec, we are waiting for RAN4

decision

RAN1 chair (Samsung): does not remember that RAN1 received an LS on this

RAN chair: strong need to do scenario 2a: 2 companies (ZTE, Intel), no need for scenario 2a: 7 companies

RAN chair: could we conclude that scenario 2a is not considered in REL-18?

ZTE: can we say "scenario 2a is not specified in REL-18"?

conclusion: scenario 2a is not specified in REL-18

The document was noted.

Therefore, in this paper, our view to Scenario 1 is provided.

Discussion on Scenario 1

<On feasible RTD condition for scenario 1>

In current RAN2 spec, i.e. TS 38.331, the following clarification for the configuration of FrequencyInfoDL is captured for intra-band SSB-less SCell.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The cell-defining SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the SpCell or an SCell if applicable as described in TS 38.213 [13], clause 4.1. This is only supported in case the SCell for which the UE obtains the timing reference is in the same frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting RedCap, if FrequencyInfoDL is included in the ReconfigurationWithSync, this field corresponds to the cell-defining SSB.

Moreover, in TS 38.213, the clarification of cell search behaviour of the UE is as follows.

TS 38.213 v17.2.0

- For a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the PCell, or on the PSCell, or on an SCell if applicable as described in [10, TS 38.133], of the cell group for the serving cell.

In TS 38.133, the activation procedure of the corresponding SCell is specified in 8.3.2. Even for the unknown case, UE needs not to perform cell search nor the synchronization to the SCell and the activation delay can be reduced to 3ms, i.e. only includes the MAC CE decoding time. However, the restriction is highlighted as follows.

TS 38.133 v17.8.0

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

For the intra-band contiguous CA case, the UE RF chain used for the SSB-less cell and the timing reference cell is the same. As long as the Rx timing of timing reference cell is obtained, there is no much room for UE in adjusting the Rx timing of the SSB-less cell. In short, UE is easier in re-using the Rx timing from timing reference cell. However, for inter-band CA, normally worst-case UE is not able to directly re-use the fine Rx timing from the timing reference cell, since the RF chains are different. However, as long as network can ensure a small enough RTD for the co-located scenario, i.e. a small TAE, a capable UE may re-use the Rx rough timing from the SSB-based sync procedure on the timing reference cell, and further perform TRS-based tracking on the SSB-less cell based on the re-used rough timing. In this case the SSB-less cell can be feasible.

Observation 1  For inter-band SSB-less SCell, no time domain rough sync based on SSB can be performed on the SCell, so UE can only use the Rx timing from the reference cell, which may be obtained by another UE Rx chain, as the initial rough timing of the SSB-less Cell.

Based on the rough timing, further TRS-based fine time tracking is needed. In this procedure, UE should be configured with periodic TRS so as to continuously track the timing of the SCell. With the availability of periodic TRS, UE may be able to compensate the residual timing error, which is resulted from using Rx timing of reference cell.

Observation 2  After the SSB-less SCell is activated, availability of periodic TRS should be guaranteed, at least for UE’s fine time/frequency tracking purpose. UE may also compensate the residual timing error based on periodic TRS of the SCell.

In last meeting, RAN4 agree to at least define requirements for RTD ≤ CP, while whether the CP is the CP of SCell or the CP of largest-SCS cell is FFS. In our view, when at least one serving cell has larger SCS than the SCS of the SCell, there is difference between these 2 options. In this case, we prefer to interpret the CP length as the one corresponds to the cell with largest SCS, because it may provide more RTD margin. The smaller RTD between serving cell and reference cell the UE may assume, then the better performance UE may have under SSB-less cell.

Proposal 1  For RTD feasible conditions, the CP length corresponds to largest SCS across CCs.

<On feasible power difference condition for scenario 1>

For intra-band SCell without SSB, the same set of RF chains could be used by the UE for the SCell and the reference serving cell with SSB. Hence, if gNB can ensure that the Tx power difference is not that large between the 2 cells, normally there will not be big Rx power difference observed at UE side. However, for the inter-band case, even if network can ensure collocated deployment and TAE << CP for the inter-band scenario, the wireless propagation for the inter-band SpCell and SCell, for example, can still be quite different. This could be resulted from the different UE antennas used for different band. For example, for n5+n78 band combination, UE antennas used for low band are normally different from the antennas used for 3.5GHz band. In realistic typical use cases, quite large difference in the Rx signal level can be observed for different band.

Observation 3  For inter-band scenario, for some certain band combinations, large difference in the Rx signal quality can be observed among different bands even if co-located gNB array can be assumed, due to the different UE RF chains used for different bands.

However, above observation does not always means that the SSB-less SCell is not feasible. However, considering some mechanism enhancements in RAN2, such issue can be addressed. For example, UE may adjust the AGC value and PA value based on the measurement on the SSB-less cell. Such measurement can be CSI-RS based. Note that in R17 fast SCell activation, the A-TRS is already used for AGC adjustment on an inter-band SCell instead of SSB, so as to reduce the SCell activation delay. Moreover, CSI-RS L3 measurement can also be configured, in our understanding, to the SSB-less SCell.

Proposal 2  Regarding the feasible condition of reception power between different bands, the SSB-less SCell operation for the inter-band scenario is feasible if one of following condition is met

- The difference of Tx power with the FR1 inter-band active serving cell is within 6dB, and the same UE RF chain is used for the SSB-less SCell

- The UE is allowed at least one CSI-RS based measurement for AGC adjustment on the SCell before or during the activation of the SCell. Such CSI-RS based measurement can be A-TRS, TRS, CSI-RS for mobility or CSI-RS for pathloss measurements.

In last meeting, companies proposed to discuss the frequency domain separation between 2 bands. This could be a potential approach to solve the issue. However, it seems not the best one. Whether and how UE uses the same RF chains for the corresponding band combination should be up-to UE capability. It is not preferred to restrict UE implementation based on band classification. Based on all above information, to extend the SSB-less operation from intra-band CA to the inter-band CA, the following additional clarification/configuration needs to be provided.

Proposal 3  For scenario 1, instead of specifying frequency domain separation condition between 2 bands, RAN4 discuss the UE capability implying the same Antenna Set and/or RF chain Set is used in the inter-band CA combination

<On L3 measurement of the SCell without SSB>

Normally L3 measurement is performed by the UE so as to provide L3 MR. Based on L3 MR, gNB may know the signal quality of the SCell before it is actually activated for a given UE. In our view, even for the SSB-less SCell, L3 measurement should be supported.

In case UE does not report the capability implying same RF chain is used in the inter-band CA band combination, CSI-RS based L3 measurement can be used to measure the signal quality. In current TS 38.331, the current signalling only support the scenario when the CSI-RS and the corresponding associated SSB are in the same band. The following clarification can be found in the MeasObjectNR.

TS 38.331 v17.2.0

freqBandIndicatorNR

The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

Therefore, with small clarification/revision in the RAN2 specs, we see the feasibility of considering CSI-RS based L3 measurement. If L3 measurement is configured on the SCell and UE has reported L3 MR for the SCell, the SCell can be considered as known.

Proposal 4  CSI-RS based L3 measurement should be supported for the SSB-less SCell. The known/unknown condition of the SCell can be defined based on the L3 MR of the CSI-RS based L3 measurement.

<On L1 measurement of the SCell without SSB>

The L1 measurement here, in our understanding, includes

- BFD/CBD measurement

- L1-RSRP/L1-SINR measurement and reporting

Since different bands are considered, we do not see the issue if UE performs CSI-RS based BFD on the 2 bands. Moreover, current CSI-RS based L1 measurement requirements are also applicable if the CSI-RSs are configured on SSB-less SCell.

Observation 4  If CSI-RS based BFD/CBD/L1-RSRP/L1-SINR measurement are configured on an SSB-less SCell, current RRM requirements are applicable without need of any enhancements.

If the SCell without SSB is a cell with both DL and UL, the UL timing and PL-RS related measurement are also needed. Since the co-location condition is already clarified in the WID, in our view there is no need to consider the case that SSB-less SCell is not in the same TAG as the reference serving cell.

Proposal 5  No RRM requirements are specified if the SSB-less SCell is not configured in the same TAG as the reference serving cell.

For the PL-RS measurement, cross-carrier indication is not supported in RAN1/2 specs. However, cross-carrier pathloss reference linking can be supported.

Observation 5  If CSI-RS based PL-RS measurement is configured on an SSB-less SCell, current RRM requirements are applicable without need of any enhancements.

<On activation procedure of the SCell without SSB>

For the known case, UE still need to perform fine time tracking on the SCell, based on either A-TRS or TRS that is QCL-C with an active serving cell. If the CSI-RS based L3 measurement is not configured on this SCell, the SCell is regarded as unknown and UE may need to adjust AGC for this SCell. In this case, the activation delay of unknown SCell can also be defined.

Moreover, for the inter-band case, the timing reference serving cell also needs to remain detectable during the SCell activation procedure. Since the CSI-RS based L3 measurement is not necessarily configured, the side condition of the SCell should be -2dB no matter it is known or unknown.

Proposal 6  The SSB-less SCell activation requirements are only applicable if the Ês/Iot ≥ -2dB and the SSB of the timing reference serving cell meeting detectable condition in 9.2 of TS 38.133

- If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is unknown

  - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms.

- Otherwise, the SSB-less SCell is unknown

  - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  For inter-band SSB-less SCell, no time domain rough sync based on SSB can be performed on the SCell, so UE can only use the Rx timing from the reference cell, which may be obtained by another UE Rx chain, as the initial rough timing of the SSB-less Cell.

Observation 2  After the SSB-less SCell is activated, availability of periodic TRS should be guaranteed, at least for UE’s fine time/frequency tracking purpose. UE may also compensate the residual timing error based on periodic TRS of the SCell.

Proposal 1  For RTD feasible conditions, the CP length corresponds to largest SCS across CCs.

Observation 3  For inter-band scenario, for some certain band combinations, large difference in the Rx signal quality can be observed among different bands even if co-located gNB array can be assumed, due to the different UE RF chains used for different bands.

Proposal 2  Regarding the feasible condition of reception power between different bands, the SSB-less SCell operation for the inter-band scenario is feasible if one of following condition is met

- The difference of Tx power with the FR1 inter-band active serving cell is within 6dB, and the same UE RF chain is used for the SSB-less SCell

- The UE is allowed at least one CSI-RS based measurement for AGC adjustment on the SCell before or during the activation of the SCell. Such CSI-RS based measurement can be A-TRS, TRS, CSI-RS for mobility or CSI-RS for pathloss measurements.

Proposal 3  For scenario 1, instead of specifying frequency domain separation condition between 2 bands, RAN4 discuss the UE capability implying the same Antenna Set and/or RF chain Set is used in the inter-band CA combination

Proposal 4  CSI-RS based L3 measurement should be supported for the SSB-less SCell. The known/unknown condition of the SCell can be defined based on the L3 MR of the CSI-RS based L3 measurement.

Observation 4  If CSI-RS based BFD/CBD/L1-RSRP/L1-SINR measurement are configured on an SSB-less SCell, current RRM requirements are applicable without need of any enhancements.

Proposal 5  No RRM requirements are specified if the SSB-less SCell is not configured in the same TAG as the reference serving cell.

Observation 5  If CSI-RS based PL-RS measurement is configured on an SSB-less SCell, current RRM requirements are applicable without need of any enhancements.

Proposal 6  The SSB-less SCell activation requirements are only applicable if the Ês/Iot ≥ -2dB and the SSB of the timing reference serving cell meeting detectable condition in 9.2 of TS 38.133

- If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is unknown

  - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms.

- Otherwise, the SSB-less SCell is unknown

  - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

- References

[1]  R4-2314381  WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.  RAN4 #108