# 3GPP TSG-RAN WG4 Meeting # 108bis	                            R4-2316327

# Xiamen, China, Oct. 9 – Oct. 13, 2023

# Source: 	ZTE Corporation

# Title: 	MSD for 3Tx band combination

# Agenda item:	5.5.3.2

# Document for:	Approval

# 1 Introduction

In RAN4 #108 meeting, the MSD for the 3Tx band combination are FFS in the WF[1] and needed to be checked in this meeting. Meanwhile, the WID was revised [2] to include more PC1.5 3Tx band combination.

In this contributions, we give some MSD analysis for some PC1.5 band combinations.

# 2	Discussion

- For the PC1.5 Tx power configuration for the IMD analysis, the agreements in the WF[1] are used, i.e.

Issue 5-1-1: PC1.5 Tx power configuration for IMD WF: 23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

## 2.1 PC1.5 CA_n41-n71 (2Tx-1Tx)

- It was observed that IMD4 caused by 2UL frequencies of 3fn71-fn41 falls into band n71 Rx. Some of the parameters used for the IMD4 calculations are listed in table 1.

- Table 1. Parameters are used for IMD4 analysis

| Component | Value |
| --- | --- |
| Ant. Switch(IP4 (dBm)) | 56 |
| Diplexer (IP4 (dBm)) | 55 |
| PA Forward(IP4 (dBm)) | 30 |
| PA reserved (IP4 (dBm)) | 33 |
| Pout @ n71 (1Tx) (dBm) | 23 |
| Pout @ n41 (2Tx) (dBm) | 27.8 |
| Antenna isolation (dB) | 10 |
| Diplexer isolation (dB) | 15 |
| PCB isolation (dB) | 65 |
| n71 duplexer isolation | 50 |
| n71 duplexer Tx rejection @n41 | 50 |
| n71 duplexer Rx rejection @n41 | 50 |
| n41 filter @n71 Tx | 45 |
| n41 filter @Bn71 Rx | 45 |
| LNA to antenna IL (3dB) | 3 |
| Correct Factor (dB) | 0 |
| Diversity Gain+SNR (dB) | 4 |

- The MSD are calculated in table 2.

- Table 2. IMD4 values for PC1.5 CA_n41-n71 (2Tx-1Tx)

|  | - Main Path | - Div. Path |
| --- | --- | --- |
| - Total IMD4 products in n71(dBm) | - -62.4 | - -72.5 |
| - REFSEN for n71 @5MHz (dBm) | - -97.2 |  |
| - MSD (dB) | - 30.8 | - 20.9 |
| - MSD after MRC (dB) | - 23.5 |  |

- Therefore, for PC1.5 CA_n41-n71 (2Tx-1Tx), we have the following proposal:

Proposal 1:     The 3Tx MSD of PC1.5 CA_n71A-n41A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 23.5 | FDD | IMD4 |

## 2.2 PC1.5 CA_n25-n77 (1Tx-2Tx)

- It was observed that IMD2/4/5 caused by 2UL frequencies of fn77-fn25 ,3*fn25-fn77 and 2*fn77-3*fn25 falls into band n25 Rx, respectively. By using the similar method as above, the MSD are calculated in table 2.

- Table 3. IMD2/4 values for PC1.5 CA_n41-n71 (2Tx-1Tx)

|  | - IMD2 |  | - IMD4 |  | - IMD5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Main Path | - Div. Path | - Main Path | - Div. Path | - Main Path | - Div. Path |
| - Total IMD4 products in n71(dBm) | - -48.7 | - -58.6 | - -61.7 | - -71.7 | - -81.1 | - -91.2 |
| - REFSEN for n71 @5MHz (dBm) | - -96.5 |  |  |  |  |  |
| - MSD (dB) | - 43.8 | - 33.9 | - 30.8 | - 20.9 | - 11.8 | - 3.9 |
| - MSD after MRC (dB) | - 36.5 |  | - 23.5 |  | - 6.2 |  |

- Therefore, for PC1.5 CA_n25-n77 (1Tx-2Tx), we have the following proposal:

Proposal 2:     The 3Tx MSD of PC1.5 CA_n71A-n41A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 36.5 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 23.5 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 6.2 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

## 2.3 PC1.5 CA_n5-n77 (1Tx-2Tx)

- By using the similar method as above, it is proposed:

Proposal 3:     The 3Tx MSD of PC1.5 CA_n5A-n77A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | n5 | 844 | 5 | 25 | 889 | 24.4 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |

# 3 Conclusion

In this contribution, we give some discussions on the MSD for the PC1.5 3Tx band combination. The conclusions are:

Proposal 1:     The 3Tx MSD of PC1.5 CA_n71A-n41A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 23.5 | FDD | IMD4 |

Proposal 2:     The 3Tx MSD of PC1.5 CA_n71A-n41A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 36.5 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 23.5 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 6.2 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

Proposal 3:     The 3Tx MSD of PC1.5 CA_n5A-n77A IMD4 can be defined as below:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | n5 | 844 | 5 | 25 | 889 | 24.4 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |

# 4	Reference

[1] R4-2314705, WF on 3Tx and 4Rx requirements, OPPO

[2] RP-231719, WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO