# 3GPP TSG-RAN WG4 Meeting # 108bis	                               R4-2316331

# Xiamen, China, Oct. 9 – Oct. 13, 2023

# Source: 	ZTE Corporation

# Title: 	Remaining issues for Tx switching

# Agenda item:	x.x.x.

# Document for:	Approval

# 1 Introduction

In last meeting, some remaining issues were discussed and included in the moderator summary [1], also two LS [2][3] were discussed but not agree. Meanwhile, the scenario clarifications on the SUL band combination were agreed in last RAN#100 meeting.

In this contribution, we give some discussions on the remaining issues of Tx switching across 3 or 4 bands for single TAG listed in the WF.

# 2	Discussion

The first remaining issue is for the relationship UplinkTxSwitchingAdditionalPeriodDualUL-r18 and band ordering approach capability, which is whether UE is allowed to indicate both capability simultaneously, and if allowed, further discuss which switching period length applies. In terms of the discussion, the common understanding among companies are these two capability cannot be reported simultaneously, and it was included in the LS [2] although it was not agreed in the end.

Observation 1. UplinkTxSwitchingAdditionalPeriodDualUL-r18 and band ordering approach capability cannot be reported simultaneously.

Another remaining issue is for the switching period for the fallback band combination when the same band pair belongs to two different super band combinations but have different switching periods.

In terms of the discussion, RAN4 confirm the issue. However, in our understanding, this issue does not only exist for Rel-18 switching, but also seems for Rel-16/17 switching. Based on the signalling design of UE capability reporting in RAN2, in order to reduce the signalling overhead, even for Rel-16/17 UL Tx switching, the UE can report a band combination that contains more than two bands, for each band combination, the UE can further indicate the supported band pair(s) and corresponding switching periods. In that case, the band combination of two bands is treated as “fallback” band combination, and the UE capability can be derived from the reported super band combination. So the problem may also appear in Rel-16 and Rel-17 UL Tx switching.

Regarding this issue, we think there are many solutions, for example, if different switching periods are reported for a given band pair in different super band combinations. Then the network and the UE can apply the maximum value of the reported switching period, e.g. in above example, if the UE is configured with A+B+C, then the switching period for A+B is max{35us, 210us} = 210us, and the switching period for A+C is max{140, 35us} = 140us.

If companies think above solution is not optimal, then another option is to let the UE to report separate BC A+B+C in UL Tx switching band combination list. And it is up to UE which set of switching period values is reported for band combination A+B+C. The UE is also allowed to report a set of different values. In our view, this solution is straightforward and is compatible with RAN2 signalling. There is no need for NW explicitly indicate which band combination of which the switching periods are applied..

Proposal 1. In case the UE has different switching periods for different super band combination, the UE can report a separate band combination A+B+C with corresponding switching periods in UE capability.

Actually, there was another remaining issue, i.e. issue 1-2-3 in the WF [4], which is the advanced optional UE ability  to allow one Tx chain to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1), which is shown below. We copy here as follow for convenience.

- Issue 1-2-3: Advanced optional UE ability Way forward after round 2 discussion: - Further discuss in the next meeting: - For the switching scenario of {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, discuss and decide whether or not to introduce the advanced optional UE ability to allow one Tx chain to start transmission earlier if the switching time for this Tx chain is shorter.

The baseline assumption, i.e. neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods, is still valid.

It is normal for different band pairs have different switch periods, on top of the baseline UE assumption, from flexibility perspective, we think it is feasible to introduce advanced optional UE capability to allow the Tx chain to be transmitted during the delta time duration, and it is assumed there are no ambiguity issues for the switching cases or the ambiguity issues are solved.

Proposal 2. For the switching scenario without ambiguity, it is feasible to introduce advanced optional UE capability (per band per band pair per BC) to allow one Tx chain to start transmission earlier if the switching time for this Tx chain is shorter.

Last, for the CR to TS38.101-1, there are two alternative CR [5, 6] but not approved in the end. Where the alternative 1 CR is same version with some updates for the IE name. Per the following agreements in RAN#101 meeting:

It was mentioned that the restrictions in the WF shall be clarified at lease in TS38.101-1, e.g., section 5.5C “Configurations for SUL”. Therefore, we think the alternative 2 CR R4-2314909 can be used as baseline for the restriction clarification in TS38.101-1, although alternative 1 CR R4-2314673 is the based on the CR which was already agreed in RAN4#107 meeting.

Proposal 3. The alternative 2 CR R4-2314909 can be used as baseline for the restriction clarification in TS38.101-1.

# 3 Conclusion

In this contribution, we give some further discussion on some remaining issues of Tx switching across 3 or 4 bands for single TAG. The proposals and conclusions are:

Observation 1. UplinkTxSwitchingAdditionalPeriodDualUL-r18 and band ordering approach capability cannot be reported simultaneously.

Proposal 1. In case the UE has different switching periods for different super band combination, the UE can report a separate band combination A+B+C with corresponding switching periods in UE capability.

Proposal 2. For the switching scenario without ambiguity, it is feasible to introduce advanced optional UE capability (per band per band pair per BC) to allow one Tx chain to start transmission earlier if the switching time for this Tx chain is shorter.

Proposal 3. The alternative 2 CR R4-2314909 can be used as baseline for the restriction clarification in TS38.101-1.

# 4	Reference

[1] R4-2314670, Ad hoc minutes for [108][142] NR_MC_enh_UERF, China Telecom (Moderator)

[2] R4-2314933, LS on Rel-18 Tx switching enhancement, Huawei

[3] R4-2312183, LS on Rel-18 UL Tx switching for parallel switching on four bands, Mediatek

[4] R4-2306621, WF on Rel-18 Tx switching across 3/4 bands

[5] R4-2314673, Alternative 1 CR for 38.101-1: Time mask for switching across three or four uplink bands, China Telecom

[6] R4-2314909, Alternative 2 CR for 38.101-1: Time mask for switching across three or four uplink bands, Qualcomm