3GPP TSG-RAN WG4 Meeting #108bis	R4-2316353

Xiamen, China, 9 – 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 have received an LS from RAN2 in [R1-2304331] with the following description wrt to monitoring of paging occasions for CG-SDT for a HD-FDD UE: “The UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s).” Based on this reply LS, it is clarified that dropping of CG-SDT is allowed only when paging occasiosns within a modification period overlap with CG-DST transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: It is clearified that the UE is allowed to drop the CG-SDT transmissions only when CG-SDT transmissions overlap with paging recpetions within the modification period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour when paging recpetions and CG-SDT occasions overlaps are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------Start of change 1 ----------------------------

#### 5.1B.2.6	Maximum interruption in paging reception

The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission within a SI modification period, then the UE shall monitor for paging in any of the paging occasions within the same SI modification period. In this case the UE is allowed to drop the CG-SDT transmission.

----------------------END OF CHANGE 1----------------------------