3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2316366

Xiamen, China, October 09 – 13, 2023

Title:	[Draft] LS on the UE SRS IL imbalance issue

Response to:	-

Release:	Rel-18

Work Item:	NR_ENDC_ RF_FR1_enh2

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	-

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1		Overall description

An issue that insertion loss (IL) for the diversity branch might be different from the main branch has been identified by RAN4, and the IL imbalances across branches are called SRS IL imbalance.

RAN4 has defined a relaxation for 4Rx as a non-ideal factor in PCMAX_L, f, c, so that the transmission power of any SRS resource from diversity branch would be allowed for a certain reduction. Additionally, such relaxation could be larger in order to deal with 8Rx RF architecture that would increase more implementation complexity, RAN4 will define related requirements for 8Rx in this release. The following definition is excerpted from TS 38.101-1.

Considering that such imperfection could lead to inaccurate channel estimation at receiver, which could lead to incorrect PMI selection that would degrade overall system performance, RAN4 think it is necessary to address such issue by enabling UE report on the actual IL imbalance (could be different from the aforementioned requirement from both value and granularity perspective) of each SRS resource other than the first one within the SRS resource set configured for antenna switching usage.

It is recognized by RAN4 that UE may change the mapping between physical antenna ports and SRS port (so the actual IL imbalance could change as well) due to some reasons e.g. human tissue blocking. For this situation, RAN2 can consider dynamic reporting in addition to static reporting.

For RAN2 information, the characteristics of this report are listed as below. Details on signaling design are up to RAN2 decision:

- The absolute value of reported actual IL imbalance of each SRS resource other than the first one within the SRS resource set configured for antenna switching usage shall not exceed the ΔTRxSRS requirements that defined in TS 38.101-1 for different AS-SRS capabilities accordingly.

- UE can indicate whether it supports static or dynamic reporting, then the network can configure the UE reporting accordingly.

  - For static reporting, it means that the UE will not change the indicated actual IL imbalance once it has reported that for configured SRS resource set with antenna switching usage.

  - For dynamic reporting, it means that the UE may trigger a reporting once it finds out the actual IL imbalance would be different from previous reporting. Existing RAN2 mechanism on UAI (UE Assistance Information) reporting as defined in TS 38.331 can be considered to comprehend the purpose/motivation.

- This report is considered from Release 18 onwards.

- 2		Actions

To RAN2

ACTION: 	RAN4 respectfully ask RAN2 to consider above information for their future work on enabling UE report on actual SRS IL imbalance.

- 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #109	     November 13 – 17, 2023		         Chicago, United States

TSG RAN WG4 Meeting #110	     February 26 – March 1, 2024	         Athens, Greek