3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2316369

Xiamen, China, October 09 – 13, 2023

Title:	[Draft] LS reply on further clarifications on enhancements to realize increasing UE power high limit for CA and DC

Response to:	R4-2315008 (R1-2308561)

Release:	Rel-18

Work Item:	NR_cov_enh2

Source:	TSG RAN WG4

To:	TSG RAN WG1, TSG RAN WG2

Cc:	-

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1		Overall description

RAN4 thanks RAN1 for the LS on further clarifications on enhancements to realize increasing UE power high limit for CA and DC in R1-2308561.

Regarding the questions from RAN1, RAN4 would like to share the following answers based on RAN4 latest discussion outcomes in R4-2314703.

- Q1: It is RAN1 understanding that ΔPPowerClass can be triggered by the cases when the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain duty cycle as specified in Clause 6.2.4 of TS 38 101-1. Could RAN4 clarify whether all these cases can trigger ΔPPowerClass reporting in PHR MAC CE?

- Answer from RAN4: RAN4 confirms that all cases when the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain duty cycle as specified in Clause 6.2.4 of TS 38 101-1 can trigger ΔPPowerClass reporting.

- Q2: In case of duty cycle exceedance, and resulting ΔPPowerClass reporting as per recommendation in R4-2310500, is a further ΔPPowerClass reporting also allowed when UE returns to advertised PC power capabilities?

- Answer from RAN4: As clarified in R4-2314703, ΔPPowerClass reporting is also allowed when UE returns to advertised power class reference after duty cycle exceedance.

- Q3: Could RAN4 confirm the correctness of RAN1’s understanding as per observation b) concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when duty cycle is exceeded?

- Answer from RAN4: RAN4 confirms that observation b) from RAN1 is correct, which means “occasion of the report” refers to the event that triggers the aperiodic PHR report, and not to the actual UL resource to send the MAC-CE carrying the report.

- Q4: Could RAN4 clarify the meaning of the recommendation related to the combination of the ΔPPowerClass report with full-power MIMO transmission capability reporting corresponding to the current power class?

- Answer from RAN4: The full-power MIMO transmission capability is essentially related to e.g. PA configuration and antenna ports arrangement. Further, it can be different when MOP reduction happens due to the change on ΔPPowerClass. Although the network can be aware of MOP reduction by ΔPPowerClass reporting, existing full-power MIMO transmission capability indication seems not to explicitly satisfy the purpose on allowing the network to acquire that capability in case it would be different from the one indicated for the advertised PC (but no MOP reduction happens). That is the meaning of “combination” from RAN4 perspective and any further solution to solve this issue would be up to RAN2 decision.

- 2		Actions

To RAN1

ACTION: 	RAN4 respectfully ask RAN1 to consider above information.

To RAN2

ACTION: 	RAN4 respectfully ask RAN2 to consider above information for their future work on the implementation of ΔPPowerClass reporting.

- 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #109	     November 13 – 17, 2023		         Chicago, United States

TSG RAN WG4 Meeting #110	     February 26 – March 1, 2024	         Athens, Greek