3GPP TSG-RAN4 Meeting # 108bis	R4-2316379

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	MPR for FR2 UE UL 256QAM

Agenda Item:	5.6.2

Document for:	Approval

- Introduction

The support of FR2 UL 256QAM was discussed in previous meetings, and latest WF was approved in [1].

In this contribution, we provide some discussions on the open issue for MPR.

- Discussion

2.1 MPR requirements

Issue 1-1-1: MRP requirements

- Proposals

  - Option 1: The MPR for UL 256 QAM shall not exceed that of 64QAM by more than 3 dB.

  - Option 2: The MPR of UL 256 QAM shall not exceed 5 dB more than 64QAM.

  - Option 3: Average the MPR simulation results from different companies, for example 29GHz PC1 100MHz/200MHz:

Issue 1-1-2: PA model

- Proposals

  - Option 1: Consider the low AM-PM distortion PA to obtain the reasonable MPR value for FR2-1 256QAM.

  - Option 2: PA model in TR 38.803, i.e., Rapp Model, PA model for ~28 GHz, CMOS

  - Option 3: Others

Agreement:

- PA models in Option 1 and Option 2 are allowed.

  - Other PA model is not precluded for the requirements.

Issue 1-1-3: IQ image and carrier leakage

- Proposals

  - Option 1: Whether -25dB/-20dB IQ image and -25dBc/-20dBc carrier leakage in current Spec are too higher for UL 256QAM.

Agreement:

- -25dB/-20dB IQ image and -25dBc/-20dBc carrier leakage should be improved for UE supporting UL 256QAM.

  - There is no change of the requirements for other modulation orders

  - It shall be better than -36dB IQ image and -36dBc carrier leakage for UL 256QAM.

Per agreements made on PA mode and IQ image at last meeting. We use the following EVM budget for our further MPR simulation.

Table 2-1 EVM budget in MPR simulation for 256QAM

| EVM Contributor | EVM(%) | SNR(dB) |
| --- | --- | --- |
| Transmitter | 1 | -40 |
| Phase Noise | 1.8 | -34.9 |
| IQ Imbalance | 1.45 | -36.8 |
| PA Non-linearity | 2.5 | -32 |
| Total | 3.5 | -29.1 |

The simulation results is shown in Table 2-2.

Table 2-2: MPR simulation for 256QAM

| Modulation |  | Companies | MPRWT (dB), BWchannel ≤ 100 MHz/200MHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 64 QAM | - | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
|  | 256QAM | Huawei | 9.1 |  |  |
| CP-OFDM | 64 QAM | - | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
|  | 256QAM | Huawei | 11.8 |  |  |

- Conclusion

In this contribution, we provide MPR simulation on FR2 UL 256QAM.

Proposal 1: MPR for FR2 UL 256QAM:

| Modulation |  | Companies | MPRWT (dB), BWchannel ≤ 100 MHz/200MHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | 64 QAM | - | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
|  | 256QAM | Huawei | 9.1 |  |  |
| CP-OFDM | 64 QAM | - | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
|  | 256QAM | Huawei | 11.8 |  |  |

- Reference

[1] R4-2314677, “WF on UL 256QAM requirements”, Xiaomi