3GPP TSG-RAN4 Meeting # 108bis	R4-2316391

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	System parameters

Agenda Item:	4.34.2

Document for:	Approval

- Introduction

During last RAN#101 meeting, new rel-18 WI on introduction of NR bands n31 and n72 was approved in [1]. The main objective is to standardize two new NR FDD bands n31 and n72 in the 450MHz range.

  - band n31 (UL 452.5 - 457.5 MHz, DL 462.5 – 467.5 MHz)

  - band n72 (UL 451 – 456 MHz, DL 461 – 466 MHz)

In this paper we provide system parameters for the two new bands.

- Discussion

For the two new bands, the text proposal for system parameters are proposed in the following tables

•	Operating band

Table 2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n31 | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | FDD |
| n72 | 451 MHz – 456 MHz | 461 MHz – 466 MHz | FDD |

•	channel bandwidth per operating band

Table 2-2: BS channel bandwidths and SCS per operating band in FR1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | … | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

•	Channel raster

Table 2-3: Applicable NR-ARFCN per operating band in FR1

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n13 | 100 | 90500 – <20> –91500 | 92500 – <20> –93500 |
| n72 | 100 | 90200– <20> –91200 | 92200– <20> –93200 |

•	Synchronization raster

At last meeting, the sync raster for 3MHz channel bandwdith is introduced for band n100, n26, n28 and n85 in Rel-18 under WI in [2]. The same approach shall be reused for band n13 and n72.

Table 2-4: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR Operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n13 | 15 kHz | Case A | 1161 – <1> – 1162 |
| n72 | 15 kHz | Case A | 1157 – <1> – 1159 |

Table 2-5: Applicable SS raster entries per operating band for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n13 | 15 kHz | Case A | 28955 – <1> –28967 |
| n72 | 15 kHz | Case A | 28947 – <1> –28959 |

Proposal: It is proposed to approve the system parameters in Table 2-1 ~ 2-5 for the two new bands.

- Conclusion

In this paper we provide system parameters for the two new bands.

Proposal: It is proposed to approve the system parameters in Table 2-1 ~ 2-5 for the two new bands.

- Reference

[1] RP-232691, “New WID on introduction of NR bands n31 and n72”, Nokia

[2]	RP-231425, “NR support for dedicated spectrum less than 5MHz for FR1”