3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316401

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the TDCP measurement accuracy requirements

Agenda item:	5.29.2.1

Document for:	Discussion

Introduction

In RAN4#108, the following agreements were made regarding the scope of the MIMO_evo_DL_UL (TDCP) work in RAN4 R4-2314347 [1]

Issue 1-1-1: Whether to define TDCP measurement delay requirements? Agreement: - Do not define TDCP measurement delay requirements. Issue 1-1-2: Whether to define TDCP measurement accuracy requirements? Agreement: - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models. - Define TDCP accuracy requirements subject to conclusions of feasibility analysis. Issue 1-1-5: Besides RAN1’s definition, for TDCP calculation, whether to define additional information for averaging across RX ports? Agreement: - For TDCP measurement averaging across RX ports should be left to UE implementation. - The reported TDCP amplitude is expected to between the minimum and maximum measured values across the Rx branches if receiver diversity is used.

Discussion

# TDCP Overview

Time Domain Channel Property (TDCP) indicator focuses on providing the BS radio physical interface with new CSI information regarding the relative mobility of the UE and associated Doppler characteristics of the link with the serving BS. In the specification, Time domain channel property (TDCP) is defined as wideband normalized correlation between two CSI-RS transmission occasions, corresponding to CSI-RS resourece(s) from NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-info, that are separated by Dn symbols or slots, depending on the configuration, where Dn is the n-th delay configured value among [placeHolderForRrcParameter-D] configured delay values {D1, …, DY} and Y is number of configured delay values. This indicator is useful for specific reconfiguration uses cases, like CSI codebook type switching (Type-II to Type-I switching based on the channel conditions or channel variability with time information; if the channel varies faster than a certain threshold then a Type-II to Type-I switch occurs) and reference signal reconfiguration (e.g., SRS).

Let hn(k) be the Tracking Reference Signal (TRS) channel measured at subcarrier n, with n=0,…,N-1, and time kTs, where Ts is the time interval between two consecutive TRS measurement occasions. Let us assume a UE takes B repetitions of such TRS measurements at time k0Ts,k0+1Ts,…,k0+B-1Ts. The normalized wideband time-correlation function [1] at lag lTs, averaged over the N subcarriers, is given by A(l), given by equations (1) and (2). Figure 1 shows an illustration of the time correlation calculation for TDCP across multiple TRS. It is worth noting that this method provides reduced variability in TDCP amplitudes with the same number of TRSs and requires uniform spreading of delays, i.e., Dk=k∙D1 (see Figure 2).

| cl=1Nn=0N-11B-lk=k0k0+B-1-lhnk+lhn*(k), l=0,…,B-1 | (1) |
| --- | --- |

The wideband time-correlation function  c(l) is then normalized by the time-correlation function with lag 0, c(0), as:

| Al=c(l)c(0) | (2) |
| --- | --- |

Figure 1. Illustration of the Time Correlation Calculation for TDCP.

In RAN1#112 meeting, alternative B (i.e., Time domain correlation profile) was down selected as the TDCP value out of a set of other alternatives. B. Time-domain correlation profile; Non-zero quantized version of amplitude At,τ for a number of delay values τ (quantized amplitude vs delay) Example equation At,τ=ct,τct,0 where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n.; - Normalized auto-correlation of a time series measured from a TRS resource. - Multiple auto-correlation values can be calculated from different lags of the same resource or different resources - The autocorrelation can be estimated by replacing the channel hn for subcarrier n in the defining formula in column 2, with the matched filter subcarrier components Xn=Rn∙Sn* of the received signal Rn where Sn* is the complex conjugate of the known transmitted TRS signal. For ct,0 one can use the arithmetic average over the two TRS symbols separated by the time τ , i.e. At,τ≈n=0N-1Xnt+τ∙Xn*t12∙n=0N-1Xnt∙Xn*t+Xnt+τ∙Xn*t+τ Or, alternatively, one may use the geometric average for ct,0, i.e. At,τ≈n=0N-1Xnt+τ∙Xn*tnXnt+τ2nXnt2 Further methods to remove noise bias and to suppress noise can be used..

TDCP is calculated as the normalized time correlation function from 2 or more channel measurements estimated based on TRS.

The normalized TDCP coefficient for delay (l) from multiple TRS transmissions may be calculated as

  - Al=clc0, where cl=1Nn=0N-11B-lk=k0k0+B-1-lhnk+lhn*k,    l=0,…,B-1

  - This method provides reduced variability in TDCP amplitudes with the same number of TRSs and requires uniform spreading of delays.

| a) | b) |
| --- | --- |
| a) | b) |

Figure 2. Polar scatter plots comparing two methods of delay correlation calculation for two TDCP configurations.

Choice of a method of TDCP calculation can influence the reported TDCP measurements.

From the simulation results in [2], the TDCP reports concentrate at |A(l)|=1 and angle(A(l)) = 0 for low UE speeds, while the amplitude |A(l)| decreases as the UE speed increases. As the speed increases, it is also possible to observe that the spread of the amplitude of |A(l)| also increases in the polar scatterplots of the correlation values for TDCP. Which is also in line with the decision from RAN1 to assign less quantization levels to smaller amplitude of |A(l)|. In this situation, also the phase of A(l) is used for helping on the detection of mobility, In the CDFs at the appendix of [8] it is possible to observe a large spread of phase of A(l) for larger speeds.

Issue 1-1-2: Whether to define TDCP measurement accuracy requirements? Agreement: - Further identify feasibility of methodology and test setup to define TDCP accuracy requirements including at least ideal TDCP definition, channel models - Define TDCP accuracy requirements subject to conclusions of feasibility analysis

To establish the accuracy requirements, the measurements must be compared to a baseline (ideal). The wireless channel experiences time-varying multipath fading due to the movement of the UE. The expected (ideal) value is a function of the UE speed, the details of the channel model, and the noise level. From the results above, the noise level is shown to affect the amplitude. If the channel is a frequency flat, there are many arrivals at the UE, and the distribution of the angles is uniform (i.e., there is a rich scattering environment near the UE), then the theoretical time correlation is the Bessel function which yields the classic Doppler spectrum.  If this were the case, the Bessel function values could be used as the ideal correlation values but would need to be degraded according to the SNR.

RRM accuracy requirements for TDCP can be useful for guaranteeing the performance of the reports in the field, just like other reporting like L1-RSRP, L1-SINR. However, the mapping to a testable environment of a theoretical TDCP value is still not clear. Therefore, it is needed to define a methodology which can be used for determining the ideal value of the TDCP measurement, including also the formula used for calculating the TDCP value.

So far there is no clear methodology for determining the ideal TDCP value.

RAN4 to study how to determine the ideal value for TDCP for the accuracy requirements.

RAN4 to define TDCP accuracy requirements assuming TDCP coefficient A(l) for lag l calculated as Al=c(l)c(0), where cl=1Nn=0N-11B-lk=k0k0+B-1-lhnk+lhn*(k),    l=0,…,B-1

RAN1 has defined for TDCP, a nonuniform quantization of the wideband normalized amplitude that has the alphabet given by; {1-2-16-q/2}  where q={0,1,2,⋯,15} and a uniform quantization of the phase that is given by alphabet; {ej2πq/16} where q={0,1,2,⋯,15}. As mentioned in session 1.4, the motivation for the non-uniform quantization is that mapping from speed to TDCP amplitude is nonlinear, therefore more bits are assigned for representing the large amplitude of A(l). Considering that motivation, it is reasonable to assume that accuracy would need to be better for values of |A(l)| which are larger and represented with more quantization levels.

RAN1 has defined 4-bit non-uniform quantization of TDCP amplitude coefficients, because lower speeds are mapped into high amplitude values and 4-bit uniform quantization of phase coefficients.

RAN4 to define accuracy requirements that depend on amplitude for TDCP.

As part of the next steps for the discussion of TDCP, RAN4 will most likely need to bring simulation results related to the TDCP accuracy. TDCP main motivation in RAN1 is to determine the best codebook type depending on the estimation of UE mobility. Therefore, as part of this work, we need to define the simulation methodology, including how speed can be roughly emulated. One approach that has been used for the definition of performance requirements is to use the maximum Doppler frequency. That in combination with a channel model can be used for defining the TDCP accuracy.

As TDCP is a metric related to mobility detection, it is important to model speed as part of the methodology for determining the accuracy requirements.

RAN4 to study how Maximum Doppler frequency of TDL models can be used for modeling speed for TDCP accuracy requirements.

In addition to channel model parameters, simulation parameters need to include the UE speed, TRS parameters, as well as TDCP parameters.

RAN4 to discuss simulation parameters, including UE speed, TRS parameters, TDCP parameters, and channel model.

Feasibility Study

# AWGN channel:

The accuracy requirements for ideal TDCP measurement can be defined for UE in presence of AWGN (generator). Here by ideal TDCP measurement we assume no mobility and channel variations due to slow or fast fading. Ideal TDCP amplitude and phase of the wideband normalized correlation between two CSI-RS transmission occasions with any delay value would be equal to 1 and 0 accordingly assuming virtually no noise (Now we will refer to them as just amplitude and phase). In presence of noise, distribution of the amplitude will have different expected value and variance as we showed in [2]. TDCP measurement accuracy can be tested in these conditions assuming ideal amplitude to be equal to 255/256 (q=0) i.e., the highest possible value to be reported. If optional phase reporting to be also tested, ideal phase should be equal to 0 (q=0). Assuming AWGN channel, there is no difference between possible delay values to choose for testing.

AWGN based test doesn’t require multiple delay configurations to be tested.

RAN4 to define AWGN based test for configuration Y = 1, D1 = 1 slot configuration, for TDCP amplitude accuracy test.

Phase reporting is supported only for Y > 1 TDCP configuration.

Depending on whether correlation phase reporting also a subject to accuracy requirements, the change of distribution of the phase in presence of noise should be studied.

RAN4 to define AWGN based test for configuration Y = 2, D1 = 1 slot, D2 = 2 slots configuration, for TDCP amplitude and phase accuracy test, subject to decision to test TDCP phase accuracy.

# AWGN extension with Jakes Model:

From the previous simulations in [2], it was shown that the measured correlation falls as the SINR goes down (i.e., the noise increases).  To determine the expected (ideal) value for the correlation we could:

- Calculate the expected correlation in a noiseless environment.  Assuming a Jakes spectrum, the correlation function is a Bessel function. The UE speed is needed to calculate the appropriate Bessel function result.

- Assuming the noise variance is known, calculate the correlation value when noise is present.  This requires a theoretical connection between the Bessel function case without noise and the case with noise.

RAN4 to further discuss possibility of defining Jakes model, TDL and CDL based TDCP accuracy tests.

RAN4 to further study about the noise level based on existing measurements and signaling (e.g., RSRQ, CQI) – SNR which can be used to calculate the expected correlation including the effects of noise.

AWGN test setup

Based on the simulation results in the Appendix, TDCP amplitude and accuracy test can be formulated for AWGN in the following format:

Assuming SNR levels γi [TBD] TDCP amplitude {ideal TDCP Amplitude (q=0)} shall be reported at least α% of the time and a TDCP amplitudes from the set {ideal Amplitude TDCP-j (q=i)} shall be reported not more than αi,j% of the time, where j=1,..N. [N and αi,j% are TBD].

TDCP phase reports can be defined in the same way:

Assuming SNR levels γi [TBD] TDCP phase {ideal TDCP Phase (q=0)} shall be reported at least α% of the time and a TDCP phases from the set {ideal Phase TDCP-j (q=i)} shall be reported not more than αi,j% of the time, where j=1,..N. [N and αi,j% are TBD].

The simplest test setup for TDCP can be established assuming AWGN channel model with a maximal percent of reported quantized amplitudes different from ideal value.

AWGN test is minimal required test and does not capture TDCP ability to capture channel variability in time due to mobility.

RAN4 to define at least TDCP accuracy test in AWGN channel conditions for a set of SNR values.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: TDCP is calculated as the normalized time correlation function from 2 or more channel measurements estimated based on TRS.

Observation 2: The normalized TDCP coefficient for delay (l) from multiple TRS transmissions may be calculated as

  - Al=clc0, where cl=1Nn=0N-11B-lk=k0k0+B-1-lhnk+lhn*k,    l=0,…,B-1

  - This method provides reduced variability in TDCP amplitudes with the same number of TRSs and requires uniform spreading of delays.

Observation 3: Choice of a method of TDCP calculation can influence the reported TDCP measurements.

Observation 4: So far there is no clear methodology for determining the ideal TDCP value.

Proposal 1: RAN4 to study how to determine the ideal value for TDCP for the accuracy requirements.

Proposal 2: RAN4 to define TDCP accuracy requirements assuming TDCP coefficient A(l)  for lag (l)  calculated as

- Al=clc0, where cl=1Nn=0N-11B-lk=k0k0+B-1-lhnk+lhn*k,    l=0,…,B-1

Observation 5: RAN1 has defined 4-bit non-uniform quantization of TDCP amplitude coefficients, because lower speeds are mapped into high amplitude values and 4-bit uniform quantization of phase coefficients.

Proposal 3: RAN4 to define accuracy requirements that depend on amplitude for TDCP.

Observation 6: As TDCP is a metric related to mobility detection, it is important to model speed as part of the methodology for determining the accuracy requirements.

Proposal 4: RAN4 to study how Maximum Doppler frequency of TDL models can be used for modeling speed for TDCP accuracy requirements.

Proposal 5: RAN4 to discuss simulation parameters, including UE speed, TRS parameters, TDCP parameters, and channel model.

Observation 7: AWGN based test doesn’t require multiple delay configurations to be tested.

Proposal 6: RAN4 to define AWGN based test for configuration Y = 1, D1 = 1 slot configuration, for TDCP amplitude accuracy test.

Observation 8: Phase reporting is supported only for Y > 1 TDCP configuration.

Observation 9: Depending on whether correlation phase reporting also a subject to accuracy requirements, the change of distribution of the phase in presence of noise should be studied.

Proposal 7: RAN4 to define AWGN based test for configuration Y = 2, D1 = 1 slot, D2 = 2 slots configuration, for TDCP amplitude and phase accuracy test, subject to decision to test TDCP phase accuracy.

Proposal 8: RAN4 to further discuss possibility of defining Jakes model, TDL and CDL based TDCP accuracy tests.

Observation 11: AWGN test is minimal required test and does not capture TDCP ability to capture channel variability in time due to mobility.

Proposal 10: RAN4 to define at least TDCP accuracy test in AWGN channel conditions for a set of SNR values.

- References

- R4-2314347, “WF on R18 NR MIMO RRM requirements”, Samsung, RAN4#108, Toulouse, France, August 21 – 25, 2023.

- R4-2313563: Nokia/Nokia Shanghai Bell, 3GPP TSG RAN WG4 Meeting #108, Agenda item 8.29.3.1 Discussion on the Time Domain Channel Property (TDCP). Aug 2023.

Appendix: AWGN based TDCP tests