3GPP TSG RAN WG4 Meeting #108 bis	R4-2316405

Xiamen, China, Octorber 9th  –  Octorber 13th , 2023

Agenda Item:	5.24.2.3

Source: 	Ericsson

Title:	Improvement on Scell/SCG setup delay

Document for:	Discussion

# Introduction

From RAN4 #104 meeting Rel-18 improvement on Scell/SCG setup delay are listed in  [1] . In this paper we provide our view on certain open issues.

Background: WID objective #7

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including:​

- Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and​

- Definition of corresponding RRM requirements [RAN4]; and​

- If necessary, based on RAN4 outcome, definition of corresponding signaling support [RAN2].​

Note 5: RAN4 will coordinate in due course with RAN2 to start the work.​

Note 6: R4-2220415 serves as baseline for future work in RAN4​

Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.​

# Discussion

## Sub-topic 2-1 scope and overall solution

Issue 2-1-3: relationship between R16 EMR and R18 enhancement to SCell/SCG setup delay - Candidate solutions: - Option 1: R18 fast FR2 CA/DC setup is independent to EMR feature. (QC, CMCC, ZTE, HW, Nokia) - Option 1a: in R18 enhancement to SCell/SCG setup delay, solutions based on existing idle/inactive measurement for cell reselection purpose and enhanced measurement are independent of UE support of R16 EMR. (Apple) - Option 2: For UE to utilize the cell reselection measurement as enhancement for Rel-18, to support Rel-16 Idle/Inactie measurement for CA/DC is a pre-requisite. (E///) Issue 2-2-1: measurement report. - Agreement: - For UE which is not capable of EMR, when the measurement results during idle/inactive mode are available and valid, UE can report these results to the network during or after RRC setup or resume procedures. The reporting procedure is up to RAN2.

Based on current solution agreements and proposal for Issue 2-1-3, our interpretation is that separate UE capability will be introduced for Rel-18 both for existing measurement based solution and enhanced measurement based solution if the enhanced measurement is feasible and agreed.

The main enhancement using the existing measurement proposed for Rel-18 is to to validate the existing cell re-selection measurement. As to our knowledge for UE do not support Rel_16 Idle/Inactive measurement, the validation of the existing measurements can only been performed by using the existing cell-reselection measurements.

Agreement has been reached during RAN4 108 meeting which clarify for the UE which is not capable of Rel-16 Idle/Inactive CA/DC measurement , the report procedure is up to RAN2. There is possibility to report Cell-reselection however, the criteria is to report valid measurement especially the cell re-selection inter-frequency measurement.

Cell-reselection inter-frequency measurements were set for the purpose of coverage purpose for the camping cell, however, the Rel-16 Idle/Inacitve measurement is for setting up capacity offloading frequencys in CA/DC. These typically require different measurement qualification and frequency characteristics. There could be no overlapping in configurations of the frequencies.

Also cell-reselection measurements, UE only being specified how to derive the beam specific measurement, is not specified to be stored or reported as it is for the purpose of UE centric mobility according to TS 38.304 clause5.2.4.2. We are questioning the feasibility to use the cell-reselection measurements to validate Idle/Inactive measurement especially for the UE that have an implementation not to store Cell-reselection measurement.

Proposal 1: RAN4 shall clarify the Rel-18 capability for how to verify the cell-reselection measurement, especially focus on the RRM requirement aspective as defined from WID.

## Sub-topic 2-2 solutions based on existing measurement

Issue 2-2-2: definition of ‘valid’ in solution based on existing measurement - Agreements in RAN4#106-bis-e: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition. - Candidate solutions: - Option 1: criterion A. (Apple, MTK, CATT, Xiaomi, vivo, HW) - Option 2: criteria A+B (CATT, ZTE, E///) - Option 3: criteria A+C (LGE) - Option 4: criteria B+C (Nokia)

As this open issue has been around several meeting’s discussion, we would like to clarify our understanding of the definition of validity.

This validity is a mutual understanding between Network and UE which implies when UE report some measurement results to the NW based on the configuration accordingly, NW can utilize these measurement to setup CA/DC purpose earlier than after RRC_Resume/Setup. This is due to the higher layer signalling delay typically will cause the system gain of CA/DC being reduced or even removed.

Currently there are 3 options for the validity definition,

    - A) the measurement are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

The option A can guarantee the UE will report measurement results that is fresh enough which solves the issue of Rel-16 validity issue due to the T331 timer and RRC_Idle/Inactive duration have no relation. Too long time ago measurement can not ensure reflection of the current radio enviourment which is very crucial for setting up radio resources for UE.

The option B is a existing criteria which reflects on the performance part for Rel-16. Due to the Idle/Inactive UE measurement and Connected mode UE measurement has different measurement accuracy, to guarantee the measurement results validity can useful to network this is very important. Also remove this criteria for Rel-18 will causing very unfair situation for UE has implement Rel-16 EMR capability due to lossen requirement.

The option C is an indirect validation method to sort out the measurement results that are invalid, it can be useful for UE to verify for example Cell-reselecton measurement as we understand for this typs of measurements different UE might have different implementation, which causing standardize a validation criteria very hard.

Our view on the validation definition option A, option B and option C is that they are not mutual exclusive, however combine them can limiting the final measurement results to send to NW.

From UE point of view, send less measurement can save certain resources however from NW point of view less measurements being sent can be less sampling point in the real time radio environment.

Even though combine all of the 3 validation criteria can causing less measurement results send to network , from effective of the RRM requirement point of view, we still think this is a reasonable combination. As this will limiting the measurements being send however will guarantee the newtwork the validity from time, measurement accuracy and measurement variation perspective.

Proposal 2: The validity definition shall combine all 3 options. A+B+C

    - A) the measurement are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

# Summary and Conclusion

In this contribution we have provided our views on improvement on Scell/SCG setup delay. The following proposals are made:

Proposal 1: RAN4 shall clarify the Rel-18 capability for how to verify the cell-reselection measurement, especially focus on the RRM requirement aspective as defined from WID.

Proposal 2: The validity definition shall combine all 3 options. A+B+C

    - A) the measurement are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

# References

- R4-2314332 “WF for NR mobility enhancements RRM requirements”, Apple