3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316426

Xiamen, China, 9th October – 13th October 2023

Source:	Ericsson, BT plc

Title:	TP for 38.718-03-01 to include CA_n3-n7-n20 and DC_n3-n7-n20

Agenda item:			4.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.718-03-01 to include CA_n3A-n7A-n20A and, DC_n3A-n7A-n20A.

This TP depends on the fallbacks specified in “R4-2316423, TP for 38.718-02-01 adding CA_n7-n20” submitted at the same meeting.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n3-n7-n20

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n7-n20 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n7+n20

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A-n20A | CA_n3A_n7A CA_n3A_n20A CA_n7A_n20A | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n7-n20, the TIB,c and RIB,c values are reused from CA_n3-n7-n26 and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n7-n20 | 0.5 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n7-n20 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Co-existence studies shows that IMD2 caused by UL CA_n3-n7 Tx might fall into band n20 Rx.

Co-existence studies shows that IMD2 and IMD3 caused by UL CA_n3-n20 Tx might fall into band n7 Rx.

Co-existence studies shows that no IMD caused by UL CA_n7-n20 fall into band n3 Rx.

#### 5.x.2.2	REFSENS requirements

IMD2 Band n20 Rx. MSD value reused from CA_n3-n7-n28.

IMD2 Band n7 Rx. MSD value is reused from CA_n3-n7-n26.

IMD3 Band n7 Rx. MSD value reused from CA_n3-n7-n28.

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n20 | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | 2505 | 10 | 50 | 2625 | 29.0 | FDD | IMD2 |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n7 | 2545 | 5 | 25 | 2665 | 17.0 | FDD | IMD3 |
|  | n20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |

---Text omitted---

### 6.x.1	Configurations for DC_n3-n7-n20

Table 6.9.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n7A-n20A | DC_n3A_n7A DC_n3A_n20A DC_n7A_n20A |

---End of changes---

# Reference

[1] 		R4-2312594, Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2) , ZTE Corporation

[2]		R4-2312591	TR38.718-03-01, v0.7.0 on Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2) , ZTE Corporation