3GPP TSG-RAN WG4 Meeting #108bis	R4-2316452

Xiamen, China, 9 – 13 Oct, 2023

Source:	ZTE Corporation

Title:	Discussion on RRM impacts of Option A

Agenda Item:	5.10.2

Document for:	Approval

- Introduction

In recent RAN plenary 101 meeting, a revised WID was approved in [1] for Rel-18 to revise the objectives of completion of specification support for BWP without restriction in NR. The main update is to add the handover aspects under Option C into the consideration. Therefore, until now, L1 measurements, RRM measurement and mobility for CONNECTED all should be considered to support the BWP without restriction operation. The revised scope for core part is summarized as below:

The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based onSSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. 3) Specify the necessary requirements to support the additional handover cases same as for RedCap (RAN4) - For Option C - Specify handover requirements for the additional handover cases based on existing RedCap handover requirements. Deprioritize the scenarios except for the scenario of handover from NCD-SSB in the active BWP of source cell to NCD-SSB in the active BWP of target cell. Note: Handover requirements for RedCap UE with 2 Rx antennas are reused as much as possible. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

According to the discussion in RAN, RAN4 and RAN1, the following potential solutions were identified in [2]:

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps (MG) - Option B-2-2) Dedicated MG or Network Controlled Small Gap (NCSG) for RLM/BFD/BM measurements - Option C) Non Cell Defining-SSB (NCD-SSB) approach which would work with existing UE hardware architectures (FG 6-1 BWP Operation With Restriction) and be compatible with existing RAN4 specifications for BM/RLM/BFD

During the discussion in RAN4#108 meeting, RAN4 achieved the following agreements in [3]:

- General aspects: Issue 1-2: Requirements for handover for Option A, C, B-1-1 and B-1-2 - It is RAN4 understanding that it is beneficial defining requirements for handover to a BWP with NCD-SSB directly for UE supporting option C. - Whether to define the HO requirements is up to RAN plenary discussion. Issue 1-3: Requirements/UE behaviour for UE supporting both option B-1-1 and C - It is RAN4 understanding that for UE supporting both option B-1-1 and C, if NCD-SSB is configured within the active BWP, then UE is expected to perform RLM/BFD/BM and L3 intra-frequency measurements based on NCD-SSB based on TS 38.331. No additional requirements will be defined for such scenario. Issue 1-6: Requirements/UE behaviour for UE supporting both option B-1-1 and B-1-2 - UE shall not indicate support of both B-1-1 and B-1-2 for the same agreed granularity reporting. Whether and how to capture this in 38.133 is FFS depending on RAN1 conclusion. - Impact of Option B-1-1: Issue 3-1: Applicable conditions of existing RLM/BFD/BM requirements for Option B-1-1 - For a UE capable of supporting Option B-1-1, the UE is required to meet BM/RLM/BFD requirements without interruptions if CD-SSB is within the channel bandwidth of the UE. - The applicable conditions are to be updated in requirements in clauses 8.1.1, 8.1.2, 8.5.1, 8.5.2, 8.5.5, 9.5.1, 9.8.1. - Note: additional conditions can be considered subject to RAN1 agreements Issue 3-4: Clarification/modification on requirements for intra-frequency measurements without gaps requirements for supporting option B-1-1 - From RAN4 perspective same UE feature group can be used for support of Option B-1-1 RLM/BFD/BM and L3 intra-frequency measurements - For UE capable of supporting Option B-1-1, the requirements in section 9.2.5 for Intra-frequency measurements without measurement gaps are applicable if CD-SSB is within the channel bandwidth of the UE. - Note: additional conditions can be considered subject to RAN1 agreements - FFS: How would UE report NeedForGap/NeedForGapNCSG with ‘no-gap’ for intra-frequency measurement when supporting of option B-1-1 is indicated already? - Impact of Option C： Issue 4-1: Applicable conditions of existing RLM/BFD/BM requirements for Option C - Clarify in RLM/BFD/BM measurement requirements in clauses 8.1, 8.5, 9.5 and 9.8 that the SSB and SMTC applies for CD-SSB and NCD-SSB if the UE is capable of supporting Option C. Detailed wording is up to CR stage discussion. Issue 4-3: Clarification on existing timing requirements for supporting Option C - In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB includes CD-SSB and NCD-SSB for UE capable of option C”. Issue 4-4: Spec impact on intra-frequency measurement requirements for UE supporting option C - Spec impact on intra-frequency measurement requirements for UE supporting option C - The requirements for L3 intra-frequency measurements are based on the existing requirements in 9.1. - The SSB and SMTC in section 9.1.1 applies for both CD-SSB and NCD-SSB for the UE capable of option C, i.e., use the same text as for RedCap. - The reference SSB definition for non-RedCap should be captured in clause 9.2.1 with the same definition as for RedCap. - Note: alignment with RAN2 terminology is needed - From RAN4 perspective same UE feature group can be used for support of NCD-SSB based RLM/BFD/BM and L3 intra-frequency measurements Issue 4-5: Requirements for transition between CD-SSB and NCD-SSB for supporting option C - The intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap can be re-used for UE supporting option C by defining requirements in clause 9.1.6. - Impact of Option B-1-2： Issue 5-5: RLM/BFD/BM requirements for Option B-1-2 - For a UE capable of supporting Option B-1-2, the UE is allowed to cause interruptions to meet CD-SSB based BM/RLM/BFD requirements if CD-SSB is within the channel bandwidth of the UE but outside of active BWP. - The interruption requirements are FFS - If the existing BM/RLM/BFD requirements can be reused is depending on how interruption ratio is defined. - The requirements are defined based on the assumption that there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured. - The applicable conditions are to be updated in requirements in clauses 8.1.1, 8.1.2, 8.5.1, 8.5.2, 8.5.5, 9.5.1, 9.8.1. - Note: additional conditions can be considered subject to RAN1 agreements

In this document, we provide some of our views on the impact of all options.

- Discussion

2.1 Discussion on Option A

Regarding the RRM impact of supporting Option A, based on the latest revised WI, no need to consider L3 aspects, so we can focus on L1 measurement requirements and timing requirements.

The following agreements had been approved around L1 measurement requirements:

<Agreement >: - For the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), existing BM/RLM/BFD requirements defined in TS 38.133 is complete and no new or additional requirements are needed. <Agreement >: - In general, no specification impact on RLM/BFD/BM requirements for UE supporting option A for BWP operation without restriction. - FFS if there is impact on FR2 CSI-RS base RLM requirements specifically. <Agreement >: - Do not introduce further enhancements for CSI-RS based RLM/BFD/BM requirements for Option A

Based on the progress achieved until now, the remaining FFS on FR2 CSI-RS based RLM due to the potential overlapping between the CSI-RS occasion and gap. For this point, it specified in [4] as below:

For FR2, - P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion. - P=11−TCSI−RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

We can not see any necessity to introduce additional clarification on the CSI-RS based RLM. The CSI-RS used for RLM is within the active BWP, same as in legacy. And similar logic applies to BFD/CBD.

Proposal 1: No need to give additional clarification on FR2 CSI-RS based RLM for Option A.

With respect to the timing requirement, it is specified in [3] as below:

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

So the condition of applying the requirement of Te is specified.

Even though still no progress achieved on the timing requirement of Option A. For Redcap UE, the following agreement was approved in previous meeting:

Agreement:(Redcap UE) The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

To our understanding, the following candidates were proposed to address the timing issue:

- Candidate 1: Make use of MG to acquire DL timing based on the SSB outside active BWP

- Candidate 2: Acquire DL timing based on CSI-RS(TRS)

- Candidate 3: Borrow DL timing of other BWP/Cell

All above candidates can address the timing issue. Especially for Candidate 1, it has been allowed for Redcap UE, extending it to non-Redcap UE is natural. So from our point of view, no need to concern the timing issue for applying Te in Option A. No RAN4 work is needed.

Proposal 2: No additional RAN4 work is needed for Option A.

Regarding whether need any clarification on existing timing requirements when CD-SSB is outside active BWP, two options were proposed in last meeting:

- Option 1: - No clarification on existing timing requirements when CD-SSB is outside active BWP is needed - Option 2: - In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB is within the channel BW of the UE”.

In our view, Option 2 is more clear even though existing spec has no constraint.

Proposal 3: In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB is within the channel BW of the UE”for Option A.

2.2 Discussion on Option B-1-1

The timing issue was discussed in last meeting. The following options were proposed in [3]:

- Option 1: - Follow conclusion of issue 2-2 (agreements in the last meeting). - Option 2: - No clarification is needed for the UE to meet the existing UE transmit timing requirements (section 7.1 of TS 38.133) when performing BM/RLM/BFD according to option B-1-1

In current UE transmit timing requirement in section 7.1 of [4], there is no condition that the SSB should be within the active DL BWP. This means the UE transmit timing requirement applies regardless of whether the SSB is within or outside the active DL BWP. Therefore, applying same logic, here we can not see the necessity to add additional clarification on existing timing requirements used for the UE supporting Option B-1-1.

Proposal 4: No clarification on existing timing requirements for Option B-1-1.

2.3 Discussion on Option B-1-2

The timing issue was discussed in recent meetings. The following options were discussed in previous meeting:

- Option 1: - The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support. - Option 2: - It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

In fact companies achieved consensus that the UE capable of Option C shall meet the transmit timing error (Te) requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last X ms. Where X=160 ms for UL SCS ≤ 120 kHz or X=80 ms for UL SCS = 480 kHz or X=40 ms for UL SCS = 960 kHz. Right now, the conflict lies in whether some clarification is needed or not. In our view, the description for RepCap UE can be set as reference as below.

- 7.1A.2 Requirements The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

Therefore CD-SSB or NCD-SSB can be explicitly mentioned in the timing requirements for non-Redcap UE.

Proposal 5: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can be CD-SSB or NCD-SSB if UE supports FG 53-3 for Option B-1-2.

Regarding the requirements of interruption caused by performing the RLM/BFD/BM based on the SSB outside active BWP but within the BW of the UE, it has been decided to introduce interruption length and interruption ratio requirement for Option B-1-2. During last meeting, the discussion of interruption length has been divided into two parts: RF tuning time and interruption length requirement. The following options were proposed in [3]:

Interruption length (RF tuning time) for option B-1-2 - Option 1: - Interruption length is 0.5ms for FR1 and 0.25ms for FR2. - Option 2: - Interruption length is 1ms for FR1 and 0.75ms for FR2. - Option 3: - Interruption length is max (slot length of the cell, 0.5ms) for FR1 and max(slot length of the cell, 0.25ms) for FR2. Interruption length requirements for option B-1-2 - Option 1: Interruption length in number of slots (based on interruption length of 0.5ms for FR1 and 0.25ms for FR2) - Option 2: Interruption length in number of slots (based on interruption length of 1ms for FR1 and 0.75ms for FR2)

In our view, such interruption would happen right before and after the SSB based RLM/BFD/BM measurement, so the location of interruption in fact can be decided as shown in Fig.1.

Regarding the exact interruption length, we believe the length of RF retuning is fine, i.e. 0.5ms for FR1 and 0.25 ms for FR2.

Fig.1 Option B-1-2

Proposal 6: The interruption length can be 0.5 ms for FR1 and 0.25 ms for FR2 for Option B-1-2.

With respect to the interruption ratio, the following options were proposed in [3]:

- Option 1: - For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. - Option 1a: - In addition to the maximum length of each interruption, the maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined: - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %. - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %. - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %. - Option 2: - For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP. - Option 2a: - The probability of missed ACK/NACK for a UE supporting Option B-1-2, due to interruptions caused by UE performing BM/RLM/BFD measurements based on SSB outside the active BWP, shall not exceed [1.0]%. - Option 2b: - In addition to the maximum length of each interruption, the maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined: - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 1 %. - Option 3: - X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

Option 1, 2 series are similar, all starting from the ACK/NACK missing ratio. While Option 3 lies in the actual ratio derived from interruption length and measurement periodicity. To our understanding, the actual interruption ratio really derives from the interruption length and measurement periodicity. Therefore we prefer Option 1.

Proposal 7: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%].

2.4 Discussion on Option C

The latest revised WID has explicitly included the L3 intra-frequency measurements without gaps for Option B-1-1 and Option C.

It has been approved that supporting L3 measurements based on NCD-SSB for Option 3 is beneficial, the only obstacle of the NCD-SSB based L3 measurement is the WID scope in previous discussion. However until now, this issue has been included in the revised WID explicitly.

To our understanding, for the UE capable of Option C, no matter CD-SSB or NCD-SSB, provided the SSB is within the active BWP, UE can perform such L1 measurement without any additional interruption. Besides L1 measurement, for intra-frequency RRM measurement, from the perspective of UE behavior, it is no doubt that such UE is able to perform intra-frequency RRM measurement without gap. Which is beneficial to aviod unnecessary interruption cased by gap. Furthermore, such enhancement has already been supported for Redcap UE.

Regarding the spec impact, to our understand, the condition of performing intra-frequency SSB based measurement without gap can be extended as below:

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the CD-SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3] or - the NCD-SSB is completely contained in the active BWP of the UE if the UE supports FG 53-3.

Proposal 8: For the UE supporting Option C, the UE is allowed to perform intra-frequency SSB based measurement without MG through the CD-SSB or NCD-SSB within the active BWP.

In last RAN4 meeting, it is recognized that it is beneficial to define requirements for handover to a BWP with NCD-SSB directly for the UE capable of Option C. In recent RAN plenary 101 meeting, such extension of scope was approved, so RAN4 would determine the requirements of such handover for UE capable of Option C.

It should be noted that the case of handover from NCD-SSB in the active BWP of source cell to NCD-SSB in the active BWP of target cell is deprioritized, and it is suggested to reuse the handover requirements for Redcap UE with 2 Rx antenna as much as possible. In R17 Redcap, two kinds of handover including intra-frequency and inter-frequency handovers are classified, and different principle of Tsearch is assumed for the two types. Such classification can be reused for Option C.

Proposal 9: Define requirements of Tsearch for intra-frequency and inter-frequency handover respectively for Option C.

The intra-frequency and inter-frequency can be identified according to whether the same frequency and SCS are applied between the active SSB of current cell and the target SSB of the target cell. Furthermore, the case of both active SSB of current cell and the target SSB of the target cell are NCD-SSB is deprioritized.

Proposal 10: Intra-frequency handover is defined as the center frequency and SCS of the active SSB of current cell is same as the center frequency and SCS of the target SSB of the target cell.

Inter-frequency handover is defined as the center frequency and SCS of the active SSB of current cell is different from the center frequency and SCS of the target SSB of the target cell.

Furthermore, regarding the known case, Tsearch is deemed as 0 ms for the intra-frequency handover. While for the inter-frequency handover, depending on whether the target SSB of the target cell is measured or not, Tsearch is 0 ms or larger than 0 ms. There are the following multiple candidates for the case of the target SSB of the target cell is not measured:

- The measured SSB is a CD-SSB and the target SSB for HO is an NCD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is a CD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is another NCD-SSB;

- In details, Tsearch is N*Trs ms for all the above candidates.

- For the unknown case, including the unknown intra-frequency and unknown inter-frequency, reuse the legacy requirements for Tsearch is fine, i.e. Tsearch is N*Trs ms for unknown intra-frequency cell and Tsearch is 3N*Trs ms for unknown inter-frequency cell.

Proposal 11: For known intra-frequency target cell, Tsearch is deemed as 0 ms.

Proposal 12: For known inter-frequency target cell, three cases are included and Tsearch is N*Trs ms.

- The measured SSB is a CD-SSB and the target SSB for HO is an NCD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is a CD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is another NCD-SSB;

Proposal 13: For unknown case, Tsearch is N*Trs ms for unknown intra-frequency cell and Tsearch is 3N*Trs ms for unknown inter-frequency cell.

- Conclusion

In this contribution, we have the following proposals for the RRM impacts:

Proposal 1: No need to give additional clarification on FR2 CSI-RS based RLM for Option A.

Proposal 2: No additional RAN4 work is needed for Option A.

Proposal 3: In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB is within the channel BW of the UE”for Option A.

Proposal 4: No clarification on existing timing requirements for Option B-1-1.

Proposal 5: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can be CD-SSB or NCD-SSB if UE supports FG 53-3 for Option B-1-2.

Proposal 6: The interruption length can be 0.5 ms for FR1 and 0.25 ms for FR2 for Option B-1-2.

Proposal 7: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%].

Proposal 8: For the UE supporting Option C, the UE is allowed to perform intra-frequency SSB based measurement without MG through the CD-SSB or NCD-SSB within the active BWP.

Proposal 9: Define requirements of Tsearch for intra-frequency and inter-frequency handover respectively for Option C.

Proposal 10: Intra-frequency handover is defined as the center frequency and SCS of the active SSB of current cell is same as the center frequency and SCS of the target SSB of the target cell.

Inter-frequency handover is defined as the center frequency and SCS of the active SSB of current cell is different from the center frequency and SCS of the target SSB of the target cell.

Proposal 11: For known intra-frequency target cell, Tsearch is deemed as 0 ms.

Proposal 12: For known inter-frequency target cell, three cases are included and Tsearch is N*Trs ms.

- The measured SSB is a CD-SSB and the target SSB for HO is an NCD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is a CD-SSB;

- The measured SSB is an NCD-SSB and the target SSB for HO is another NCD-SSB;

Proposal 13: For unknown case, Tsearch is N*Trs ms for unknown intra-frequency cell and Tsearch is 3N*Trs ms for unknown inter-frequency cell.

- Reference

- RP-232677 “Further revised WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo.

- RP-230766 “Summary of offline discussion about BWP without restriction”, Vodafone.

- R4-2314339 “WF on support for bandwidth part operation without restriction”, vivo.

- TS 38.133-i30.