3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316465

Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.3.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#108 meeting, a WF related to 8Rx UE PDSCH requirements was agreed [1]. In this contribution, we provide simulation results and discuss outstanding issues.

- Discussion

2.1	PDSCH Requirements for single carrier

It was agreed to use the following MCS to define PDSCH single carrier requirements in TDD in RAN4#108.

Issue 4-1-1: MCS for Rank 2 test MCS 19 (Table 1) Issue 4-1-2: MCS for Rank 4 test MCS 17 (Table 1) Issue 4-1-3: MCS for Rank 8 test MCS 17 (Table 1)

Also, for FDD, it was agreed to use same parameters including MCS values, MIMO layers, antenna configuration, and propagation conditions as TDD requirements.

RAN4 defines PDSCH FDD requirements with same parameters including MCS values, MIMO layers, antenna configuration, and propagation conditions as TDD requirements.

Based on the above agreements and assumptions from Annex A.1 in [1], we provide our simulation results in Table 1.

Table 1. SNR point of 70% maximum throughput for rank 2, 4, 8

| Rank | MCS | Propagation Condition | Channel Correlation | Alignment results TDD SNR (dB) | Impairment results TDD SNR (dB) | Alignment results FDD SNR (dB) | Impairment results FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 12.5 | 15.0 | 12.6 | 15.1 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 12.5 | 15.0 | 12.7 | 15.2 |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 21.2 | 23.7 | 21.4 | 23.9 |

2.1	PDSCH Requirements for CA

Issue 4-3-1: CBW for PDSCH CA requirements - Option 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e. - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz - Option 2: Single bandwidth for each CC: 15kHz/10MHz for FDD and 30kHz/40MHz for TDD separately

There are no example band combinations introduced in RF session. Following the same methodology applied for 2Rx and 4Rx in Rel-15, we support Option 1 with test applicability that UE only needs to pass the requirements with supported band combination.

Proposal 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

- FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

- TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Issue 4-3-2: Duplex model - Option 1: Consider the following duplex model for 8Rx CA - FDD 15kHz + TDD 30kHz - FDD 15kHz + FDD 15kHz - TDD 30kHz + TDD 30kHz - Other options not precluded

For duplex mode, we support to preclude FDD 15kHz + TDD 30kHz and consider FDD 15kHz + TDD 30kHz and TDD 30kHz + TDD 30kHz only.

Proposal 2: Consider only duplex mode FDD 15kHz + TDD 30kHz and TDD 30kHz + TDD 30kHz to define requirements for 8Rx CA PDSCH.

Issue 4-3-3: Test scenario One single test scenario to be defined, with same test configurations as single carrier case except the bandwidth: - Option 1: Rank8 case - Option 2: Rank2 case

Regarding the rank for CA PDSCH requirements, we prefer to use Rank8, MCS17 (Table 1) and TDLA30-10 Low propagation conditions, which is the same as Rank8 configuration in single carrier requirements.

Proposal 3: For test scenario of CA PDSCH requirements, we prefer to use Rank8, MCS17 (Table 1) and TDLA30-10 low propagation conditions, which is the same as Rank8 configuration in single carrier requirements.

Based on the proposals above, we provide our simulation results for TDD and FDD as below.

Table 2. SNR point of 70% maximum throughput for each CBW in TDD

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 21.62 |
|  |  | 10 |  |  |  |  | 21.16 |
|  |  | 15 |  |  |  |  | 21.26 |
|  |  | 20 |  |  |  |  | 21.04 |
|  |  | 25 |  |  |  |  | 21.27 |
|  |  | 30 |  |  |  |  | 21.43 |
|  |  | 40 |  |  |  |  | 21.22 |
|  |  | 50 |  |  |  |  | 21.24 |
|  |  | 60 |  |  |  |  | 21.48 |
|  |  | 80 |  |  |  |  | 21.61 |
|  |  | 90 |  |  |  |  | 21.73 |
|  |  | 100 |  |  |  |  | 21.99 |

Table 3. SNR point of 70% maximum throughput for each CBW in FDD

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | 8 | 17 (Table 1) | TDLA30-10 | 8x8 low | 21.39 |
|  |  | 10 |  |  |  |  | 21.38 |
|  |  | 15 |  |  |  |  | 21.45 |
|  |  | 20 |  |  |  |  | 21.39 |
|  |  | 25 |  |  |  |  | 21.56 |
|  |  | 30 |  |  |  |  | 21.43 |
|  |  | 35 |  |  |  |  | 21.62 |
|  |  | 40 |  |  |  |  | 21.82 |
|  |  | 45 |  |  |  |  | 21.79 |
|  |  | 50 |  |  |  |  | 21.65 |

- Conclusion

In this contribution, we provide our views about CA PDSCH requirements for 8Rx and also simulation results for single carrier and CA.

Proposal 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

- FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

- TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Proposal 2: Consider only duplex mode FDD 15kHz + TDD 30kHz and TDD 30kHz + TDD 30kHz to define requirements for 8Rx CA PDSCH.

Proposal 3: For test scenario of CA PDSCH requirements, we prefer to use Rank8, MCS17 (Table 1) and TDLA30-10 low propagation conditions, which is the same as Rank8 configuration in single carrier requirements.

- Reference

- R4-2313877, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon