3GPP TSG-RAN WG4 Meeting # 108bis	 R4-2316466

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-4: Reference measurement channels for 8Rx PDSCH requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PDSCH requirements for 8Rx UEs were agreed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce reference measurement channels for 8Rx PDSCH requirements - Reuse Table A.3.2.2.2-3 R.PDSCH.2-3.2 TDD for Rank 2, MCS19 - R.PDSCH.2-3.6 TDD for Rank 4, MCS17 - R.PDSCH.2-3.7 TDD for Rank 8, MCS17 - Reuse Table A.3.2.1.1-3 R.PDSCH.1-3.2 FDD for Rank 2, MCS19 - R.PDSCH.1-3.6 FDD for Rank 4, MCS17 - R.PDSCH.1-3.7 FDD for Rank 8, MCS17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for 8Rx UE will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.2.2 and A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

Table A.3.2.2.2-3: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD | R.PDSCH.2-3.2 TDD | R.PDSCH.2-3.3 TDD | R.PDSCH.2-3.4 TDD | R.PDSCH.2-3.5 TDD | R.PDSCH.2-3.6 TDD | R.PDSCH.2-3.7 TDD (Note 5) |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 | 20 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 53 (Note 3) | 53 (Note 4) | 51 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 | 19 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 | 0.51 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 1 | 4 | 4 |
| Number of DMRS REs |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 12 | 12 | 12 | 6 | 12 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 24 | 24 | 24 | 12 | 24 | 48 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 23040 | 11528 | 11528 | 6528 | 38963 | 26120 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 83976 | 77896 | 38936 | 38936 | 20496 | 131176 | 104496 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 3 | 2 | 2 | 1 | 5 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 | 10 | 5 | 5 | 3 | 16 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 160272 | 137376 | 68688 | 68688 | 38556 | 290304 | 229248 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 45792 | 22896 | 22896 | 12852 | 91584 | 61056 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 | 152640 | 76320 | 76320 | 40392 | 305280 | 244224 |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 109.768 | 54.869 | 54.869 | 28.975 | 184.88 | 146.29 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 52. Note 4: PDSCH is scheduled in PRB numbers from 53 to 105. Note 5: Two codewords and given per codeword |  |  |  |  |  |  |  |  |

END OF CHANGE 1

START OF CHANGE 2

Table A.3.2.1.1-3: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 FDD | R.PDSCH.1-3.2 FDD | R.PDSCH.1-3.3 FDD | R.PDSCH.1-3.4 FDD | R.PDSCH.1-3.5 FDD | R.PDSCH.1-3.6 FDD | R.PDSCH.1-3.7 FDD (Note 5) |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 26 (Note 3) | 26 (Note 4) | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 | 19 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 | 0.51 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 1 | 4 | 4 |
| Number of DMRS REs |  | 12 | 24 | 24 | 24 | 12 | 24 | 48 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 42016 | 37896 | 18960 | 18960 | 21000 | 64552 | 51216 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 5 | 5 | 3 | 3 | 3 | 8 | 7 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 78624 | 67392 | 33696 | 33696 | 39312 | 142272 | 112320 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 82368 | 74880 | 37440 | 37440 | 41184 | 149760 | 119808 |
| Max. Throughput averaged over 2 frames | Mbps | 39.915 | 36.001 | 18.012 | 18.012 | 19.950 | 61.32 | 48.66 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 25. Note 4: PDSCH is scheduled in PRB numbers from 26 to 51. Note 5: Two codewords and given per codeword |  |  |  |  |  |  |  |  |

END OF CHANGE 2