3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316472

Xiamen, China, Oct 9 – Oct 13, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 RSD for FDD bands

Agenda Item:	4.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

This paper discusses the remaining PC2 RSD issues as listed in the WF [1].

# 2	Discussion

Following the methodology used in our previous contribution [2], the PC2 RSD for band n13, n14 and n7 are estimated as in the table below:

Table 1: REFSENS degradation for PC2 (SCS=15kHz)

| Band | Tx arch | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n13 | 1Tx | 0.9 | 1.0 | - | - | - | - | - | - | - | - |
|  | 2Tx | 2.0 | 2.0 | - | - | - | - | - | - | - | - |
| n14 | 1Tx | 0.9 | 1.0 | - | - | - | - | - | - | - | - |
|  | 2Tx | 2.0 | 2.0 | - | - | - | - | - | - | - | - |
| n7 2Rx | 1Tx | 0.8 | 0.8 | 0.8 | 1.1 | 1.1 | 1.1 | 1.1 | 1.1 | - | 3.2 |
|  | 2Tx | 1.4 | 1.5 | 1.4 | 1.3 | 1.5 | 1.5 | 1.5 | 1.5 | - | 6.2 |
| n7 4Rx | 1Tx | 0.7 | 0.7 | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | - | 3.2 |
|  | 2Tx | 1.1 | 1.1 | 1.1 | 1.1 | 1.2 | 1.2 | 1.2 | 1.2 | - | 3.6 |
| Note: 4Rx is the baseline for band n7. 2Rx results are presented for information. |  |  |  |  |  |  |  |  |  |  |  |

# 3	Conclusion

Based on the analysis in Section 2, we have the following proposal:

Proposal 1: Take into account the RSD values in Table 1 when deciding the PC2 RSD for band n13, n14 and n7.

# References

- R4-2314646 WF on HPUE for FDD single bands, China Unicom, RAN4#108

- R4-2300715 MSD for PC2 FDD bands, Huawei, HiSilicon, RAN4#106

- RP-230352 Revised WID: high power UE (power class 2) for NR FR1 FDD single band, China Unicom, RAN#99