3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316473

Xiamen, China, Oct 9 – Oct 13, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for band n7 NS_46

Agenda Item:	4.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

In this paper, we present our analysis on the additional spectrum emission requirements of NS_46 as well as the A-MPR simulation results.

# 2	Discussion

### 2.1 Additional Spectrum Emission Requirements of NS_46

##### 6.5.3.3.25	Requirement for network signalling value "NS_46"

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

Observation 1: RB restrictions (<=54 RB) are applied for BW=15/20MHz when NS_46 is signalled.

Observation 2: The PC3 A-MPR is only specified for BW>20MHz.

### 2.2 A-MPR Simulation Results

The simulation results are obtained for NS_46 based on 1Tx PC2, which are shown in Table 1 below. The figures in the 2nd column show the differences in the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 1: A-MPR simulation results for NS_46

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 25MHz |  |  |
| 30MHz |  |  |
| 35MHz |  |  |
| 40MHz |  |  |
| 50MHz |  |  |

### 2.3 A-MPR Simulation Results for BW ≤ 20MHz

Although PC3 A-MPR is not defined for BW ≤ 20MHz, we provide the following PC2 A-MPR simulation results to facilitate the discussion on the potential extension.

Table 2: A-MPR simulation results for NS_46 for BW ≤ 20MHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 20MHz |  |  |
| 15MHz |  |  |

### 2.4 A-MPR Requirements for NS_46

<Agreement>: NS_46 PC2 A-MPR will be specified, and use the values proposed in R4-2313553 (below) ) as starting point.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 19.8 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

As per the WF [1], the A-MPR shown above is used as the starting point for further study. It’s noticed that:

Observation 3: In the starting point proposal [1], the carrier centre frequency range for BW=25MHz is set to 2512.5 ≤ FC ≤ 2557.5 MHz, which is different from the range specified for PC3 A-MPR, namely 2534.5 ≤ FC ≤ 2557.5 MHz.

Observation 4: In the starting point proposal [1], the A-MPR values of A5/A6/A7/A8 are almost the same as that for PC3, which seems unreasonable for PC2.

# 3	Conclusion

In this paper, we have presented further simulation results for NS_46 based on 1Tx PC2. From our analysis, we have made the following observations:

Observation 1: RB restrictions (<=54 RB) are applied for BW=15/20MHz when NS_46 is signalled.

Observation 2: The PC3 A-MPR is only specified for BW>20MHz for NS_46.

Observation 3: In the starting point proposal [1], the carrier centre frequency range for BW=25MHz is set to 2512.5 ≤ FC ≤ 2557.5 MHz, which is different from the range specified for PC3 A-MPR, namely 2534.5 ≤ FC ≤ 2557.5 MHz.

Observation 4: In the starting point proposal [1], the A-MPR values of A5/A6/A7/A8 are almost the same as that for PC3, which seems unreasonable for PC2.

Based on our study, we propose the following:

Proposal 1: Align the carrier centre frequency range for BW=25MHz with that for PC3 when defining the A-MPR requirements.

Proposal 2: Consider specifying A-MPR and removing RB restrictions for BW=15/20MHz for PC2 capable UEs.

Proposal 3: Take into account the simulation results in this contribution when determining the PC2 A-MPR for NS_46, especially for the A-MPR values of A5/A6/A7/A8.

# References

- R4-2314646 WF on HPUE for FDD single bands, China Unicom, RAN4#108