3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316475

Xiamen, China, Oct 9 – Oct 13, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for NS_07

Agenda Item:	4.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

In this paper, we present our A-MPR simulation results for NS_07 based on 1Tx PC2.

# 2	Discussion

### 2.1 Additional Spectrum Emission Requirements of NS_07

##### 6.5.2.3.4	Requirements for network signalling value "NS_06" or “NS_07”

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.4-1.

Table 6.5.2.3.4-1: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

##### 6.5.3.3.26	Requirement for network signalling value "NS_07"

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.26-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.26-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard standard deviation < 0.5 dB. |  |  |

Observation 1: Only BW=10MHz is included in Table 6.5.3.3.26-1 for NS_07. It seems that BW=5MHz is missing.

### 2.2 A-MPR Simulation Results

The simulation results are obtained for NS_07 based on 1Tx PC2, which are shown in Table 1 below. The figures in the 2nd column show the differences in the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 1: A-MPR simulation results for NS_07

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 5MHz |  |  |
| 5MHz guard=2.5MHz |  |  |
| 10MHz |  |  |

# 3	Conclusion

In this paper, we have presented further simulation results for NS_07 based on 1Tx PC2.

Observation 1: Only BW=10MHz is included in Table 6.5.3.3.26-1 for NS_07. It seems that BW=5MHz is missing.

Proposal 1: Take the simulation results into account when deciding the PC2 A-MPR for NS_07.

# References

- R4-2314646 WF on HPUE for FDD single bands, China Unicom, RAN4#108