3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316493

Xiamen, China, October 09 – October 13, 2023

Source:	Ericsson

Title:	NTN RRM testing work for NGSO deployments

Agenda item:	8.1

Document for:	Approval

# 1	Introduction

In plenary the following WF was agreed [1]:

… - RAN tasks RAN4 to discuss and conclude in Oct meeting on - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications. - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing - Further discuss TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model in future release i.e. Rel-19 - No changes to the RAN4 core requirements …

In this contribution we discuss the condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications.

# 2	Discussion

The WF [1] gives some advice as what to develop:

- The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications.

  - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test

  - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

More specifically the WF gives guidance that RAN5 needs the range of delay values, from RAN 4, for verification of NGSO UE delay pre-compensation functionality.

The rel-16 TR [2] lists the following orbits for NTN:

Table 1: Types of NTN platforms (Table 4.1-1 in [2])

| Platforms | Altitude range | Orbit | Typical beam footprint size |
| --- | --- | --- | --- |
| Low-Earth Orbit (LEO) satellite | 300 – 1500 km | Circular around the earth | 100 – 1000 km |
| Medium-Earth Orbit (MEO) satellite | 7000 – 25000 km |  | 100 – 1000 km |
| Geostationary Earth Orbit (GEO) satellite | 35 786 km | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | 200 – 3500 km |
| UAS platform (including HAPS) | 8 – 50 km (20 km for HAPS) |  | 5 - 200 km |
| High Elliptical Orbit (HEO) satellite | 400 – 50000 km | Elliptical around the earth | 200 – 3500 km |

LEO orbits below 450 km require periodic orbit raising to be economical. In the end the rel-16 NTN prestudy opted for two LEO scenarios of 600 km and 1200 km orbit altitude above earth.

Table 2: Reference scenario parameters (Table 4.2-2 in [2])

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) 12.89 ms (600km) 20.89 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

The very lowest altitude ranges < 450 km require orbit raising to stay in orbit due to drag. From this range the NTN feasibility study in rel-16 , documented in [2], picked the following scenarios: NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above.

From Table 2 above we can conclude these parameters for LEO scenarios:

Table 3: LEO scenarios

| Scenarios | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- |
| Altitude | 600 km 1,200 km |
| Max distance between satellite and user equipment at min elevation angle | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) 12.89 ms (600km) 20.89 ms (1200km) |

A set of typical RTT as functions of time when the satellite enters above azimuth limit and move towards UE at origo, passing straight above UE and moving away beyond azimuth limit can be found in Figure 1 below:

Figure 1: RTT for LEO 600 km and 1200 km with UE at origo and satellite passing straight above

The Max Round Trip Delay (propagation delay only) for service and feeder link distance together is calculated as:

4*Max distance between satelite and userc						(1)

For example for 600 km altitude we get max distance between satellite and user equipment at min elevation angle of 1932 km and the formula above gives 4*1932 km / c ≈ 25.77 ms.

The Max Round Trip Delay (propagation delay only) for service link distance together is calculated as:

2*Max distance between satelite and userc						(2)

We get max round trip delays of 25.77 ms (600km) and 41.77 ms (1200km) for service and feeder link together and half of this, 12.89 ms (600km) and 20.89 ms (1200km) for only the service link only depending on if the reference point is at the gateway (round trip is service and feeder link) or if the reference point is at the satellite (round trip is service link only).

Observation 1: TR 38.821 lists max round trip delays of 25.77 ms (600km) and 41.77 ms (1200km) for service and feeder link together and half of this, 12.89 ms (600km) and 20.89 ms (1200km) for only the service link only.

For the Min Round Trip Delay (propagation delay only) we can use the altitude of 600 km and 1200 km in formulas (1) and (2). We get min round trip delays of 8 ms (600km) and 16 ms (1200km) for service and feeder link together and half of this, 4 ms (600km) and 8 ms (1200km) for only the service link only depending on if the reference point is at the gateway (round trip is service and feeder link) or if the reference point is at the satellite (round trip is service link only).

Observation 2: We get min round trip delays of 8 ms (600km) and 16 ms (1200km) for service and feeder link together and half of this, 4 ms (600km) and 8 ms (1200km) for only the service link only.

This can be summarized in a table 4:

Table 4: Min and max round trip delays for LEO orbits

| LEO orbit | Min Round Trip Delay (ms) (propagation delay only) |  | Max Round Trip Delay (ms) (propagation delay only) |  |
| --- | --- | --- | --- | --- |
|  | service and feeder links | Only service link | service and feeder links | Only service link |
| 600 km | 8 | 4 | 25.77 | 12.89 |
| 1200 km | 16 | 8 | 41.77 | 20.89 |

Based on the table we propose:

Proposal 1:	For LEO orbit 600 km we get a delay between 8 to 25.77 ms for service and feeder links (ref point at GW) and between 4 to 12.89 ms (ref point at satellite).

Proposal 2:	For LEO orbit 1200 km we get a delay between 16 to 41.77 ms for service and feeder links (ref point at GW) and between 8 to 20.89 ms (ref point at satellite).

The RTT delay compensation of the NTN UE UL is a key system aspect to test. Once the UE has read SIB19 ephimeris data and the UE has its own position the UE shall use these input to calculate total TA and apply to UL TX symbols. It is therefore important that During the test, the RTT trajectories follow RTT as a function in time as displayed in Figure 1.

Proposal 3: 	During the test, the RTT trajectories follow RTT as a function in time for LEO 600 km and LEO 1200 km.

# 3 	Summary

Observation 1: TR 38.821 lists max round trip delays of 25.77 ms (600km) and 41.77 ms (1200km) for service and feeder link together and half of this, 12.89 ms (600km) and 20.89 ms (1200km) for only the service link only.

Observation 2: We get min round trip delays of 8 ms (600km) and 16 ms (1200km) for service and feeder link together and half of this, 4 ms (600km) and 8 ms (1200km) for only the service link only.

Proposal 1:	For LEO orbit 600 km we get a delay between 8 to 25.77 ms for service and feeder links (ref point at GW) and between 4 to 12.89 ms (ref point at satellite).

Proposal 2:	For LEO orbit 1200 km we get a delay between 16 to 41.77 ms for service and feeder links (ref point at GW) and between 8 to 20.89 ms (ref point at satellite).

Proposal 3: 	During the test, the RTT trajectories follow RTT as a function in time for LEO 600 km and LEO 1200 km.

# References

[1] 		RP-232682, Summary of offline discussions for NTN testing, RAN WG4 Chair, RAN WG5 Chair, 3GPP TSG-RAN Meeting #101

[2]	3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), 3rd Generation Partnership Project