# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316510

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:			5.2.2

Source:					Qualcomm Incorporated

Title:                        Discussion on MU assessment for FR2 multi-Rx UE test methodology

Document for:		Approval

# Introduction

In RAN4#108, the MU framework of measurement grid was initially discussed [1]. In our paper submitted in RAN4#108bis [2], the MU analysis framework is proposed. In this paper, we will further discuss the MU assessment for multi-Rx UE RF, RRM and Demodulation testing.

# Discussion

## MU assessment for multi-Rx UE RF testing

As discussed in [2], the principle of MU assessment for multi-Rx UE RF testing is to leverage the uncertainty budget of legacy EIS spherical coverage specified in TR 38.903 considering the 2AoA can be controlled independently.

Observation 1: The uncertainty budget of legacy EIS spherical coverage can be leveraged for 2AoA multi-Rx UE RF testing considering the 2AoA can be controlled independently.

Proposal 1: The MU assessment of 2AoA RF testing shown in Table 1 and Table 2 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 1: Uncertainty assessment for wanted DL signal absolute power in 2AoA coverage measurement with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 7) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 6) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.00 | Rectangular | 1.73 | 0.00 |
| 15 | Influence of spherical coverage grid (NOTE 4) | 0.12 | Actual | 1 | 0.12 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | 0.75 |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 23 | Quality of quiet zone for calibration process (NOTE 7) | 0.4 | Actual | 1.00 | 0.4 |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Measurement uncertainty |  |  |  | Value |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 4.86 |

Table 2: Total uncertainty assessment for 2AoA coverage measurement with IFF

| Measurement uncertainty | Value |
| --- | --- |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [%] | X% |
| Uncertainty related to measurement grid | Y% |
| Total Measurement uncertainty | Value |
| [2AoA spherical coverage] expanded uncertainty (1.96σ - confidence interval of 95 %) [%] | X+Y% |
| NOTE 1: X% is derived based on the simulations with different DL power vs percentage of 2AoA metric. NOTE 2: Y% is derived based on the simulations with measurement step size vs percentage of 2AoA metric. |  |

## MU assessment for multi-Rx UE RRM testing

Similar observation as multi-Rx UE RF testing, the uncertainty budget of legacy RRM can be leveraged for 2AoA multi-Rx UE RF testing considering the 2AoA can be controlled independently. And it should be noted that for multi-Rx UE RRM testing, the metric of power level should be used for MU analysis.

Proposal 2: The MU assessment of 2AoA UE RRM testing shown in Table 3 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 3: Uncertainty assessment for Multi-Rx RRM testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 22 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.19 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex D.2 of TR 38.810 [13] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

## MU assessment for multi-Rx UE Demodulation testing

Similar observation as multi-Rx UE RRM testing, the uncertainty budget of legacy Demodulation can be leveraged for 2AoA multi-Rx UE RF testing considering the 2AoA can be controlled independently. And the metric of power level should be used for demodulation MU analysis.

Note that the element of impact on non-ideal isolation between branches for the wireless cable mode is FFS which is related to the min. isolation requirements and corresponding MU.

Proposal 3: The MU assessment of 2AoA demodulation testing shown in Table 4 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 4: Uncertainty assessment for Multi-Rx demodulation testing with IFF

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Signal-to-noise ratio uncertainty |  |  |  |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Positioning misalignment |  |  | [Normal] | [2.00] |  |
| 2 | Measure distance uncertainty |  |  | [Rectangular] | [1.73] |  |
| 3 | Quality of Quiet Zone |  |  | [Actual] | [1.00] |  |
| 4 | Mismatch |  |  | [Actual] | [1.00] |  |
| 5 | Standing wave between the DUT and measurement antenna |  |  | [U-shaped] | [1.41] |  |
| 6 | gNB emulator SNR uncertainty |  |  | [Normal] | [2.00] |  |
| 7 | Phase curvature |  |  | [U-shaped] | [1.41] |  |
| 8 | Amplifier uncertainties |  |  | [Normal] | [2.00] |  |
| 9 | Random uncertainty |  |  | [Normal] | [2.00] |  |
| 10 | Influence of the XPD |  |  | [U-shaped] | [1.41] |  |
| 11 | Insertion Loss Variation |  |  | [Rectangular] | [1.73] |  |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |  |  | [Actual] | [1.00] |  |
| 13 | Multiple measurement antenna uncertainty |  |  | [Actual] | [1.00] |  |
| 14 | DUT repositioning |  |  | [Rectangular] | [1.73] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 15 | Mismatch |  |  | [U-shaped] | [1.41] |  |
| 16 | Amplifier Uncertainties |  |  | [Normal] | [2.00] |  |
| 17 | Misalignment of positioning System |  |  | [Normal] | [2.00] |  |
| 18 | Uncertainty of the Network Analyzer |  |  | [Normal] | [2.00] |  |
| 19 | Uncertainty of the absolute gain of the calibration antenna |  |  | [Normal] | [2.00] |  |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  |  | [Rectangular] | [1.73] |  |
| 21 | Phase centre offset of calibration antenna |  |  | [Rectangular] | [1.73] |  |
| 22 | Quality of quiet zone for calibration process |  |  | [Actual] | [1.00] |  |
| 23 | Standing wave between reference calibration antenna and measurement antenna |  |  | [U-shaped] | [1.41] |  |
| 24 | Influence of the calibration antenna feed cable |  |  | [Normal] | [2.00] |  |
| 25 | Insertion Loss Variation |  |  | [Rectangular] | [1.73] |  |
|  | Systematic uncertainties |  |  |  |  | Value |
| 26 | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  | FFS (Note 1) |
| Total Signal-to-Noise ratio uncertainty |  |  |  |  |  |  |
| Other contributors affecting test result |  |  |  |  |  |  |
| 27 | gNB emulator fading model impairments |  |  | [Normal] | [2.00] |  |
| 28 | AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig (Note 3) |  |  | [Actual] | 1.00 |  |
| 29 | Result variation due to finite test time |  |  | [Actual] | [1.00] |  |
| Note 1: FFS which is relying on the min. isolation requirements. |  |  |  |  |  |  |

# Conclusions

In this paper, we discuss the MU for the multi-Rx UE testing. The following observation and proposals are made:

Observation 1: The uncertainty budget of legacy EIS spherical coverage can be leveraged for 2AoA multi-Rx UE RF testing considering the 2AoA can be controlled independently.

Proposal 1: The MU assessment of 2AoA RF testing shown in Table 1 and Table 2 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 1: Uncertainty assessment for wanted DL signal absolute power in 2AoA coverage measurement with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 7) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 6) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.00 | Rectangular | 1.73 | 0.00 |
| 15 | Influence of spherical coverage grid (NOTE 4) | 0.12 | Actual | 1 | 0.12 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 16 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 17 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | 0.75 |
| 20 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 21 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 22 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 23 | Quality of quiet zone for calibration process (NOTE 7) | 0.4 | Actual | 1.00 | 0.4 |
| 24 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 25 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 26 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Measurement uncertainty |  |  |  | Value |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 4.86 |

Table 2: Total uncertainty assessment for 2AoA coverage measurement with IFF

| Measurement uncertainty | Value |
| --- | --- |
| Wanted DL signal absolute power (1.96σ - confidence interval of 95 %) [%] | X% |
| Uncertainty related to measurement grid | Y% |
| Total Measurement uncertainty | Value |
| [2AoA spherical coverage] expanded uncertainty (1.96σ - confidence interval of 95 %) [%] | X+Y% |
| NOTE 1: X% is derived based on the simulations with different DL power vs percentage of 2AoA metric. NOTE 2: Y% is derived based on the simulations with measurement step size vs percentage of 2AoA metric. |  |

Proposal 2: The MU assessment of 2AoA UE RRM testing shown in Table 3 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 3: Uncertainty assessment for Multi-Rx RRM testing with IFF

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 22 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.19 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex D.2 of TR 38.810 [13] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

Proposal 3: The MU assessment of 2AoA demodulation testing shown in Table 4 should be adopted for IFF measurement setup and captured in the TR 38.871.

Table 4: Uncertainty assessment for Multi-Rx demodulation testing with IFF

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Signal-to-noise ratio uncertainty |  |  |  |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Positioning misalignment |  |  | [Normal] | [2.00] |  |
| 2 | Measure distance uncertainty |  |  | [Rectangular] | [1.73] |  |
| 3 | Quality of Quiet Zone |  |  | [Actual] | [1.00] |  |
| 4 | Mismatch |  |  | [Actual] | [1.00] |  |
| 5 | Standing wave between the DUT and measurement antenna |  |  | [U-shaped] | [1.41] |  |
| 6 | gNB emulator SNR uncertainty |  |  | [Normal] | [2.00] |  |
| 7 | Phase curvature |  |  | [U-shaped] | [1.41] |  |
| 8 | Amplifier uncertainties |  |  | [Normal] | [2.00] |  |
| 9 | Random uncertainty |  |  | [Normal] | [2.00] |  |
| 10 | Influence of the XPD |  |  | [U-shaped] | [1.41] |  |
| 11 | Insertion Loss Variation |  |  | [Rectangular] | [1.73] |  |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |  |  | [Actual] | [1.00] |  |
| 13 | Multiple measurement antenna uncertainty |  |  | [Actual] | [1.00] |  |
| 14 | DUT repositioning |  |  | [Rectangular] | [1.73] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 15 | Mismatch |  |  | [U-shaped] | [1.41] |  |
| 16 | Amplifier Uncertainties |  |  | [Normal] | [2.00] |  |
| 17 | Misalignment of positioning System |  |  | [Normal] | [2.00] |  |
| 18 | Uncertainty of the Network Analyzer |  |  | [Normal] | [2.00] |  |
| 19 | Uncertainty of the absolute gain of the calibration antenna |  |  | [Normal] | [2.00] |  |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  |  | [Rectangular] | [1.73] |  |
| 21 | Phase centre offset of calibration antenna |  |  | [Rectangular] | [1.73] |  |
| 22 | Quality of quiet zone for calibration process |  |  | [Actual] | [1.00] |  |
| 23 | Standing wave between reference calibration antenna and measurement antenna |  |  | [U-shaped] | [1.41] |  |
| 24 | Influence of the calibration antenna feed cable |  |  | [Normal] | [2.00] |  |
| 25 | Insertion Loss Variation |  |  | [Rectangular] | [1.73] |  |
|  | Systematic uncertainties |  |  |  |  | Value |
| 26 | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  | FFS (Note 1) |
| Total Signal-to-Noise ratio uncertainty |  |  |  |  |  |  |
| Other contributors affecting test result |  |  |  |  |  |  |
| 27 | gNB emulator fading model impairments |  |  | [Normal] | [2.00] |  |
| 28 | AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig (Note 3) |  |  | [Actual] | 1.00 |  |
| 29 | Result variation due to finite test time |  |  | [Actual] | [1.00] |  |
| Note 1: FFS which is relying on the min. isolation requirements. |  |  |  |  |  |  |

# References

- R4-2313888, WF for FR2 OTA test SI, Qualcomm, RAN4#108, August 2023.

- R4-2316507, Discussion on RF test method for FR2 multi-Rx UE, RAN4#108bis, Oct. 2023