3GPP TSG-RAN WG4 Meeting#108bis                              R4-2316524

Xiamen, China, Oct 9 – Oct 13,2023

Agenda item:	5.13.2.3

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Rx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on the remaining issues for ATG UE Rx RF requirements.

Discussion

## 2.1. Maximum input power

In the previous discussions for maximum input power, the major difference between two campus is about the ATG BS antenna installation or ATG deployment assumption used to define the corresponding RF requirement for ATG BS and ATG UE. As shown in the following figure 1, ATG BS with its antenna installation steer vertically towards to the sky to provide the service. However as far as we know based on the published information from US GoGo [9], EAN (Europe aviation network [10,11]) and Chines deployment, ATG BS antenna in reality is installed as shown in Figure 2. Because once the BS antenna is steering towards the sky, then this will increase the power consumption together with heating problems since there are limited wing heat dissipation in the vertical domain compared with horizontal domain wing dissipation.

In addition, the ATG BS installation as shown in Figure 1 is also not aligned with the coexistence simulation assumption, therefore based on above reasoning, we propose to use the ATG deployment as shown in Figure 2 for further requirement definition for both ATG BS and ATG UE.

Figure 1. illustration of ATG deployment -1

Figure 2. Figure 1. illustration of ATG deployment -2

Figure 3. EAN deployment [11]

Based on the coupling loss obtained from the coexistence study as shown in Figure 4, the minimum coupling loss between ATG UE and ATG BS is around 100dBc which is 30dBc lower than MCL in FR1 Urban macro scenario, therefore based on this evaluation results, we propose to further relax the maximum input power by 30dBc.

Figure 4. Coupling loss between ATG CPE and ATG BS

Proposal 1: the maximum input power for ATG CPE is proposed as following:

Table 7.4-1: Maximum input level for ATG CPE

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -552 |  |  |  | -542 | -532 | -522 | -512 | -502 |  |  |  |
|  |  | -573 |  |  |  | -563 | -553 | -543 | -533 | -523 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.2. OOBB requirement

During the last RAN4 meeting, there were some proposals to investigate the OOBB requirement from the mounted aircraft altimeter perspective within frequency range 4.2-4.4GHz. Indeed the perceived OOBB requirement should be also decided by isolation distance between ATG CPE antenna and altimeter antenna and transmit power of aircraft altimeter. In addition, as shown in the following figure, there are a couples of other incumbent service which might have the impacts on the receiver blocking of ATG CPE from various scenarios, e.g. Radar, Satellite, ADS-B, MLS, TACAN, ATC etc. In short, all these blocking factors should be taken into the account for the practical implementation at the end to ensure its robust operation under the different environment, however from 3GPP perspective, all these regulatory related information are very difficult to be further discussed, therefore we propose to reuse the FR1 UE OOBB requirement as baseline and any regulatory requirement higher than baseline requirement, more stringent OOBB requirement should be applicable similar as the principle agreed for LTE based broadcast BS requirement definition;

Proposal 2: use the existing OOBB requirements in TS 38.101-1 as baseline; if there are any regulatory requirement higher than baseline requirement, then regulatory requirement should be applicable.

## 2.3. Receiver spurious response requirement

Similar as in-band blocking requirement, we propose to reuse the receiver spurious response requirement for ATG CPE.

Proposal 3: use the receiver spurious response requirement in TS 38.101-1 as baseline

Conclusions

In this contribution, we shared some further views for ATG UE Rx RF requirements and proposals are made as following:

Proposal 1: the maximum input power for ATG CPE is proposed as following:

Table 7.4-1: Maximum input level for ATG CPE

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -552 |  |  |  | -542 | -532 | -522 | -512 | -502 |  |  |  |
|  |  | -573 |  |  |  | -563 | -553 | -543 | -533 | -523 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: use the existing requirements in TS 38.101-1 as baseline; if there are any regulatory requirement higher than baseline requirement, then regulatory requirement should be applicable.

Proposal 3: use the receiver spurious response requirement in TS 38.101-1 as baseline

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738,WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.

- R4-2306609,WF on ATG UE RF requirements, Huawei, approved.

- R4-2310400,WF on UE RF requirements for ATG, Huawei, approved.

- R4-2314761,WF on NR_ATG_UERF_part2

- https://www.gogoair.com/gogo-5g-progress/

- https://www.youtube.com/watch?v=CyUa6sib_zc

- Progress of ATG Network Abroad and Design of ATG Ground Antenna with Cosecant Squared Beam, China Academic Journal Electronic Publishing House.

Status of the requirement

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | Status: completed |
| Reference sensitivity | Yes | Status: completed |
| Maximum input level | No | Status: ongoing |
| ACS | No | Status: complete Issue 3-2-1: ATG UE ACLR / ACS Agreement: 30dBc ACLR and [33dBc] ACS for ATG UE.. |
| Blocking characteristics |  |  |
| - In-band | No | Status: ongoing To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | Status: ongoing |
| - Narrow band | No | Status: completed |
| Spurious response | No | Status: ongoing |
| Intermodulation | No | Status: ongoing Not applicable |
| Spurious emissions | No | Status: completed |