3GPP TSG-RAN WG4 Meeting#108bis                              R4-2316527

Xiamen, China, Oct 9 – Oct 13, 2023

Agenda item:	5.19.2.1

Source: 	ZTE Corporation

Title: 	Simulation results for full duplex coexistence in adjacent channel scenario

Document for:	Approval

Introduction

In RAN#94e meeting, the work item [RP-221352] on study on evolution of NR duplex operation was approved as one of Rel-18 RAN1 package. During last RAN4 meeting, we have lot reached lots of consensus on adjacent co-existence in [5] . In this contribution, we want to share some evaluation results for further discussions.

Table 2.1-1: Scenarios for SBFD co-ex study

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | Low |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 7 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Micro | Urban Micro | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |

The aggressor and victim combination is listed in Table 2.1-2 below.

[Editor’s Note: Table 2.1-2 refers to R4-2217466. The color of figure would be updated for clarification purpose later.]

Table 2.1-2: Victim, aggressor and aggressor baseline for SBFD co-ex study

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |

Discussion

## 2.1. FR1

### 2.1.1 Case 1: FR1 Urban Macro scenario

| Victim | Aggressor | Company | Victim WITH Aggressor ACI compared to Vctim WITHOUT ACI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB |  |  |  |  | SINR degradation @50% in dB |  |  |  |  | TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
|  |  |  | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| NR TDD DL | SBFD (DU) | ZTE | 1.00 | 0.82 | 0.55 | 0.43 | 0.36 | 0.78 | 0.60 | 0.48 | 0.39 | 0.32 | 9.73 | 8.12 | 5.39 | 4.33 | 3.79 | 3.03 | 2.36 | 1.88 | 1.51 | 1.25 |
| SBFD (UL) | NR TDD DL | ZTE | 7.65 | 6.36 | 5.14 | 4.10 | 3.21 | 1.78 | 1.38 | 1.08 | 0.82 | 0.63 | 76.99 | 69.83 | 61.26 | 52.53 | 43.50 | 13.00 | 10.14 | 7.91 | 6.02 | 4.67 |
| SBFD (DL) | NR TDD DL | ZTE | 0.36 | 0.30 | 0.20 | 0.11 | 0.11 | 0.44 | 0.37 | 0.28 | 0.19 | 0.15 | 4.41 | 3.55 | 2.36 | 1.31 | 1.29 | 1.93 | 1.62 | 1.22 | 0.84 | 0.66 |

Based on the simulation results in the above table, it could be found that the performance degradation for NR TDD DL interfering SBFD UL is very severe. It could be expected that similar observation could be made for SBFD interfering NR TDD UL, however the latter scenario is de-prioritized in Rel-18.

Observation 1: for Case 1 in FR1 Urban macro scenario, the performance degradation in NR TDD DL interfering SBFD UL is very severe,, other coexistence performance in Case 1 is still acceptable.

### 2.1.2 Case 3: FR1 Indoor scenario

| Victim | Aggressor | Company | Victim WITH Aggressor ACI compared to Vctim WITHOUT ACI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB |  |  |  |  | SINR degradation @50% in dB |  |  |  |  | TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
|  |  |  | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| NR TDD DL | SBFD (DU) | ZTE | 0.04 | 0.03 | 0.03 | 0.03 | 0.00 | 0.35 | 0.26 | 0.19 | 0.13 | 0.06 | 0.52 | 0.41 | 0.40 | 0.37 | 0.04 | 0.96 | 0.69 | 0.49 | 0.35 | 0.24 |
| SBFD (UL) | NR TDD DL | ZTE | 0.07 | 0.03 | 0.02 | 0.01 | 0.01 | 0.12 | 0.09 | 0.07 | 0.05 | 0.03 | 0.87 | 0.31 | 0.29 | 0.07 | 0.07 | 0.96 | 0.67 | 0.45 | 0.30 | 0.20 |
| SBFD (DL) | NR TDD DL | ZTE | 0.03 | 0.01 | 0.01 | 0.01 | 0.00 | 0.26 | 0.18 | 0.12 | 0.07 | 0.07 | 0.37 | 0.15 | 0.09 | 0.04 | 0.02 | 0.81 | 0.58 | 0.40 | 0.28 | 0.19 |

Based on the simulation results in the above table, there are no major performance degradation identified yet, in short,  there are no major coexistence problems for SBFD deployment in Indoor scenario.

Observation 2: for Case 3 in FR1 indoor scenario, the performance degradation in the above scenarios are less than 5% coexistence criteria.

### 2.1.3 Case 5: FR1 Micro-Micro scenario

| Victim | Aggressor | Company | Victim WITH Aggressor ACI compared to Vctim WITHOUT ACI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB |  |  |  |  | SINR degradation @50% in dB |  |  |  |  | TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
|  |  |  | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| NR TDD DL | SBFD (DU) | ZTE | 0.65 | 0.61 | 0.58 | 0.56 | 0.55 | 0.16 | 0.14 | 0.13 | 0.13 | 0.12 | 11.37 | 10.69 | 10.20 | 9.90 | 9.65 | 2.09 | 1.87 | 1.75 | 1.68 | 1.65 |
| SBFD (UL) | NR TDD DL | ZTE | 0.84 | 0.51 | 0.25 | 0.07 | 0.02 | 0.30 | 0.22 | 0.16 | 0.11 | 0.07 | 15.41 | 9.54 | 4.59 | 1.09 | 0.12 | 4.59 | 3.34 | 2.47 | 1.67 | 1.13 |
| SBFD (DL) | NR TDD DL | ZTE | 0.03 | 0.03 | 0.02 | 0.02 | 0.01 | 0.04 | 0.02 | 0.02 | 0.01 | 0.01 | 0.63 | 0.48 | 0.38 | 0.38 | 0.27 | 0.53 | 0.35 | 0.28 | 0.17 | 0.14 |

Based on the simulation results in the above table, he performance degradation for NR TDD DL interfering SBFD UL is higher than 5% coexistence criteria. Considering the better achievable ACLR and ACS requirement of SBFD BS and the legacy BS, then it should be feasible to maintain the performance degradation less than 5%.

Observation 3: for Case 5 in FR1 micro scenario, the performance degradation for NR TDD DL interfering SBFD UL is still higher than 5% coexistence criteria.

## 2.2. FR2

### 2.2.1 Case 6: FR2 Urban Macro scenario

| Victim | Aggressor | Company | Victim WITH Aggressor ACI compared to Vctim WITHOUT ACI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB |  |  |  |  | SINR degradation @50% in dB |  |  |  |  | TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
|  |  |  | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| NR TDD DL | SBFD (DU) | ZTE | 1.49 | 1.37 | 1.28 | 1.24 | 1.18 | 0.43 | 0.37 | 0.31 | 0.27 | 0.24 | 8.91 | 8.16 | 7.62 | 7.40 | 7.02 | 1.52 | 1.38 | 1.26 | 1.18 | 1.11 |
| SBFD (UL) | NR TDD DL | ZTE | 1.52 | 1.11 | 0.75 | 0.53 | 0.38 | 0.04 | 0.03 | 0.03 | 0.02 | 0.02 | 13.12 | 9.60 | 6.57 | 4.61 | 3.36 | 2.04 | 1.49 | 1.07 | 0.75 | 0.52 |
| SBFD (DL) | NR TDD DL | ZTE | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.03 | 0.03 | 0.02 | 0.02 | 0.02 |

Based on the simulation results in the above table, he performance degradation for NR TDD DL interfering SBFD UL is higher than 5% coexistence criteria. Considering the better achievable ACLR and ACS requirement of SBFD BS and the legacy BS, then it might be feasible to maintain the performance degradation less than 5% or acceptable with slightly higher performance degradation than 5% .

Observation 4: for Case 6 in FR2 Urban macro scenario, the performance degradation for NR TDD DL interfering SBFD UL is still slightly higher than 5% coexistence criteria.

### 2.2.2 Case 9: FR2 Indoor scenario

| Victim | Aggressor | Company | Victim WITH Aggressor ACI compared to Vctim WITHOUT ACI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB |  |  |  |  | SINR degradation @50% in dB |  |  |  |  | TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
|  |  |  | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| NR TDD DL | SBFD (DU) | ZTE | 0.74 | 0.48 | 0.35 | 0.26 | 0.18 | 0.88 | 0.69 | 0.54 | 0.41 | 0.34 | 5.37 | 3.44 | 2.51 | 1.89 | 1.26 | 1.41 | 1.08 | 0.82 | 0.63 | 0.48 |
| SBFD (UL) | NR TDD DL | ZTE | 0.59 | 0.45 | 0.35 | 0.26 | 0.18 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 5.35 | 4.12 | 3.19 | 2.41 | 1.72 | 0.10 | 0.08 | 0.07 | 0.05 | 0.04 |
| SBFD (DL) | NR TDD DL | ZTE | 0.11 | 0.10 | 0.09 | 0.05 | 0.03 | 0.17 | 0.13 | 0.10 | 0.09 | 0.07 | 0.86 | 0.72 | 0.69 | 0.46 | 0.28 | 0.24 | 0.18 | 0.13 | 0.10 | 0.07 |

Based on the simulation results in the above table, there are no major performance degradation identified yet, in short,  there are no major coexistence problems for SBFD deployment in FR2 Indoor scenario.

Observation 5: for Case 9 in FR2 indoor scenario, the performance degradation in the above scenarios are less than 5% coexistence criteria.

Conclusions

In this contribution, we want to share some further views and initial simulation results on the coexistence of full duplex BS in the adjacent channel scenario and proposals are made as following:

Observation 1: for Case 1 in FR1 Urban macro scenario, the performance degradation in NR TDD DL interfering SBFD UL is very severe,, other coexistence performance in Case 1 is still acceptable.

Observation 2: for Case 3 in FR1 indoor scenario, the performance degradation in the above scenarios are less than 5% coexistence criteria.

Observation 3: for Case 5 in FR1 micro scenario, the performance degradation for NR TDD DL interfering SBFD UL is still higher than 5% coexistence criteria.

Observation 4: for Case 6 in FR2 Urban macro scenario, the performance degradation for NR TDD DL interfering SBFD UL is still slightly higher than 5% coexistence criteria.

Observation 5: for Case 9 in FR2 indoor scenario, the performance degradation in the above scenarios are less than 5% coexistence criteria.

References

- RP-221352, Study on evolution of NR duplex operation, CMCC, Samsung, Approved

- TR38.828,Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)

- R4-2214379,WF on Simulation assumption for adjacent co-existence study, CMCC, approved.

- R4-2214378,WF on adjacent channel co-existence study, Samsung, approved.

- R4-2217466 WF for adjacent channel co-existence evaluation of SBFD operation, Samsung,CMCC, approved.

- R4-2217513,WF on SBFD feasibility study and RF impact: UE aspects, Qualcomm, approved.