3GPP TSG-RAN WG4 Meeting #108bis									            R4-2316550

Xiamen, China, October 9 – 13, 2023

Agenda item:	5.9.2.2

Source:	Apple

Title:	Discussion on case 1 requirements of R18 gap enhancement

Document for:	Discussion

# Introduction

Case 1 requirements of R18 gap enhancement was discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we continue discussing the open issue.

# Discussion

## 2.1 Sub-topic 3-2: Collision handling for dynamic collisions

Sub-topic description: This sub-topic covers issues related to the collision cases for concurrent gaps with Pre-MG. The summary of the issues on this topic are provided below: Scenario 1: the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) • RAN4 has an agreement. • Open issue: further clarification to the definition of this scenario might be needed. Scenario 2: pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG) • Open issue: whether to follow the same agreement from Scenario 1. Scenario 3: pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG. • Open issue: whether to follow - the same agreement from Scenario 1, or - the dropping role based on priority rule, or - other options. Scenario 4: One pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision (This scenario is for Pre-MG + Pre-MG). • Open issue: whether to follow - the same agreement from Scenario 1, and/or scenario 3, or - extend the delay to align with (5ms + T1), or - Other options Figure: the collision scenarios for concurrent gaps with Pre-MG during dynamic collision.

### Issue 3-2-1: [Case 1] Whether to update the definition of dynamic collision?

< Agreement >:

- Discuss the issue in the CR draft directly.

### Issue 3-2-2: [Case 1] - [Scenario 1] Further clarification on the agreement from scenario 1?

< Background >

- The collision scenario in this issue is depicted in the figure below:

< Agreement >:

- A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG and UE should continue the measurement within the MG#2

  - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used

  - FFS the exact wording to be captured in the specification in CR draft directly.

Regarding the TBD on whether same Pre-MG activation delay requirements as Rel-17 can still be re-used, we think it has already been covered by the agreement part “the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG”

Proposal 1: no need to further discuss Pre-MG activation delay for scenario 1 (the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)), since it has already been covered by the agreement “Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG”.

### Issue 3-2-3: [Case 1] - [Scenario 2] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Background:

  - The collision scenario in this issue is depicted in the figure below:

- FFS the options:

  - Option 1:

    - Same agreement as in (scenario 1), which is Agreement of Issue 3-3-2 from WF [R4-2310175].

  - Option 2:

    - When a pre-MG and a Type-2 MG collide and the pre-MG has higher priority, UE should drop the colliding Type-2 MG occasion, if

      - the deactivation procedure of pre-MG overlaps with time period T, where T starts from 4ms before the Type-2 MG occasion and ends at 4ms after the Type-2 MG occasion, and

      - the deactivation procedure of pre-MG ends earlier than the start of pre-MG occasion.

    - Option 2a: Additionally,

      - the Pre-MG will be deactivated immediately after the Pre-MG deactivation procedure.

      - data scheduling is expected within the MG occasion colliding with the Pre-MG deactivation procedure and the Pre-MG occasion after Pre-MG deactivation procedure.

Regarding scenario 2, we support to apply same agreement as that for scenario 1 to simplify requirement and UE design.

Proposal 2: for scenario 2 (when the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)), apply same agreement as in scenario 1.

### Issue 3-2-4: [Case 1] - [Scenario 3] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG

- Background:

  - The collision scenario in this issue is depicted in the figure below:

< Way forward>:

- Option 1: Same agreement as issue 3-3-2 (scenario 1).

- Option 2: The UE continues the measurement within the overlapped concurrent gap occasion (MG#2), i.e. existing priority rule applies without any change.

To simplify RRM requirements and UE design, we support a unified solution regardless of priority of Pre-MG and concurrent gap. i.e. option 1.

Proposal 3: regarding scenario 3 (when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG), apply same agreement as in scenario 1.

### Issue 3-2-5: [Case 1] - [Scenario 4] When one pre-configured MG deactivation procedure is fully overlapped with another pre-configured MG activation procedure triggered by single BWP switching during the dynamic collision

Background:

  - NW configures Pre-MG1 associated with BWP-1 and Pre-MG2 associated with BWP-2.

  - When UE switches the active DL BWP from BWP-1 to BWP-2, the SSB1 associated with BWP-1 will be outside the active BWP-2, but the SSB2 associated with BWP-2 will be within the active DL BWP. The Pre-MG1 will be activated and the Pre-MG2 will be deactivated.

< Way forward >:

    - Option 1:

      - UE can still perform measurement within the overlapping gap. Activation delay is further extended to the end of the overlapping MG plus (5ms + T1).

    - Option 2:

      - Follow the same agreement as scenario 1 when the Type-2 MG is replaced by an activated pre-MG.

    - Option 3:

      - UE is NOT required to perform measurement within the overlapping gap. No further delay is needed.

    - Option 4:

      - No need to discuss the case when two pre-configured MGs activation procedures are overlapped during the dynamic collision.

    - Option 5:

      - No gap dropping rule shall be applied (no gap collision will happen) and UE shall perform measurement within each activated Pre-MG.

To simplify RRM requirements and UE design, we support to follow the same agreement as scenario 1 when the Type-2 MG is replaced by an activated pre-MG.

Proposal 4: regarding scenario 4 (when one pre-configured MG deactivation procedure is fully overlapped with another pre-configured MG activation procedure triggered by single BWP switching during the dynamic collision), apply same agreement as in scenario 1 when the Type-2 MG is replaced by an activated pre-MG.

# Conclusion

In this contribution, we provide discussion on case 1 requirements. After discussion, the following conclusions are provided:

Proposal 1: no need to further discuss Pre-MG activation delay for scenario 1 (the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)), since it has already been covered by the agreement “Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG”.

Proposal 2: for scenario 2 (when the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)), apply same agreement as in scenario 1.

Proposal 3: regarding scenario 3 (when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG), apply same agreement as in scenario 1.

Proposal 4: regarding scenario 4 (when one pre-configured MG deactivation procedure is fully overlapped with another pre-configured MG activation procedure triggered by single BWP switching during the dynamic collision), apply same agreement as in scenario 1 when the Type-2 MG is replaced by an activated pre-MG.

# References

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc.

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation