3GPP TSG-RAN WG4 Meeting #108bis									            R4-2316564

Xiamen, China, October 9 – 13, 2023

Agenda item:	5.24.3

Source:	Apple

Title:	Discussion on RRM performance requirements of part 2

Document for:	Discussion

# Introduction

Performance part of this work item starts from this meeting. This is a high-level contribution on performance scope of this work item. Since part 1 is still unstable, this contribution only focuses on part 2.

# Discussion

### 2.1 NR-DC with selective activation of cell groups via L3 enhancements

New core requirements of this objective (Subsequent Conditional PSCell Change) are to be introduced in clause 8.11E [1], covering PCell in FR1 with PSCell in FR1 and FR2, respectively. Therefore, two separate test cases can be considered to cover FR1-FR1 NR-DC and FR1-FR2 NR-DC, respectively. Note that there is testability issue for FR1-FR1 to FR1-FR2. RAN4 can introduce it as a place holder. Regarding test parameters, existing configuration in test case for conditional PSCell change can be used as baseline.

Proposal 1: introduce the following two test cases for subsequent Conditional PSCell Change:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR2 NR-DC (with testability issue)

Proposal 2: test parameters in existing conditional PSCell change can be used as baseline for subsequent Conditional PSCell Change test cases.

### 2.2 Improvement on SCell/SCG setup delay

Both solutions based on existing measurement and enhanced measurement for improvement on SCell/SCG setup delay are quite open so far. Feasibility of each solution is still being challenged by different companies. No any core requirements are agreed so far. Therefore, we don’t expect any discussion on performance part for this objective in this meeting.

### 2.3 Enhanced CHO configurations

New core requirements of this objective are to be introduced in clause 6.1.6 and 6.1.7 (endorsed in the big CR [1]), respectively for NR Conditional Handover including target MCG and target SCG and NR Conditional Handover including target MCG and candidate SCG.

- Conditional Handover including target MCG and target SCG (obj.3)

From RRM requirements point of view, the new procedure in obj.3 is very similar with existing procedures of conditional handover and handover with PSCell. Note that UE has to pass those legacy test cases, in which the basic functionalities of conditional handover and handover with PSCell have already been verified. There is nothing new on top of these two procedures.

Therefore, we propose not to introduce new test cases for obj.3.

Observation 1: From RRM requirements point of view, the new procedure in obj.3 is very similar with existing procedures of conditional handover plus handover with PSCell.

Proposal 3: given that UE anyway has to pass conditional handover test cases and handover with PSCell test cases, no need to introduce new test case for conditional Handover including target MCG and target SCG (obj.3).

- Conditional Handover including target MCG and candidate SCG (obj.4)

The following scenarios are supposed to be covered:

- Scenario 1: FR1-FR1 NR-DC to FR1-FR1 NR-DC, Tprocessing = 25ms

- Scenario 2: FR1-FR1 NR-DC to FR1-FR2 NR-DC, Tprocessing = 45ms

- Scenario 3: FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 45ms

- Scenario 4: FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 25ms

Note that in RRM requirements only Tprocessing is different among different scenarios. Specifically, Tprocessing is same between scenario 1 and 4, and same between scenario 2 and 3. It seems two new test cases should be enough, e.g. scenario 1 and 2.

Proposal 4: introduce the following two test cases for conditional handover including target MCG and candidate SCG:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR2 NR-DC (with testability issue)

Regarding test parameters, similarly, existing configuration in test cases for conditional handover and handover with PSCell can be used as baseline.

Proposal 5: test parameters in test cases for conditional handover and handover with PSCell can be used as baseline for conditional handover including target MCG and candidate SCG

# Conclusion

In this contribution, we provide initial discussion on performance scope of part 2. After discussion, the following conclusions are provided:

Proposal 1: introduce the following two test cases for subsequent Conditional PSCell Change:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR2 NR-DC (with testability issue)

Proposal 2: test parameters in existing conditional PSCell change can be used as baseline for subsequent Conditional PSCell Change test cases.

Observation 1: From RRM requirements point of view, the new procedure in obj.3 is very similar with existing procedures of conditional handover plus handover with PSCell.

Proposal 3: given that UE anyway has to pass conditional handover test cases and handover with PSCell test cases, no need to introduce new test case for conditional Handover including target MCG and target SCG (obj.3).

Proposal 4: introduce the following two test cases for conditional handover including target MCG and candidate SCG:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC

- FR1-FR1 NR-DC to FR1-FR2 NR-DC (with testability issue)

Proposal 5: test parameters in test cases for conditional handover and handover with PSCell can be used as baseline for conditional handover including target MCG and candidate SCG

# References

- R4-2314488, Draft Big CR to TS 38.133 on RRM core requirements for Further NR mobility enhancements, Apple