3GPP TSG-RAN WG4 Meeting #108bis									            R4-2316568

Xiamen, China, October 9 – 13, 2023

Agenda item:	5.25.2.4

Source:	Apple

Title:	Discussion on network B requirements of R18 MUSIM

Document for:	Discussion

# Introduction

Network B requirements of R18 MUSIM have been widely discussed in previous RAN4 meetings. However, there are still some open issues. In this contribution, we provide our view on these issues.

# Discussion

Issue 4-1-1: Network B requirements conditions

- Proposals

  - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (Apple China Telecom CMCC Ericsson vivo Huawei Nokia )

    - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Apple China Telecom Ericsson vivo Nokia )

  - P2: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements. (Apple vivo)

  - P3: Do not add a condition stating that NW-B requirements only apply if “MUSIM gaps will not collide with other MUSIM gaps (Huawei Nokia)

  - P4: Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Continue discussion other conditions during or once NW B requirements are agreed. (Nokia)

  - P5: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm)

Recommendations: Continue discussion

For cell reselection in NW B, longer MGRP can be expected and it is not expected to be dropped due to collision with NW A gap. Otherwise all the MUSIM gaps would be dropped. Therefore, it makes sense to us that RAN4 can assume no MUSIM gap is dropped in NW B requirements. With this assumption, in general, we support P1, P1-1 and P2 with some modification.

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not be dropped due to collision with other MUSIM gaps” when defining NW B requirements.

Issue 4-1-2: Network B requirements framework

- Proposals

  - P1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline (Apple xiaomi Qualcomm vivo )

    - P1-1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (Apple xiaomi Qualcomm vivo)

    - P1-2: DRX cycle is replaced by max(DRX cycle, MGRP), MGRP is the MGRP of the mandatory gap pattern if it is agreed to define the mandatory MUSIM gap patterns; Otherwise, the solution that NW-B’s requirement is decouple with MUSIM gaps requested by UE can be considered (CMCC)

    - P1-3: The NW-B’s requirement should decouple with MUSIM gaps requested by UE. Error! Reference source not found. (China Telecom, Ericsson)

      - P1-3-1: Existing IDLE mode requirements are reused for NW B with a relaxation factor of 2 (Ericsson)

      - P1-3-2: Existing IDLE mode requirements are reused for NW B with a relaxation factor of 4 (Huawei)

    - P1-4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements and current DRX cycle is replaced with Max(DRX cycle, Min(MUSIM gap MGRP)). (Nokia)

    - P1-5: Replace DRX cycle by max(DRX cycle, MGRP_max) and introduce a scaling factor of 2. For instance, serving cell measurement can be as follows: (MTK)

- Table 1: Nserv

| max(DRX cycle, MGRP_maxNote2) [s] | Scaling Factor (N1) |  |  | Nserv [number of cycles based on max(DRX cycle, MGRP_max)] |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |
| 0.32 | 1 | 8 | 12 | M1*N1*4*K1 |
| 0.64 |  | 5 | 8 | M1*N1*4*K1 |
| 1.28 |  | 4 | 6 | N1*2*K1 |
| 2.56 |  | 3 | 5 | N1*2*K1 |
| Note 1: K1=2 Note 2: MGRP_max is the maximum MGRP among all configured MUSIM gaps |  |  |  |  |

  - P2: P1-1 and P1-3 is not reasonable. The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. (Nokia)

  - P3: M1 scaling factor shall be removed (Nokia)

Recommendations: To moderator’s understanding the maximum relaxation based on  P1-1 max(DRX cycle, MGRP_max) is 16 when MGRP of MUSIM gap is 5.12 and DRX of NW B is 0.32. (5.12/0.32). We can consider different ways of relaxation.  Continue discussion

We continue support P1 and P1-1.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline.

Proposal 5: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Issue 4-1-3: Requirement when MGRP = 5.12s

- Proposals

  - P1: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined. (Apple CMCC vivo Huawei Nokia)

    - P1-1: The new requirements for 5.12s could reuse corresponding requirements (number of DRX cycles) when DRX = 2.56s. (Apple vivo Huawei)

  - P2: RAN4 not need to discuss the requirement for MGRP=5.12s if the NW-B’s requirement is only related to NW-B’s DRX. (Ericsson)

  - P3: NW B requirements does not apply when MUSIM gap’s MGRP=5.12s (MTK)

Recommendations: Continue discussion

For completion, we support P1 to introduce requirement for 5.12s MGRP. P1-1 can also be considered since this is the last meeting for core part design.

Proposal 6: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 7: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

Issue 4-1-4: NW B inter-frequency and inter-RAT measurement

- P1: Do not define inter-RAT measurement/evaluation/detection requirements of NW B. (vivo)

- P2: Clarify the need for performing inter-frequency measurement in NW-B; Clarify the need to for RAN4 to define UE requirements for NW-B inter-frequency measurements. (Nokia)

Recommendations: Continue discussion

We support to postpone inter-RAT requirements, at least until RAN4 finalizes intra-NR requirements.

Proposal 8: deprioritize NW B inter-RAT measurement requirements, at least until RAN4 finalizes intra-NR requirements for NW B.

Issue 4-1-5: Solutions when different MGRPs are used for measurement

- Proposals

  - P1: When multiple MUSIM gaps with different MGRP configured for measurement, requirements could be option 1 by using max(DRX cycle, MGRP_max)  (using inter-frequency detection requirements as an example) (vivo):

    - Option 1. The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier_total * Tdetect,NR_Inter where Tdetect,NR_Inter (Tmeasure,NR_Inter/ Tevaluate,NR_Inter) depends on max(DRX cycle, MGRP_max) and Kcarrier_total is the total number of inter frequency layers to be measured of NW B.

  - P4: RAN4 not need to discuss the solution when different MGRPs will be used for NW-B’s measurement if the NW-B’s requirement is only related to NW-B’s DRX (Ericsson)

  - P5: Postpone after conclusion of Issue 4-1-2 (Huawei)

Recommendations: Continue discussions

We support P5, i.e. postpone after conclusion of Issue 4-1-2.

Issue 4-1-6: Network B requirements test case

- Proposals

  - P1: Do not define test cases to verify any new requirements in network B. (Qualcomm vivo Huawei MTK )

  - P2: Do not exclude defining test cases to verify any new requirements in network B (Nokia)

  - P3: RAN4 to postpone the test case discussion to performance part (Ericsson)

Recommendations: Continue discussion

We support P1.

Proposal 9: Do not define test cases to verify any new requirements in network B.

# Conclusion

In this contribution, we provide discussion on network B requirements of R18 MUSIM. After discussion, the following conclusions are provided:

Proposal 1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only.

Proposal 2: The inactive state requirement should be the same as NW B’s Idle state.

Proposal 3: Add the condition “MUSIM gaps will not be dropped due to collision with other MUSIM gaps” when defining NW B requirements.

Proposal 4: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline.

Proposal 5: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps.

Proposal 6: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined.

Proposal 7: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s.

Proposal 8: deprioritize NW B inter-RAT measurement requirements, at least until RAN4 finalizes intra-NR requirements for NW B.

Proposal 9: Do not define test cases to verify any new requirements in network B.

# References

- R4-2314364, WF on R18 MUSIM, vivo

- R4-2310165, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo